NWC SAF Winds Intercomparison Study Report: 2018

Version 2

08 Apr 2018

David Santek1, Javier García-Pereda2, Rich Dworak1,

Steve Wanzong1, Katherine Winiecki1, Régis Borde3

1Cooperative Institute for Meteorological Satellite Studies

University of Wisconsin-Madison

Madison, Wisconsin 53706 USA

2NWC SAF/AEMET

Madrid, Spain

3 EUMETSAT

Darmstadt, Germany

Table of Contents

1.	Abstract	5

2.	Motivation	5

3.	Case Study	5

4.	Overview	7

5.	Input Data Files	7

6.	Wind Retrieval Algorithms	8

7.	Experiment 1	10

a)	Approach	11

b)	Parameter distributions	12

c)	Collocation plots	18

d)	Rawinsonde comparison	26

e)	Model Grid comparison	33

f)	Best fit height	39

g)	Statistical comparison	75

8.	Experiment 2	87

a)	Approach	89

b)	Parameter distributions	89

c)	Collocation plots	96

d)	Rawinsonde comparison	104

e)	Model Grid comparison	110

f)	Best fit height	114

g)	Statistical comparison	150

9.	Experiment 3	163

a)	Approach	164

b)	Parameter distributions	164

c)	Collocation plots	173

d)	Model Grid comparison	179

e)	Best fit height	183

f)	Statistical comparison	217

10.	Comparison to CALIPSO	220

11.	Summary and Conclusions	224

a)	EUMETSAT	225

b)	Japan Meteorological Agency	225

c)	NOAA	225

d)	Korea Meteorological Administration	226

e)	NWC SAF	226

f)	Brazilian Meteorological Center	226

12.	References	228

13.	Acknowledgements	228

14.	Appendix A: Parameter Distribution Histograms	229

Experiment 1: QINF Parameter Distribution Histograms	229

Experiment 1: QIWF Parameter Distribution Histograms	233

Experiment 1: CQI Parameter Distribution Histograms	235

Experiment 2: QINF Parameter Distribution Histograms	240

Experiment 2: QIWF Parameter Distribution Histograms	244

Experiment 2: CQI Parameter Distribution Histogram	246

Experiment 3: QINF Parameter Distribution Histogram	250

Experiment 3: QIWF Parameter Distribution Histograms	254

Experiment 3: CQI Parameter Distribution Histograms	257

Experiment 4: QINF Parameter Distribution Histograms	261

Experiment 4: QIWF Parameter Distribution Histograms	261

15.	Appendix B: Matlab t-test Documentation	262

16.	Appendix C: t-test Results	263

Experiment 1 t-test Results	263

Experiment 2 t-test Results	271

Experiment 3 t-test Results	279

Experiment 4 t-test Results	287

17.	Appendix D: Best Fit Speed and Vector Difference	289

Experiment 1 Best Fit speed difference	289

Experiment 1 Best Fit vector difference	306

Experiment 2 Best Fit speed difference	323

Experiment 2 Best Fit vector difference	340

Experiment 3 Best Fit speed difference	357

Experiment 3 Best Fit vector difference	374

Experiment 4 Best Fit speed difference	391

Experiment 4 Best Fit vector difference	392

18.	Appendix E: Shell Scripts	394

input_fix.sh	394

19.	Appendix F: Best-Fit Python Scripts	395

test_amv.py	395

amv.py	397

20.	Appendix G: Matlab Scripts	404

Winds_Bulk_QINF.m	405

Winds_Bulk_QIF.m	409

Winds_Match_QIF.m	413

Winds_Match_QINF.m	418

Stats_QIF_One.m	422

Winds_Height_Diff.m	426

# Abstract

Previous Atmospheric Motion Vector (AMV) intercomparison studies, conducted from 2007 to 2014, compared the operational AMV algorithms of various satellite-derived wind producers using a common set of MSG/SEVIRI images and ancillary data. The studies assessed how the cloudy AMVs from the unique wind producers compared in terms of coverage, speed, direction, and cloud height (Genkova et al. 2008; Genkova et al. 2010, Santek et al. 2014).

One of the main conclusions of the 2014 study was that “the two centers using Borde & Oyama’s Cross Correlation Contribution height assignment method (EUMETSAT and NWCSAF) were in general the ones obtaining the best validation statistics.”

In this continuation, AMVs calculated with JMA’s Himawari-8/AHI satellite data were compared to address two main goals:

- To verify the advantages of the calculation of AMVs with the new generation of geostationary satellites that began in 2014 with Himawari-8, continuing with GOES-R over the past two years, and the expected launch of the MTG-I series in the early 2020s, as compared to those calculated with MSG series.

- To determine the best options for the calculation of AMVs with MTG-I with NWC/GEO-HRW algorithm, considering the options taken by the different satellite centres for their AMV calculation.

Also, a Common QI was computed to determine if better agreement in the winds would result if the QI was compute similarly.

# Motivation

From the 2017 AMV Intercomparison Study Call for Proposals:

JMA’s Himawari-8/AHI satellite data will be used in the processing. This way, improvements of the different AMV algorithms respect to AMVs calculated with MSG satellite data will be shown. Additionally, a verification will be done of the best options available for the processing of AMVs with the new generation of geostationary satellites (Himawari-8/9, GOES-R, MTG-I, etc), with higher spatial resolution, higher temporal resolution, and more spectral channels. The new spectral channels, bearing new information on cloud microphysics and with a higher temporal resolution, should allow to better understand the characteristics of the tracked features.

The study will act as an input for the development of NWC/GEO-HRW algorithm (and also the other AMV algorithms), for their optimal use with this new generation of geostationary satellites. The corresponding conclusions will be taken into account during the scientific developments of “NWC/GEO vMTG day-1 and day-2” versions, in the year 2018 and onwards, to which this VSA activity will contribute with its Final Report.

# Case Study

There were two cases processed from 21 July 2016 (day 203), using several different configurations. The primary case is from 1200 UTC; a second case from 0530 UTC is for comparison to CALIPSO cloud heights. The primary case study for the AMV experiments is a triplet of infrared (10.4µ; band 13) Himawari-8, full–disk images at 1200, 1210, 1220 UTC (Figure 3-1).

A second dataset was processed for the times 0530, 0540, 0550 UTC, which is coincident with an overpass from CALIPSO.

The experiments are numbered, thusly:

Table 3-1: Experiment numbering

| Experiment | Configuration | Time | Participants |
| --- | --- | --- | --- |
| 1 | Common (21) | 1200 | All |
| 2 | Own (22) | 1200 | All |
| 3 | Own (52) | 0530 | All |
| 4 | Common (51) | 0530 | ??? |
| 5 | Own (23) | 1200 | JMA (high res data) |
| 6 | Own (53) | 0530 | JMA (high res data) |
| 7 | Own (23) | 1200 | NWC (own input data) |
| 8 | Own (53) | 0530 | NWC (own input data) |

Figure 3-1: Himawari-8 10.4 µm (band 13) from 21 July 2016 at 1200 UTC.

# Overview

The output provided by the winds producers was analyzed for three primary experiments, each of which is located in a separate section, including text from the proposal describing the experiment, the approach used in the analysis, highlights of the results, and figures and tables supporting the results. Text that is in italics is extracted directly from the CIMSS proposal.

The approach taken began with the scripts used in the previous intercomparison studies, resulting in plots and statistics that can be compared and contrasted with the previous work (Genkova et al. 2008, 2010; Santek et al. 2014). This includes using the paired t-test, which was first used in the 2014 study:

While the Standard (Student’s) t-test determines the likelihood a statistically significant difference exists between two data sets, a paired t-test assumes data points from the two different data sets are related. For example, a paired t-test is often used to compare before-and-after data sets because each “before” data point is paired with a specific “after” data point. In our case, each data point from center X was paired, by having both latitude and longitude coordinates within a specified distance, with its corresponding data point from center Y. For each of the comparisons, paired t-tests were calculated for several variables in each experiment in order to compare every combination of centers, with a 95% confidence setting.

# Input Data Files

Each wind producer provided files containing the results of the experiments with the same parameters using a text file format.

The wind producers are:

- BRZ:	Brazil Weather Forecast and Climatic Studies Center

- EUM:	EUMETSAT (European Organization for the Exploitation of

Meteorological Satellites)

- JMA:	Japan Meteorological Agency

- KMA: 	Korea Meteorological Administration

- NOA:	National Oceanic and Atmospheric Administration

- NWC:	NWC SAF (Satellite Application Facility on Support to   Nowcasting & Very Short Range Forecasting)

The three-letter abbreviations above are used throughout the remainder of this report. The variables reported by the centers are in the same order as described by the header from NWC:

Table 5-1: Reported Variables

| 1 | ID | Identification number |
| --- | --- | --- |
| 2 | LAT[DEG] | Latitude |
| 3 | LON[DEG] | Longitude |
| 4 | TS[PIX] | Target box size |
| 5 | SS[PIX] | Search box size |
| 6 | SPD[MPS] | AMV speed |
| 7 | DIR[DEG] | AMV direction |
| 8 | PRES[hPa] | AMV pressure |
| 9 | L | Low-level correction |
| 10 | NWSSPD[MPS] | Background guess wind speed |
| 11 | NWSDIR[DEG] | Background guess wind direction |
| 12 | ALB[%] | Albedo |
| 13 | CORR[%] | Correlation |
| 14 | T | Brightness temperature |
| 15 | PRESSERR[hPa] | AMV pressure error |
| 16 | H | Height assignment method |
| 17 | QIN[%] | QI without forecast |
| 18 | QIF[%] | QI with forecast |
| 19 | QIC[%] | Common QI |

The ancillary data consisted of two ECMWF forecast grids for the 12- and 18-hour forecast from 0000 UTC on 17 September 2012. It was subsected and reformatted by EUMETSAT to the Meteosat-9 domain with the following specifications:

- 135x135 grid centered at 0°N/O°E

- Domain: 67° S to 67° N; 67° E to 67° W

- 1° spatial resolution

- 40 vertical levels

- Parameters: pressure, geopotential height, temperature, water vapor mixing ratio, ozone mixing ratio, wind speed, wind direction, and dewpoint temperature.

Note: The NOAA AMV processing software requires additional parameters, so this background was not used. See the Error! Reference source not found. section in the Error! Not a valid bookmark self-reference. for more details on the grids used.

# Wind Retrieval Algorithms

Please refer to the 2014 Intercomparison Report for details on the algorithms for each of the AMV producers, as provided to us at that time. We do not have updated information on updates or modifications to algortihms that have occurred in the last four years.

# Experiment 1

AMV producers extract infrared channel AMVs considering a prescribed AMV algorithm configuration, but only using the Himawari-8/AHI IR10.4 images and the ECMWF model data for the height assignment. This experiment was used to test the Tracking and Quality control steps in all AMV algorithms, considering similar targets. The prescribed AMV configuration defines target scene size, search scene size, etc. to be the same for all AMV producers, which is similar to Experiment 3 in the previous study (Santek et al. 2014).

In the 2nd study (Genkova et al. 2010) it was found that:

Winds data sets retrieved using common target and search box sizes revealed that each producer’s algorithm is finely tuned to a specific imagery temporal and spatial resolution, as well as target and search box sizes.

In the 3rd study (Santek et al. 2014):

Collocated vectors only number 370, due to the lower overall numbers of AMVs when using prescribed target and search box sizes. Considering this configuration, there are more similarities between centers: speed and direction differences are not statistically different, although, pressure and QI values are significantly different.

For each one of the AMV producer’s datasets, a distribution of AMV speed, direction, vector height, and QI are shown. Differences of these quantities between AMV producers are shown. Also, differences in AMV coverage and number of vectors are presented as bulk statistics and geographic plots.

Collocated AMVs from the different algorithms are used to measure the differences. Also, a comparison of the AMVs with the NWP model winds and height assignment investigations using NWP model best-fit pressure are used for verification.

Experiment 1 Highlights

A fixed displacement of four elements and two lines were applied to a single image in order to create an artificial triplet. This tests the tracking differences between the data producers.

There were two positive results:

- All AMV algorithms detected this shift correctly, generally with no more than 0.1-pixel difference (related to subpixel tracking turned on). See Figure 7-1 to Figure 7-7.

- There were 10876 colocated vectors when using a distance threshold of 35 km. The differences of horizontal and vertical displacements between EUM and each of the other centers were not statistically significant (Table 7-1 and Table 7-2).

Two sites, BRZ and CMA, appear to have an AMV speed dependence on distance from satellite subpoint (Figure 7-10 and Figure 7-14), which may be due to the way to compute distance (BRZ) or to truncation in speed (CMA). Both centers are investigating this behaviour.

## Approach

By using a prescribed AMV configuration, we were able to better quantify differences in AMV density between the producers. In cases with similar densities among producers, the QI can also be compared quantitatively.

This was similar to Study 2 by Genkova et al. (2010) and Study 3 (Santek et al. 2014):

- Only the window IR channel was used for the height assignment.

- Producers used their algorithm.

- Prescribed target and search box sizes were used.

- ECMWF grids were used as NWP data.

- A 55 km distance to define collocation was used (Note: Genkova et al. (2010) used 0.5°).

- The best fit analysis was used to further analyze differences in cloud heights.

- The paired t-test was computed to determine if the observed differences in collocated AMVs were statistically significant.

## Parameter distributions

The bulk statistics are presented in both tables and histograms, for QINF >= 50 and 80, QIWF >= 50 and 80, and CQI >=50 and 80 (because not all centers reported all QI values).

Table 7-1 through Table 7-6 list basic Experiment 1 statistics for the AMVs for each winds producer, without and with forecast quality, and the common QI. Most of the values in this table are approximately the same, with the largest variation being in the number of AMVs, ranging from 14602 (NWC) to 76075 (JMA).

Table 7-1: Experiment 1 statistical summary of AMV datasets for QINF>=50.

|  | EUM | BRZ | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- | --- |
| Total AMVs | 69546 | 70312 | 76075 | 38242 | 62669 | 14602 |
| QI>=50 | 63722 | 61806 | 57586 | 36172 | 57727 | 12511 |
| SPD_min | 2.50 | 3.01 | 2.50 | 3.00 | 2.50 | 2.56 |
| SPD_max | 182.31 | 66.05 | 79.06 | 92.29 | 91.32 | 68.77 |
| SPD_mean | 14.04 | 11.66 | 13.36 | 14.06 | 13.40 | 13.96 |
| P_min | 59.53 | 100.00 | 100.00 | 102.05 | 105.40 | 104.00 |
| P_max | 1003.97 | 1000.00 | 1013.81 | 977.82 | 990.00 | 965.50 |
| P_mean | 418.98 | 530.88 | 488.55 | 432.19 | 463.46 | 391.90 |
| Low_winds | 24.26 | 36.49 | 29.63 | 33.77 | 23.73 | 24.64 |
| Mid_winds | 13.62 | 20.94 | 21.40 | 2.63 | 28.76 | 8.61 |
| High_winds | 62.12 | 42.56 | 48.97 | 63.59 | 47.50 | 66.75 |
| Low_SPD_min | 2.50 | 3.01 | 2.50 | 3.00 | 2.50 | 2.78 |
| Low_SPD_max | 103.32 | 38.08 | 41.20 | 40.52 | 78.26 | 28.57 |
| Low_SPD_mean | 9.16 | 9.58 | 9.66 | 10.21 | 10.51 | 10.11 |
| Low_P_min | 700.00 | 700.00 | 700.01 | 700.06 | 700.03 | 700.00 |
| Low_P_max | 1003.97 | 1000.00 | 1013.81 | 977.82 | 990.00 | 965.50 |
| Low_P_mean | 818.89 | 872.68 | 883.51 | 837.59 | 781.96 | 781.64 |
| Mid_SPD_min | 2.52 | 3.07 | 2.50 | 3.06 | 2.50 | 2.82 |
| Mid_SPD_max | 123.63 | 66.05 | 69.25 | 78.86 | 80.01 | 48.12 |
| Mid_SPD_mean | 15.01 | 13.36 | 13.53 | 22.14 | 14.25 | 17.44 |
| Mid_P_min | 400.01 | 400.02 | 400.01 | 400.01 | 400.00 | 400.50 |
| Mid_P_max | 699.83 | 699.98 | 699.99 | 449.96 | 699.99 | 699.50 |
| Mid_P_mean | 523.77 | 536.15 | 525.92 | 425.91 | 543.00 | 536.58 |
| High_SPD_min | 2.50 | 3.05 | 2.50 | 3.00 | 2.50 | 2.56 |
| High_SPD_max | 182.31 | 62.47 | 79.06 | 92.29 | 91.32 | 68.77 |
| High_SPD_mean | 15.74 | 12.61 | 16.08 | 15.77 | 14.33 | 14.92 |
| High_P_min | 59.53 | 100.00 | 100.00 | 102.05 | 105.40 | 104.00 |
| High_P_max | 400.00 | 400.00 | 399.98 | 399.99 | 399.98 | 400.00 |
| High_P_mean | 239.81 | 235.22 | 233.19 | 217.14 | 256.20 | 229.36 |

Table 7-2: Experiment 1 statistical summary of AMV datasets for QIWF  >= 50.

|  | EUM | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- |
| Total AMVs | 69546 | 76075 | 38242 | 62669 | 14602 |
| QI>=50 | 59430 | 56769 | 35868 | 56132 | 12526 |
| SPD_min | 2.50 | 2.50 | 3.00 | 2.50 | 2.63 |
| SPD_max | 139.93 | 92.86 | 92.29 | 91.32 | 68.77 |
| SPD_mean | 13.73 | 13.40 | 14.10 | 13.49 | 14.13 |
| P_min | 59.53 | 100.00 | 102.05 | 105.40 | 104.00 |
| P_max | 1003.97 | 1013.81 | 977.82 | 990.00 | 965.50 |
| P_mean | 425.13 | 489.60 | 433.85 | 463.88 | 392.13 |
| Low_winds | 25.30 | 29.66 | 34.04 | 23.99 | 24.59 |
| Mid_winds | 13.68 | 21.62 | 2.62 | 28.54 | 8.70 |
| High_winds | 61.02 | 48.72 | 63.35 | 47.47 | 66.71 |
| Low_SPD_min | 2.50 | 2.50 | 3.00 | 2.50 | 2.78 |
| Low_SPD_max | 72.74 | 92.86 | 40.52 | 70.27 | 28.57 |
| Low_SPD_mean | 9.07 | 9.53 | 10.21 | 10.51 | 10.16 |
| Low_P_min | 700.00 | 700.01 | 700.06 | 700.03 | 700.00 |
| Low_P_max | 1003.97 | 1013.81 | 977.82 | 990.00 | 965.50 |
| Low_P_mean | 817.72 | 884.43 | 837.57 | 781.99 | 781.81 |
| Mid_SPD_min | 2.51 | 2.50 | 3.06 | 2.50 | 2.82 |
| Mid_SPD_max | 94.60 | 69.25 | 70.50 | 74.05 | 48.12 |
| Mid_SPD_mean | 14.76 | 13.27 | 22.27 | 14.36 | 17.53 |
| Mid_P_min | 400.01 | 400.01 | 400.01 | 400.00 | 400.50 |
| Mid_P_max | 699.83 | 699.99 | 449.96 | 699.99 | 699.50 |
| Mid_P_mean | 524.50 | 525.76 | 425.77 | 542.96 | 535.43 |
| High_SPD_min | 2.51 | 2.50 | 3.00 | 2.50 | 2.63 |
| High_SPD_max | 139.93 | 79.06 | 92.29 | 91.32 | 68.77 |
| High_SPD_mean | 15.42 | 15.82 | 15.86 | 14.47 | 15.15 |
| High_P_min | 59.53 | 100.00 | 102.05 | 105.40 | 104.00 |
| High_P_max | 400.00 | 399.98 | 399.99 | 399.98 | 400.00 |
| High_P_mean | 240.07 | 233.16 | 217.27 | 255.57 | 229.81 |

Table 7-3: Experiment 1 statistical summary of AMV datasets for CQI >= 50

|  | EUM | BRZ | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- | --- |
| Total AMVs | 69546 | 70312 | 76075 | 38242 | 62669 | 14602 |
| QI>=50 | 26148 | 57613 | 33717 | 36172 | 55498 | 14283 |
| SPD_min | 2.51 | 2.56 | 2.50 | 3.00 | 2.50 | 2.50 |
| SPD_max | 115.86 | 66.05 | 69.97 | 92.29 | 97.01 | 68.77 |
| SPD_mean | 12.46 | 11.81 | 13.29 | 14.06 | 13.54 | 13.64 |
| P_min | 59.53 | 100.00 | 125.00 | 102.05 | 105.40 | 101.00 |
| P_max | 1003.97 | 1000.00 | 994.32 | 977.82 | 990.00 | 965.50 |
| P_mean | 427.08 | 533.19 | 496.89 | 432.19 | 461.70 | 386.40 |
| Low_winds | 26.31 | 37.10 | 31.43 | 33.77 | 23.91 | 23.61 |
| Mid_winds | 13.35 | 20.33 | 19.34 | 2.63 | 28.02 | 8.96 |
| High_winds | 60.34 | 42.57 | 49.23 | 63.59 | 48.06 | 67.43 |
| Low_SPD_min | 2.52 | 2.56 | 2.50 | 3.00 | 2.50 | 2.57 |
| Low_SPD_max | 73.69 | 35.67 | 41.20 | 40.52 | 70.27 | 28.57 |
| Low_SPD_mean | 9.24 | 9.61 | 9.06 | 10.21 | 10.55 | 9.95 |
| Low_P_min | 700.00 | 700.00 | 700.01 | 700.06 | 700.03 | 700.00 |
| Low_P_max | 1003.97 | 1000.00 | 994.32 | 977.82 | 990.00 | 965.50 |
| Low_P_mean | 815.26 | 874.49 | 888.89 | 837.59 | 781.69 | 782.70 |
| Mid_SPD_min | 2.52 | 2.61 | 2.50 | 3.06 | 2.50 | 2.51 |
| Mid_SPD_max | 71.05 | 66.05 | 69.25 | 78.86 | 80.01 | 48.12 |
| Mid_SPD_mean | 14.76 | 13.61 | 14.03 | 22.14 | 14.48 | 17.05 |
| Mid_P_min | 400.03 | 400.02 | 400.01 | 400.01 | 400.00 | 400.05 |
| Mid_P_max | 699.83 | 699.98 | 699.99 | 449.96 | 699.99 | 699.50 |
| Mid_P_mean | 523.48 | 536.33 | 528.20 | 425.91 | 542.77 | 536.54 |
| High_SPD_min | 2.51 | 2.58 | 2.50 | 3.00 | 2.50 | 2.50 |
| High_SPD_max | 115.86 | 62.47 | 69.97 | 92.29 | 97.01 | 68.77 |
| High_SPD_mean | 13.36 | 12.88 | 15.69 | 15.77 | 14.48 | 14.47 |
| High_P_min | 59.53 | 100.00 | 125.00 | 102.05 | 105.40 | 101.00 |
| High_P_max | 399.78 | 400.00 | 399.98 | 399.99 | 399.98 | 400.00 |
| High_P_mean | 236.53 | 234.17 | 234.32 | 217.14 | 255.23 | 227.70 |

Table 7-4: Experiment 1 statistical summary of AMV datasets for QINF>=80.

|  | EUM | BRZ | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- | --- |
| Total AMVs | 69546 | 70312 | 76075 | 38242 | 62669 | 14602 |
| QI>=80 | 48943 | 41304 | 34036 | 30376 | 47164 | 7285 |
| SPD_min | 2.50 | 3.01 | 2.50 | 3.00 | 2.50 | 2.75 |
| SPD_max | 139.93 | 66.05 | 70.45 | 92.29 | 80.31 | 68.77 |
| SPD_mean | 14.08 | 11.37 | 14.35 | 14.77 | 14.03 | 15.70 |
| P_min | 59.53 | 100.00 | 100.00 | 102.05 | 105.40 | 104.50 |
| P_max | 1003.97 | 1000.00 | 1012.82 | 976.20 | 990.00 | 922.50 |
| P_mean | 430.99 | 552.18 | 488.37 | 439.29 | 468.23 | 412.39 |
| Low_winds | 26.60 | 40.23 | 30.64 | 34.93 | 25.33 | 27.60 |
| Mid_winds | 12.92 | 19.92 | 18.78 | 2.48 | 27.77 | 8.72 |
| High_winds | 60.48 | 39.85 | 50.58 | 62.60 | 46.90 | 63.68 |
| Low_SPD_min | 2.50 | 3.01 | 2.50 | 3.01 | 2.50 | 2.86 |
| Low_SPD_max | 72.74 | 35.46 | 41.20 | 39.11 | 70.27 | 28.57 |
| Low_SPD_mean | 9.23 | 9.57 | 9.48 | 10.50 | 10.79 | 10.88 |
| Low_P_min | 700.00 | 700.00 | 700.01 | 700.08 | 700.03 | 700.00 |
| Low_P_max | 1003.97 | 1000.00 | 1012.82 | 976.20 | 990.00 | 922.50 |
| Low_P_mean | 817.61 | 875.45 | 889.28 | 838.05 | 782.87 | 779.27 |
| Mid_SPD_min | 2.52 | 3.10 | 2.50 | 3.37 | 2.50 | 3.11 |
| Mid_SPD_max | 94.60 | 66.05 | 69.25 | 60.21 | 74.05 | 48.12 |
| Mid_SPD_mean | 15.27 | 13.05 | 15.41 | 23.70 | 15.17 | 18.97 |
| Mid_P_min | 400.03 | 400.05 | 400.03 | 400.01 | 400.00 | 400.50 |
| Mid_P_max | 699.83 | 699.97 | 699.99 | 449.96 | 699.99 | 699.50 |
| Mid_P_mean | 524.90 | 537.17 | 527.88 | 425.27 | 544.39 | 549.54 |
| High_SPD_min | 2.51 | 3.05 | 2.50 | 3.00 | 2.50 | 2.75 |
| High_SPD_max | 139.93 | 62.47 | 70.45 | 92.29 | 80.31 | 68.77 |
| High_SPD_mean | 15.95 | 12.35 | 16.90 | 16.79 | 15.01 | 17.35 |
| High_P_min | 59.53 | 100.00 | 100.00 | 102.05 | 105.40 | 104.50 |
| High_P_max | 400.00 | 400.00 | 399.98 | 399.99 | 399.98 | 400.00 |
| High_P_mean | 240.92 | 233.34 | 230.85 | 217.36 | 253.22 | 234.57 |

Table 7-5: Experiment 1 statistical summary of AMV datasets for QIWF>=80.

|  | EUM | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- |
| Total AMVs | 69546 | 76075 | 38242 | 62669 | 14602 |
| QI>=80 | 40336 | 35786 | 28418 | 29731 | 6571 |
| SPD_min | 2.50 | 2.50 | 3.00 | 2.50 | 2.76 |
| SPD_max | 75.43 | 70.45 | 92.29 | 76.09 | 68.77 |
| SPD_mean | 15.02 | 13.95 | 15.17 | 15.12 | 16.34 |
| P_min | 69.76 | 100.00 | 106.23 | 107.15 | 109.00 |
| P_max | 1000.57 | 1012.82 | 976.20 | 990.00 | 922.50 |
| P_mean | 439.02 | 486.33 | 447.85 | 495.93 | 418.80 |
| Low_winds | 28.03 | 30.20 | 36.15 | 30.78 | 28.25 |
| Mid_winds | 12.69 | 19.14 | 2.53 | 26.73 | 9.05 |
| High_winds | 59.28 | 50.66 | 61.31 | 42.49 | 62.70 |
| Low_SPD_min | 2.50 | 2.50 | 3.01 | 2.51 | 3.07 |
| Low_SPD_max | 29.58 | 41.20 | 39.11 | 35.05 | 28.57 |
| Low_SPD_mean | 9.44 | 9.33 | 10.62 | 11.02 | 11.15 |
| Low_P_min | 700.03 | 700.01 | 700.08 | 700.06 | 700.00 |
| Low_P_max | 1000.57 | 1012.82 | 976.20 | 990.00 | 922.50 |
| Low_P_mean | 814.98 | 889.25 | 838.40 | 788.86 | 779.75 |
| Mid_SPD_min | 2.64 | 2.50 | 3.55 | 2.50 | 3.11 |
| Mid_SPD_max | 71.05 | 69.25 | 60.21 | 74.05 | 48.12 |
| Mid_SPD_mean | 16.20 | 14.75 | 24.06 | 15.86 | 19.22 |
| Mid_P_min | 400.03 | 400.03 | 400.01 | 400.01 | 400.50 |
| Mid_P_max | 699.83 | 699.99 | 449.96 | 699.99 | 699.50 |
| Mid_P_mean | 523.94 | 526.22 | 425.40 | 541.99 | 549.79 |
| High_SPD_min | 2.51 | 2.50 | 3.00 | 2.51 | 2.76 |
| High_SPD_max | 75.43 | 70.45 | 92.29 | 76.09 | 68.77 |
| High_SPD_mean | 17.40 | 16.40 | 17.48 | 17.63 | 18.26 |
| High_P_min | 69.76 | 100.00 | 106.23 | 107.15 | 109.00 |
| High_P_max | 400.00 | 399.98 | 399.99 | 399.96 | 400.00 |
| High_P_mean | 243.04 | 231.06 | 218.49 | 254.74 | 237.29 |

Table 7-6: Experiment 1 statistical summary of AMV datasets for CQI>=80.

|  | EUM | BRZ | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- | --- |
| Total AMVs | 69546 | 70312 | 76075 | 38242 | 62669 | 14602 |
| QI>=80 | 4441 | 37137 | 29740 | 30376 | 44820 | 11919 |
| SPD_min | 2.59 | 2.57 | 2.50 | 3.00 | 2.50 | 2.51 |
| SPD_max | 73.36 | 61.35 | 69.97 | 92.29 | 76.09 | 68.77 |
| SPD_mean | 12.91 | 11.76 | 13.68 | 14.77 | 14.13 | 14.02 |
| P_min | 90.17 | 100.00 | 125.00 | 102.05 | 105.40 | 104.50 |
| P_max | 975.00 | 1000.00 | 994.32 | 976.20 | 990.00 | 917.50 |
| P_mean | 448.66 | 553.83 | 500.37 | 439.29 | 459.38 | 386.93 |
| Low_winds | 29.48 | 41.54 | 32.13 | 34.93 | 24.85 | 24.25 |
| Mid_winds | 14.57 | 17.60 | 18.96 | 2.48 | 26.08 | 8.03 |
| High_winds | 55.96 | 40.86 | 48.91 | 62.60 | 49.07 | 67.72 |
| Low_SPD_min | 2.59 | 2.57 | 2.50 | 3.01 | 2.50 | 2.70 |
| Low_SPD_max | 26.98 | 35.46 | 41.20 | 39.11 | 70.08 | 28.57 |
| Low_SPD_mean | 9.05 | 9.69 | 9.27 | 10.50 | 10.80 | 10.22 |
| Low_P_min | 700.00 | 700.00 | 700.01 | 700.08 | 700.03 | 700.00 |
| Low_P_max | 975.00 | 1000.00 | 994.32 | 976.20 | 990.00 | 917.50 |
| Low_P_mean | 812.41 | 880.02 | 887.95 | 838.05 | 781.10 | 780.25 |
| Mid_SPD_min | 2.71 | 2.61 | 2.50 | 3.37 | 2.50 | 2.51 |
| Mid_SPD_max | 67.22 | 60.84 | 69.25 | 60.21 | 74.05 | 48.12 |
| Mid_SPD_mean | 14.55 | 13.80 | 14.66 | 23.07 | 15.46 | 17.76 |
| Mid_P_min | 400.85 | 400.09 | 400.03 | 400.01 | 400.00 | 400.50 |
| Mid_P_max | 699.68 | 699.82 | 699.99 | 449.96 | 699.99 | 699.50 |
| Mid_P_mean | 508.09 | 534.75 | 531.09 | 425.27 | 544.24 | 540.48 |
| High_SPD_min | 2.69 | 2.76 | 2.50 | 3.00 | 2.50 | 2.53 |
| High_SPD_max | 73.36 | 61.35 | 69.97 | 92.29 | 76.09 | 68.77 |
| High_SPD_mean | 14.52 | 12.99 | 16.19 | 16.79 | 15.11 | 14.94 |
| High_P_min | 90.17 | 100.00 | 125.00 | 102.05 | 105.40 | 104.50 |
| High_P_max | 399.39 | 400.00 | 399.98 | 399.99 | 399.96 | 400.00 |
| High_P_mean | 241.57 | 230.40 | 233.92 | 217.36 | 251.38 | 227.91 |

## Collocation plots

AMVs are first quality controlled, retaining only those with a QINF >= 50. For collocation the distance threshold was 55 km, resulting in 7050 AMVs. The following three figures depict the collocated vectors in terms of parameters (from top to bottom: speed direction, pressure, QI, maximum pressure difference between collocated AMVs, and a scatter plot of AMV pressure from each centre pressure vs. EUM AMV pressure.

Note: There are points plotted on the y-axis in Figure 8-3 and

Figure 8-6, indicating zero pressure values for EUM AMVs. These zero values are also in the original text files from EUMETSAT.

Figure 7-1: (QINF) Plots of collocated AMVs of speed (top), direction (2nd), pressure (3rd), and QI (bottom) are color-coded based on legend in upper-right. The x-axis is AMV number.

Figure 7-2: (QINF) The maximum pressure difference between any two collocated AMVs.

Figure 7-3: (QINF) Scatter plot of AMV pressure for each center vs. EUM pressure.

Figure 7-4: (QIWF)  Plots of collocated AMVs of speed (top), direction (2nd), pressure (3rd), and QI (bottom) are color-coded based on legend in upper-right. The x-axis is AMV number.

Figure 7-5: (QIWF) The maximum pressure difference between any two collocated AMVs.

Figure 7-6: (QIWF) Scatter plot of AMV pressure for each center vs. EUM pressure.

Figure 7-7: (CQI) Plots of collocated AMVs of speed (top), direction (2nd), pressure (3rd), and QI (bottom) are color-coded based on legend in upper-right. The x-axis is AMV number.

Figure 7-8: (CQI) The maximum pressure difference between any two collocated AMVs.

Figure 7-9: (CQI) Scatter plot of AMV pressure for each center vs. EUM pressure.

## Rawinsonde comparison

The comparison of all AMVs to nearby rawinsondes is summarized in ??? (QINF >= 50) and ??? (QIWF >= 50). Also, the AMVs are grouped into three layers (high, medium, low) for both QINF (??? To ???) and QIWF (??? To ???), and as collocated AMVs for all levels (??? To ???).

Note: The sample was rather small for some AMV producers (the range was from 60 to 2500 AMV matches to rawinsondes).

The vector RMS ranges from 6 ms-1 (NWC) to 9 ms-1 (BRZ, EUM, CMA) and the speed RMS ranges from 4 ms-1 (NWC) to 7 ms-1 (EUM, CMA). A similar behaviour is also seen when the AMVs are grouped by height. Since the tracking differences in Experiment 1 were small, these large RMS differences are probably due to the substantial variation in height assignment (see ??? and ???). The coldest temperature will be computed by NWC, followed by JMA, BRZ, KMA, and NOA. We do not have enough detail of the technique used by EUM and CMA to define the brightness temperature used in the height assignment.

There are not substantial differences when collocated data are considered. Vector RMS ranges from 6 ms-1 (with NWC, JMA, KMA showing the best results) to 9 ms-1 (with EUM showing the worst results). Correspondingly, the speed RMS ranges from 3-4 ms-1 (for NWC, JMA, KMA) to 6 ms-1 (for EUM). This ranking agrees with the coldest temperatures found for NWC, JMA, and KMA. Meanwhile NOA has the warmest (therefore, the lowest) high-level clouds. It is suspected that EUM will also have a very warm temperature based on the comparison statistics and histogram of heights.

Because this is a small sample of data (only one time period), the centers with the best and worst comparison to rawinsondes vary depending on considering all AMVs, grouped by height, or collocated winds. However, closer examination reveals some consistency. For example, JMA has a very good speed bias as compared to rawinsondes in all categories in the tables below, except for the mid-level winds due to a very low count in AMVs. Similarly for EUM, which has very good direction bias, except in cases of very few AMVs.

Table 7-7: Experiment 1: All AMVs (QI no forecast >= 50) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 834 | 0.57 | 14.51 | -1.23 | 10.19 | -11.72 | 12.60 |
| EUM | 1144 | -1.57 | 15.85 | -1.41 | 11.02 | 4.57 | 24.19 |
| JMA | 807 | -0.36 | 13.53 | -0.93 | 4.08 | -0.26 | 5.66 |
| KMA | 917 | -1.16 | 15.94 | 0.10 | 8.74 | -3.93 | 11.20 |
| NOA | 511 | -1.03 | 14.16 | -0.71 | 5.26 | 0.93 | 7.38 |
| NWC | 132 | -0.75 | 15.21 | -0.88 | 5.05 | -4.68 | 6.85 |

Table 7-8: Experiment 1: All AMVs (QI no forecast >= 80) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 532 | 0.34 | 14.13 | 0.61 | 5.81 | -13.87 | 8.61 |
| EUM | 772 | -0.90 | 15.89 | -2.00 | 7.36 | 6.45 | 8.91 |
| JMA | 433 | 0.75 | 12.89 | -1.16 | 4.18 | 0.74 | 5.90 |
| KMA | 711 | -0.66 | 15.22 | 0.95 | 7.89 | -3.20 | 10.02 |
| NOA | 427 | -1.49 | 13.96 | -0.89 | 5.42 | 0.45 | 7.52 |
| NWC | 63 | 0.39 | 15.64 | -0.10 | 4.05 | -3.90 | 6.47 |

Table 7-9: Experiment 1: High-level AMVs (QI no forecast >= 50) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 332 | -0.04 | 12.31 | -5.18 | 14.73 | -1.73 | 17.02 |
| EUM | 612 | -1.13 | 14.82 | -1.45 | 12.94 | 2.01 | 31.45 |
| JMA | 482 | 0.30 | 10.39 | -1.60 | 4.51 | -1.40 | 6.14 |
| KMA | 463 | -0.53 | 11.04 | -2.10 | 8.05 | -1.92 | 10.43 |
| NOA | 362 | -1.31 | 11.61 | -0.94 | 5.20 | 3.12 | 7.71 |
| NWC | 108 | 0.38 | 13.07 | -0.96 | 5.32 | -4.65 | 7.17 |

Table 7-10: Experiment 1: High-level AMVs (QI no forecast >= 80) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 184 | -1.08 | 11.14 | -0.63 | 6.61 | -7.37 | 9.50 |
| EUM | 409 | -0.18 | 16.04 | -2.72 | 8.36 | -0.70 | 9.79 |
| JMA | 298 | 1.07 | 10.23 | -1.73 | 4.47 | -1.17 | 6.13 |
| KMA | 376 | -0.56 | 10.75 | -1.37 | 5.16 | -0.98 | 7.31 |
| NOA | 318 | -1.18 | 11.67 | -1.02 | 5.29 | 2.40 | 7.71 |
| NWC | 58 | 0.53 | 14.65 | 0.01 | 4.18 | -4.77 | 6.70 |

Table 7-11: Experiment 1: Mid-level AMVs (QI no forecast >= 50) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 193 | -1.52 | 13.35 | 1.26 | 7.15 | -42.38 | 11.72 |
| EUM | 64 | 1.93 | 17.40 | -4.14 | 10.84 | -2.81 | 12.57 |
| JMA | 108 | -2.22 | 15.97 | -1.44 | 3.98 | -1.68 | 6.14 |
| KMA | 241 | -4.39 | 20.30 | 3.72 | 11.73 | -16.30 | 14.49 |
| NOA | 13 | 15.47 | 21.51 | -7.22 | 14.09 | 2.51 | 15.14 |
| NWC | 1 | -4.00 | 4.00 | 2.57 | 2.57 | 0.18 | 2.57 |

Table 7-12: Experiment 1: Mid-level AMVs (QI no forecast >= 80) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 134 | -1.41 | 11.58 | 0.97 | 6.98 | -46.01 | 11.02 |
| EUM | 44 | 1.69 | 19.32 | -3.61 | 10.80 | 4.16 | 12.58 |
| JMA | 36 | -2.65 | 15.76 | -2.64 | 5.20 | -1.16 | 8.55 |
| KMA | 186 | -3.11 | 19.91 | 5.49 | 12.50 | -17.36 | 14.74 |
| NOA | 11 | 16.14 | 22.82 | -7.22 | 14.90 | 2.29 | 15.92 |
| NWC | 0 | NaN | NaN | NaN | NaN | NaN | NaN |

Table 7-13: Experiment 1: Low-level AMVs (QI no forecast >= 50) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 309 | 2.53 | 17.15 | 1.46 | 3.91 | -3.30 | 5.61 |
| EUM | 468 | -2.62 | 16.90 | -0.99 | 7.86 | 8.93 | 10.74 |
| JMA | 217 | -0.90 | 17.71 | 0.82 | 2.97 | 2.99 | 4.11 |
| KMA | 213 | 1.10 | 19.03 | 0.78 | 5.72 | 5.71 | 8.13 |
| NOA | 136 | -1.89 | 18.70 | 0.54 | 3.61 | -5.03 | 4.95 |
| NWC | 23 | -5.93 | 22.93 | -0.61 | 3.66 | -5.04 | 5.25 |

Table 7-14: Experiment 1: Low-level AMVs (QI no forecast >= 80) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 214 | 2.66 | 17.48 | 1.44 | 3.98 | 0.67 | 5.52 |
| EUM | 319 | -2.17 | 15.15 | -0.86 | 5.02 | 15.93 | 6.88 |
| JMA | 99 | 1.02 | 17.94 | 1.10 | 2.51 | 7.21 | 3.56 |
| KMA | 149 | 2.17 | 17.86 | 1.11 | 5.91 | 8.90 | 8.55 |
| NOA | 98 | -4.46 | 18.68 | 0.27 | 3.53 | -6.09 | 5.03 |
| NWC | 5 | -1.30 | 24.34 | -1.33 | 1.95 | 6.22 | 2.50 |

Table 7-15: Experiment 1: Collocated AMVs (QI no forecast >= 50) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 113 | 0.89 | 15.96 | -1.66 | 9.84 | -33.01 | 12.64 |
| EUM | 110 | -0.75 | 12.42 | -0.57 | 10.81 | -5.72 | 14.32 |
| JMA | 112 | 2.72 | 13.66 | -1.48 | 4.00 | -3.77 | 5.56 |
| KMA | 112 | 0.01 | 11.57 | 0.70 | 5.48 | -9.90 | 7.11 |
| NOA | 112 | -3.10 | 13.12 | -0.32 | 4.30 | -4.85 | 5.75 |
| NWC | 111 | -1.13 | 14.53 | -0.97 | 5.15 | -6.92 | 6.55 |

Table 7-16: Experiment 1: All AMVs (QI with forecast >= 50) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | NaN | NaN | NaN | NaN | NaN | NaN |
| EUM | 999 | -1.41 | 16.10 | -2.22 | 7.61 | 5.70 | 11.06 |
| JMA | 820 | -0.12 | 13.66 | -0.95 | 4.08 | 0.88 | 5.66 |
| KMA | 888 | -1.16 | 15.95 | 0.51 | 7.56 | -3.20 | 9.96 |
| NOA | 509 | -1.06 | 14.21 | -0.87 | 6.27 | 0.65 | 8.09 |
| NWC | 134 | -0.60 | 15.24 | -0.85 | 5.12 | -4.94 | 6.88 |

Table 7-17: Experiment 1: All AMVs (QI with forecast >= 80) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | NaN | NaN | NaN | NaN | NaN | NaN |
| EUM | 561 | -0.24 | 15.86 | -2.59 | 7.54 | -0.71 | 8.80 |
| JMA | 462 | 0.54 | 12.90 | -1.12 | 4.11 | 0.96 | 5.82 |
| KMA | 426 | -0.18 | 13.80 | -0.52 | 5.04 | 0.10 | 6.90 |
| NOA | 391 | -1.56 | 13.96 | -1.40 | 6.51 | -1.00 | 8.39 |
| NWC | 55 | -0.23 | 16.82 | 0.17 | 3.88 | -1.01 | 6.33 |

Table 7-18: Experiment 1: High-level AMVs (QI with forecast >= 50) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | NaN | NaN | NaN | NaN | NaN | NaN |
| EUM | 530 | -1.17 | 15.31 | -3.09 | 8.76 | -0.98 | 13.25 |
| JMA | 467 | 0.32 | 10.15 | -1.54 | 4.36 | -1.27 | 5.97 |
| KMA | 454 | -0.65 | 11.05 | -1.59 | 5.54 | -1.37 | 8.42 |
| NOA | 362 | -1.38 | 11.68 | -1.15 | 6.59 | 2.83 | 8.66 |
| NWC | 110 | 0.55 | 13.14 | -0.93 | 5.39 | -4.96 | 7.20 |

Table 7-19: Experiment 1: High-level AMVs (QI with forecast >= 80) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | NaN | NaN | NaN | NaN | NaN | NaN |
| EUM | 369 | -0.65 | 16.31 | -2.90 | 8.60 | -3.19 | 9.90 |
| JMA | 312 | 0.96 | 10.41 | -1.69 | 4.39 | -1.03 | 6.03 |
| KMA | 274 | -0.73 | 10.69 | -1.03 | 4.84 | -2.14 | 6.80 |
| NOA | 305 | -1.25 | 11.82 | -1.55 | 6.93 | 1.54 | 8.95 |
| NWC | 49 | -0.04 | 16.03 | 0.27 | 4.04 | -1.77 | 6.65 |

Table 7-20: Experiment 1: Mid-level AMVs (QI with forecast >= 50) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | NaN | NaN | NaN | NaN | NaN | NaN |
| EUM | 59 | 2.72 | 17.88 | -3.32 | 10.60 | -0.15 | 12.46 |
| JMA | 117 | -1.54 | 16.08 | -1.95 | 4.84 | -0.52 | 6.88 |
| KMA | 228 | -4.22 | 20.48 | 4.32 | 11.47 | -16.25 | 13.62 |
| NOA | 13 | 15.47 | 21.51 | -7.22 | 14.09 | 2.51 | 15.14 |
| NWC | 1 | -4.00 | 4.00 | 2.57 | 2.57 | 0.18 | 2.57 |

Table 7-21: Experiment 1: Mid-level AMVs (QI with forecast >= 80) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | NaN | NaN | NaN | NaN | NaN | NaN |
| EUM | 37 | 3.17 | 20.42 | -5.35 | 9.12 | 7.16 | 11.18 |
| JMA | 42 | -2.90 | 14.88 | -2.34 | 5.05 | 3.07 | 8.25 |
| KMA | 77 | -1.44 | 19.79 | 0.72 | 6.66 | 0.20 | 8.26 |
| NOA | 9 | 18.25 | 25.01 | -5.63 | 10.78 | 1.06 | 12.17 |
| NWC | 1 | -4.00 | 4.00 | 2.57 | 2.57 | 0.18 | 2.57 |

Table 7-22: Experiment 1: Low-level AMVs (QI with forecast >= 50) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| EUM | 410 | -2.30 | 16.83 | -0.95 | 5.06 | 15.18 | 6.98 |
| JMA | 236 | -0.28 | 17.78 | 0.71 | 2.97 | 5.82 | 4.18 |
| KMA | 206 | 1.08 | 19.05 | 0.92 | 5.74 | 7.21 | 8.15 |
| NOA | 134 | -1.79 | 18.81 | 0.50 | 3.62 | -5.43 | 4.92 |
| NWC | 23 | -5.93 | 22.93 | -0.61 | 3.66 | -5.04 | 5.25 |

Table 7-23: Experiment 1: Low-level AMVs (QI with forecast >= 80) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| EUM | 155 | -0.09 | 13.31 | -1.18 | 3.17 | 3.33 | 4.13 |
| JMA | 108 | 0.68 | 17.69 | 1.00 | 2.59 | 5.91 | 3.64 |
| KMA | 75 | 3.14 | 16.16 | 0.05 | 3.60 | 8.16 | 5.63 |
| NOA | 77 | -5.12 | 19.05 | -0.35 | 3.35 | -11.33 | 4.73 |
| NWC | 5 | -1.30 | 24.34 | -1.33 | 1.95 | 6.22 | 2.50 |

Table 7-24: Experiment 1 collocated AMVs with QI >= 50 (with forecast) compared to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | Pbias | PRMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| EUM | 121 | -0.79 | 13.05 | -1.84 | 5.77 | -5.87 | 7.69 |
| JMA | 121 | 2.77 | 13.53 | -1.81 | 4.15 | -2.23 | 6.09 |
| KMA | 122 | -0.61 | 11.65 | 0.94 | 6.08 | -7.59 | 8.03 |
| NOA | 123 | -2.30 | 13.03 | -0.46 | 4.36 | -3.17 | 6.44 |
| NWC | 120 | -0.67 | 14.61 | -0.86 | 5.10 | -5.89 | 6.92 |

Table 7-25: Experiment 1: All AMVs CQIs ( >=50)

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 774 | 0.90 | 14.68 | -1.28 | 10.00 | -13.13 | 12.61 |
| EUM | 378 | -2.06 | 15.65 | -1.99 | 9.52 | 6.20 | 12.45 |
| JMA | 400 | 0.50 | 13.81 | -0.91 | 3.95 | 1.30 | 5.74 |
| KMA | 955 | -1.14 | 15.97 | 0.08 | 7.56 | -3.42 | 9.78 |
| NOA | 512 | -1.05 | 14.15 | -0.86 | 6.29 | 1.10 | 8.16 |
| NWC | 163 | -0.69 | 15.11 | -1.14 | 4.99 | -1.47 | 6.80 |

Table 7-26: Experiment 1: All AMVs CQIs (>=80)

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 448 | 0.02 | 13.97 | 0.31 | 6.07 | -14.61 | 8.62 |
| EUM | 52 | -1.97 | 18.22 | -3.04 | 8.13 | 4.80 | 9.65 |
| JMA | 344 | 0.78 | 13.77 | -1.07 | 4.07 | 1.09 | 5.93 |
| KMA | 697 | -0.58 | 14.24 | 0.58 | 7.58 | -3.29 | 9.70 |
| NOA | 427 | -1.49 | 13.96 | -0.89 | 5.42 | 0.45 | 7.52 |
| NWC | 132 | -0.41 | 15.08 | -0.97 | 5.01 | -5.98 | 6.94 |

Table 7-27: Experiment 1: High-level AMVs CQI (>=50)

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 313 | 0.32 | 12.90 | -5.22 | 14.36 | -5.64 | 16.92 |
| EUM | 191 | -1.83 | 13.67 | -2.08 | 11.86 | -0.80 | 15.51 |
| JMA | 234 | 1.32 | 10.75 | -1.87 | 4.35 | -1.71 | 6.10 |
| KMA | 458 | -0.59 | 11.01 | -1.64 | 5.97 | -0.78 | 8.60 |
| NOA | 363 | -1.33 | 11.61 | -1.16 | 6.62 | 3.35 | 8.74 |
| NWC | 125 | 0.11 | 12.91 | -1.22 | 5.32 | -5.26 | 7.17 |

Table 7-28: Experiment 1: High-level AMVs CQI (>=80)

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 166 | -1.45 | 11.77 | -0.92 | 7.30 | -6.10 | 9.75 |
| EUM | 22 | 4.20 | 18.56 | -3.11 | 9.64 | -11.67 | 11.22 |
| JMA | 215 | 1.38 | 10.99 | -1.91 | 4.45 | -1.50 | 6.25 |
| KMA | 382 | -0.23 | 10.26 | -1.36 | 4.97 | -2.15 | 7.11 |
| NOA | 318 | -1.18 | 11.67 | -1.02 | 5.29 | 2.40 | 7.71 |
| NWC | 113 | 0.49 | 13.34 | -1.03 | 5.28 | -6.06 | 7.22 |

Table 7-29: Experiment 1: Mid-level AMVs CQI (>=50)

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 177 | -1.23 | 12.70 | 1.47 | 7.00 | -45.87 | 11.88 |
| EUM | 20 | -3.89 | 22.28 | -10.02 | 13.18 | 10.70 | 15.41 |
| JMA | 41 | -3.52 | 16.19 | -2.13 | 4.62 | -0.10 | 7.99 |
| KMA | 231 | -4.34 | 19.52 | 3.97 | 11.48 | -16.23 | 13.61 |
| NOA | 13 | 15.47 | 21.51 | -7.22 | 14.09 | 2.51 | 15.14 |
| NWC | 2 | -7.75 | 8.61 | -2.46 | 5.60 | 15.33 | 5.76 |

Table 7-30: Experiment 1: Mid-level AMVs CQI (>=80)

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 105 | -1.58 | 11.24 | 0.72 | 7.00 | -52.63 | 10.96 |
| EUM | 3 | -26.54 | 32.81 | -12.71 | 17.61 | 7.17 | 18.41 |
| JMA | 29 | -3.82 | 17.07 | -2.57 | 5.10 | 1.19 | 9.03 |
| KMA | 165 | -3.44 | 19.66 | 5.32 | 12.89 | -19.02 | 15.17 |
| NOA | 11 | 16.14 | 22.82 | -7.22 | 14.90 | 2.29 | 15.92 |
| NWC | 1 | -4.00 | 4.00 | 2.57 | 2.57 | 0.18 | 2.57 |

Table 7-31: Experiment 1: Low-level AMVs CQI (>50)

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 284 | 2.87 | 17.42 | 1.34 | 3.86 | -0.98 | 5.44 |
| EUM | 167 | -2.10 | 16.77 | -0.93 | 4.86 | 13.68 | 6.85 |
| JMA | 125 | 0.27 | 17.54 | 1.29 | 2.75 | 7.41 | 3.83 |
| KMA | 266 | 0.68 | 19.38 | -0.34 | 5.44 | 3.16 | 7.43 |
| NOA | 136 | -1.89 | 18.70 | 0.54 | 3.61 | -5.03 | 4.95 |
| NWC | 36 | -3.07 | 21.25 | -0.78 | 3.60 | 10.74 | 5.40 |

Table 7-32: Experiment 1: Low-level AMVs CQI (>80)

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 177 | 2.34 | 17.01 | 1.22 | 3.77 | -0.05 | 5.24 |
| EUM | 27 | -4.26 | 15.46 | -1.91 | 4.15 | 17.95 | 6.27 |
| JMA | 100 | 0.83 | 17.56 | 1.16 | 2.62 | 6.63 | 3.66 |
| KMA | 150 | 1.68 | 15.77 | 0.33 | 4.64 | 11.12 | 7.43 |
| NOA | 98 | -4.46 | 18.68 | 0.27 | 3.53 | -6.09 | 5.03 |
| NWC | 18 | -5.81 | 23.44 | -0.81 | 3.00 | -5.82 | 5.06 |

Table 7-33: Experiment 1: Collocated AMVs CQI

| Site | N | Pbias | PRMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 93 | -0.19 | 13.20 | -1.12 | 9.97 | -28.09 | 14.21 |
| EUM | 90 | -2.17 | 9.94 | 3.46 | 20.28 | 6.84 | 26.80 |
| JMA | 94 | 1.80 | 12.46 | -1.10 | 3.35 | -0.46 | 5.18 |
| KMA | 95 | 0.06 | 10.57 | 0.03 | 4.22 | -8.12 | 6.64 |
| NOA | 95 | -2.33 | 11.35 | -0.81 | 4.11 | -4.45 | 5.80 |
| NWC | 92 | -0.78 | 11.64 | -1.09 | 5.05 | -4.79 | 6.52 |

## Model Grid comparison

The Python scripts that we used to find the Best Fit also output comparison of all AMVs to the background grid. This comparison was based on 12-hour forecast using QINF >= 80.  The tables are for all AMVs (???), all AMVs by height range (high, medium, and low) (??? To ???), and collocated AMVs for all levels (???).

The results are very similar to what was found with the rawinsonde comparisons: NWC and JMA have the lowest error, while BRZ and EUM have the highest errors. Considering collocated data, NWC, JMA, KMA show again the best results while EUM shows again the worst results.

QINF

Table 7-34: Experiment 1 all AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 41304 | 8805 | 6.20 | 8.69 | 5.54 | 8.31 |
| EUM | 50629 | 15779 | 4.99 | 7.13 | 4.04 | 6.42 |
| JMA | 34160 | 12656 | 2.53 | 3.13 | 2.26 | 2.92 |
| KMA | 46597 | 13384 | 5.45 | 7.50 | 4.60 | 6.95 |
| NOA | 30377 | 10965 | 3.98 | 5.24 | 3.10 | 4.48 |
| NWC | 7286 | 2646 | 4.26 | 5.30 | 3.28 | 4.38 |

Table 7-35: Experiment 1 high-level AMVs (<400 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 16360 | 5809 | 7.18 | 9.66 | 5.92 | 8.94 |
| EUM | 30125 | 12531 | 5.68 | 7.92 | 4.40 | 7.01 |
| JMA | 17217 | 9031 | 2.93 | 3.63 | 2.58 | 3.35 |
| KMA | 21823 | 9689 | 5.31 | 6.90 | 4.05 | 5.95 |
| NOA | 19002 | 8880 | 4.91 | 6.18 | 3.73 | 5.24 |
| NWC | 4636 | 2277 | 4.69 | 5.77 | 3.39 | 4.59 |

Table 7-36: Experiment 1 mid-level AMVs (400-700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 8196 | 683 | 9.65 | 12.25 | 9.38 | 12.15 |
| EUM | 6405 | 1280 | 5.98 | 7.99 | 5.29 | 7.58 |
| JMA | 6389 | 1463 | 2.65 | 3.09 | 2.46 | 2.94 |
| KMA | 12656 | 2002 | 7.01 | 9.38 | 6.43 | 9.07 |
| NOA | 752 | 306 | 5.10 | 6.50 | 4.15 | 5.88 |
| NWC | 633 | 138 | 5.39 | 6.41 | 4.72 | 5.96 |

Table 7-37: Experiment 1 low-level AMVs (>700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 16649 | 2311 | 3.50 | 4.53 | 3.24 | 4.36 |
| EUM | 14066 | 1962 | 3.05 | 4.42 | 2.69 | 4.12 |
| JMA | 10526 | 2151 | 1.79 | 2.12 | 1.63 | 1.99 |
| KMA | 12060 | 1676 | 4.05 | 6.19 | 3.66 | 5.97 |
| NOA | 10609 | 1775 | 2.22 | 2.69 | 1.91 | 2.36 |
| NWC | 2013 | 229 | 2.93 | 3.46 | 2.57 | 3.13 |

Table 7-38: Experiment 1 collocated AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 5556 | 1341 | 5.87 | 8.41 | 5.23 | 8.09 |
| EUM | 5556 | 1806 | 3.93 | 4.90 | 3.18 | 4.28 |
| JMA | 5556 | 2006 | 2.47 | 2.95 | 2.22 | 2.75 |
| KMA | 5556 | 1737 | 4.30 | 5.53 | 3.54 | 4.96 |
| NOA | 5556 | 1957 | 3.43 | 4.29 | 2.75 | 3.72 |
| NWC | 5556 | 1953 | 3.97 | 4.72 | 3.12 | 3.98 |

QIWF

Table 7-39: Experiment 1 all AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | 0 | NaN | NaN | NaN | NaN |
| EUM | 40816 | 13703 | 3.99 | 5.25 | 3.12 | 4.44 |
| JMA | 35977 | 12838 | 2.68 | 3.31 | 2.41 | 3.10 |
| KMA | 29243 | 9151 | 3.28 | 4.11 | 2.73 | 3.66 |
| NOA | 28418 | 10457 | 3.73 | 4.89 | 2.89 | 4.13 |
| NWC | 6570 | 2411 | 3.95 | 4.98 | 3.04 | 4.11 |

Table 7-40: Experiment 1 high-level AMVs (<400 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | 0 | NaN | NaN | NaN | NaN |
| EUM | 23993 | 11033 | 4.65 | 5.90 | 3.45 | 4.84 |
| JMA | 18154 | 9161 | 3.09 | 3.81 | 2.73 | 3.54 |
| KMA | 12346 | 6384 | 3.18 | 3.79 | 2.41 | 3.12 |
| NOA | 17410 | 8441 | 4.62 | 5.78 | 3.47 | 4.84 |
| NWC | 4114 | 2096 | 4.36 | 5.47 | 3.14 | 4.32 |

Table 7-41: Experiment 1 mid-level AMVs (400-700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | 0 | NaN | NaN | NaN | NaN |
| EUM | 5075 | 1147 | 4.81 | 6.21 | 4.10 | 5.69 |
| JMA | 6864 | 1484 | 2.82 | 3.29 | 2.63 | 3.14 |
| KMA | 7673 | 1520 | 4.29 | 5.33 | 3.75 | 4.92 |
| NOA | 720 | 303 | 4.87 | 6.13 | 3.90 | 5.46 |
| NWC | 593 | 130 | 5.00 | 5.98 | 4.39 | 5.56 |

Table 7-42: Experiment 1 low-level AMVs (>700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | 0 | NaN | NaN | NaN | NaN |
| EUM | 11725 | 1517 | 2.27 | 2.81 | 2.02 | 2.56 |
| JMA | 10928 | 2180 | 1.92 | 2.30 | 1.76 | 2.17 |
| KMA | 9196 | 1238 | 2.57 | 3.25 | 2.30 | 3.04 |
| NOA | 10274 | 1709 | 2.13 | 2.58 | 1.83 | 2.27 |
| NWC | 1858 | 182 | 2.69 | 3.19 | 2.40 | 2.92 |

Table 7-43: Experiment 1 collocated AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | n/a | n/a | n/a | n/a | n/a | n/a |
| EUM | 4935 | 1759 | 3.67 | 4.77 | 2.87 | 3.98 |
| JMA | 4935 | 1860 | 2.41 | 2.89 | 2.16 | 2.68 |
| KMA | 4935 | 1782 | 3.22 | 3.93 | 2.62 | 3.43 |
| NOA | 4935 | 1835 | 3.32 | 4.32 | 2.56 | 3.54 |
| NWC | 4935 | 1819 | 3.76 | 4.75 | 2.91 | 3.91 |

CQI

Table 7-44: Experiment 1 all AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 37180 | 8522 | 5.85 | 8.28 | 5.16 | 7.87 |
| EUM | 4418 | 1046 | 4.68 | 6.74 | 3.85 | 6.03 |
| JMA | 29740 | 10482 | 2.63 | 3.26 | 2.37 | 3.07 |
| KMA | 45211 | 13421 | 5.20 | 7.10 | 4.34 | 6.51 |
| NOA | 30377 | 10965 | 3.98 | 5.24 | 3.10 | 4.48 |
| NWC | 11960 | 4289 | 4.29 | 5.24 | 3.32 | 4.34 |

Table 7-45: Experiment 1 high-level AMVs (<400 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 15093 | 5697 | 6.78 | 9.10 | 5.48 | 8.31 |
| EUM | 2491 | 759 | 5.34 | 7.44 | 4.27 | 6.54 |
| JMA | 14527 | 7397 | 3.06 | 3.79 | 2.71 | 3.54 |
| KMA | 22060 | 9874 | 5.13 | 6.60 | 3.89 | 5.62 |
| NOA | 19002 | 8880 | 4.91 | 6.18 | 3.73 | 5.24 |
| NWC | 8083 | 3723 | 4.63 | 5.60 | 3.41 | 4.49 |

Table 7-46: Experiment 1 mid-level AMVs (400-700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 6511 | 591 | 9.78 | 12.47 | 9.50 | 12.38 |
| EUM | 596 | 85 | 5.42 | 7.83 | 4.90 | 7.43 |
| JMA | 5622 | 1185 | 2.75 | 3.19 | 2.57 | 3.05 |
| KMA | 11521 | 1874 | 6.80 | 9.12 | 6.20 | 8.80 |
| NOA | 752 | 306 | 5.10 | 6.50 | 4.15 | 5.88 |
| NWC | 956 | 186 | 5.24 | 6.27 | 4.66 | 5.88 |

Table 7-47: Experiment 1 low-level AMVs (>700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 15482 | 2230 | 3.24 | 4.13 | 2.98 | 3.94 |
| EUM | 1330 | 202 | 3.11 | 4.46 | 2.59 | 3.97 |
| JMA | 9571 | 1891 | 1.91 | 2.28 | 1.75 | 2.16 |
| KMA | 11576 | 1657 | 3.73 | 5.53 | 3.33 | 5.29 |
| NOA | 10609 | 1775 | 2.22 | 2.69 | 1.91 | 2.36 |
| NWC | 2914 | 377 | 3.03 | 3.55 | 2.62 | 3.18 |

Table 7-48: Experiment 1 collocated AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 2672 | 569 | 4.73 | 6.32 | 4.23 | 6.03 |
| EUM | 2672 | 674 | 3.76 | 4.61 | 3.11 | 4.03 |
| JMA | 2672 | 759 | 2.48 | 2.89 | 2.25 | 2.71 |
| KMA | 2672 | 668 | 4.01 | 4.97 | 3.36 | 4.45 |
| NOA | 2672 | 740 | 3.38 | 4.21 | 2.87 | 3.81 |
| NWC | 2672 | 752 | 3.75 | 4.40 | 3.05 | 3.79 |

## Best fit height

The Best Fit height analysis was completed for each wind producer according to the method described by Salonen et al. (2012). This technique finds the background model best-fit pressure associated with the AMV, which is where the vector difference between the observed AMV and model background is at a minimum.

It does this by first locating the model pressure levels within the troposphere up to 150 hPa, which is a tunable parameter, above and below the AMV.  Using a parabolic fit, it then analyzes the located model pressure levels and detects the single model pressure level which has the minimum vector difference, which must be both less than or equal to 4 ms-1 and at least 2 ms-1 smaller than the vector differences +/- 100 hPa from the best-fit pressure level.  Therefore, this method is dependent on the model vertical resolution.  It is possible using a requirement of at least 2 ms-1 smaller than the vector differences +/- 100 hPa from the best-fit pressure level is too demanding of a requirement.

Similar to previous studies, the number of best-fit matches was generally less than 30% of the AMVs (Salonen et al. 2012).

??? to ??? show the:

- Distribution of Best Fit minus AMV pressure differences, color-coded by low, medium, and high clouds (upper-left),

- Spatial distribution with same color coding (upper-right),

- Relationship between AMV pressure and latitude, color-coded to indicate if the Best fit moved the AMV higher (red) or lower (blue) (lower-left),

- Relationship between AMV pressure and speed, color-coded to indicate if the Best fit moved the AMV higher (red) or lower (blue) (lower-right).

Figure 7-10: BRZ (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-11: BRZ (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-12: EUM (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-13: EUM (QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-14: EUM (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-15: JMA (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-16: JMA (QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-17: JMA (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-18: KMA (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-19: KMA (QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-20: KMA (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-21: NOA (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-22: NOA (QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-23: NOA (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-24: NWC (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-25: NWC (QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 7-26: NWC (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

The depiction of the distribution of Best Fit statistics is located in ??? to ???.  Depending on the site, 16% to 24% of the AMVs are adjusted to a Best Fit pressure (lower-left in each figure). This pie chart shows the fraction of AMVs with found best-fit, not constrained (another minimum was found close by), or did not meet minimum vector difference limits (which was 4 ms-1). Results of the Best Fit algorithm show an approximate Gaussian distribution of the pressure difference centered near zero (upper-right panel in figures) extending ±200 hPa. There does not appear to be a relationship between latitude or longitude and adjusted pressure (upper-left and middle-left panels in figures).

The lower-right panels depict the geographic distribution where red dots indicate the AMV should be higher and blue where it should be lower in the atmosphere. JMA (???) shows a spatial pattern in the Best Fit shift of the AMVs: high clouds are moved higher (cyclone northwest of Africa) and low clouds are moved lower (marine stratus) in the mid-South Atlantic Ocean.

In the upper-right corner of the figures is the distribution of the pressure difference (AMV Best Fit pressure minus the Original Pressure). For sites BRZ, CMA, EUM, NOA, and KMA the pressure difference is centered near zero. However, JMA has two peaks (-50 and  +100) with a minimum near zero and NWC is slightly skewed to the right of zero. For experiment 2, since the AMV heights were assigned using only the IR brightness temperature, these offset and skewed distributions may be the result of the specific implementation of the IR brightness temperature height assignment, shown to be erroneous.

Figure 7-27: BRZ (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-28: BRZ (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-29: EUM (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-30: EUM (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-31: EUM (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-32: JMA (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-33: JMA (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-34: JMA (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-35: KMA (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-36: KMA (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-37: KMA (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-38: NOA (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-39: NOA (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-40: NOA (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-41: NWC (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-42: NWC (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 7-43: NWC (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Additional figures were generated to show the distribution of the differences between the AMVs and the background grid, before and after applying the Best Fit algorithm. The lower half of ??? to ??? shows the change in the speed difference distribution before (left) and after (right) Best Fit adjustment. As expected, the speed bias is usually reduced, as the standard deviation.

Similarly, the lower half of Figure 17-52 to Figure 17-66 shows the change in the vector difference distribution before (left) and after (right) Best Fit adjustment and as expected, the vector difference and standard deviation is usually reduced.  BRZ and CMA have the largest deviation before the best fit, with a vector difference of over 7 ms-1, as compared to the background grid. NWC has the smallest deviation before the best fit, with a vector difference of less than 4 ms-1.

## Statistical comparison

To quantify the observed differences in the previous plots, a paired t-test was used with all combinations of producers and parameters to determine if the differences were statistically significant for collocated AMVs. This was done because there is no ‘ground truth’ for AMVs and one of the goals of the project is to determine the similarity in the AMVs from the different centers.

The statistics were computed using the Matlab t-test function, which performs a t-test of the hypothesis that the data come from a distribution with mean zero (see Appendix C: t-test Results for more details). The data in this case were differences between the parameters from each pair of data producers; therefore, a mean of zero is expected. In the following tables, green indicates no statistical difference between the centers at the 95% confidence level and red symbolizes a statistical difference.

To quantify the observed differences in the previous plots, a paired t-test is once again used with all combinations of producers and parameters.  Further information about the test is located in the section for Experiment 2 Statistical comparison. ??? to ??? summarize the results in terms of statistical significance for each of the five variables; green signifies not statistically different while red is statistically different.

AMVs were first quality controlled, retaining only those with a QINF >= 50. For collocation the distance threshold is 35 km, resulting in 7050 AMVs.

NWC SAF and EUMETSAT are the only combinations very close in terms of both speed and direction, despite having a cloud height bias of 130 hPa.

Table 7-49: Experiment 1 QINF 50 speed t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-50: Experiment 1 QINF 50 direction t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-51: Experiment 1 QINF 50 pressure t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA did not report speed.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-52: Experiment 1 QINF 50 QI t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA was not included.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-53: Experiment 1 QINF 80 speed t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-54: Experiment 1 QINF 80 direction t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-55: Experiment 1 QINF 80 pressure t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA did not report speed.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-56: Experiment 1 QINF 80 QI t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA was not included.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-57: Experiment 1 QIWF 50 speed t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 7-58: Experiment 1 QIWF 50 direction t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 7-59: Experiment 1 QIWF 50 pressure t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA did not report speed.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 7-60: Experiment 1 QIWF 50 QI t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA was not included.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 7-61: Experiment 1 QIWF 80 speed t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 7-62: Experiment 1 QIWF 80 direction t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 7-63: Experiment 1 QIWF 80 pressure t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA did not report speed.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 7-64: Experiment 1 QIWF 80 QI t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA was not included.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 7-65: Experiment 1 CQI 50 speed t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-66: Experiment 1 CQI 50 direction t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-67: Experiment 1 CQI 50 pressure t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA did not report speed.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-68: Experiment 1 CQI 50 QI t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA was not included.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-69: Experiment 1 CQI 80 speed t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-70: Experiment 1 CQI 80 direction t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-71: Experiment 1 CQI 80 pressure t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA did not report speed.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 7-72: Experiment 1 CQI 80 QI t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA was not included.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

# Experiment 2

AMV producers extract IR10.8 channel AMVs considering their standard AMV algorithm configuration, but only using the MSG/SEVIRI IR10.8 images and the ECMWF model data for the Height assignment. This experiment will be used to test the Target selection, Tracking and Quality control steps in all AMV algorithms.

The standard AMV configuration defines target scene size, search scene size, etc. to be the typical settings used by each AMV producers.

For each one of the AMV producer’s datasets, a distribution of AMV speed, direction, vector height, and QI will be generated. Differences of these quantities between AMV producers will be made. Also, differences in AMV coverage and number of vectors will be presented as bulk statistics and geographic plots.

Collocated AMVs from the different algorithms will be used to measure the differences. Also, a comparison of the AMVs with the NWP model winds and height assignment investigations using NWP model best-fit pressure will be used for verification.

Experiment 2 Highlights

The bulk distribution of AMV height was highly variable among the different centers, which was surprising since they are required to use only the IR brightness temperature. This indicates the variability is likely due to how the representative TB was determined.

There are 7050 collocated AMVs (QI no forecast > 50). EUM was not statistically different with NWC and JMA and differences among all other centers are usually 0.3 to 1.0 ms-1. Assigned pressures are all statistically different with differences ranging from 30 to 80 hPa. However, the largest differences appear when compared to EUM: up to 130 hPa.

When the AMVs are compared to rawinsondes, NWC has the lowest error while BRZ and EUM have the highest: Vector RMS ranges from 6 ms-1 (NWC) to 9 ms-1 (BRZ, EUM) and the speed RMS ranges from 4.5 ms-1 (NWC) to 7 ms-1 (EUM).

For EUM:

- The distribution of AMV heights (high-level winds are too low; low-level winds are too high),

- The large differences of heights compared to other centers, and

- Large errors compared to rawinsondes,

all point to the IR brightness temperature height assignment as not performing well.

Considering collocated data there are not significant changes in the validation statistics. NWC, JMA and KMA show the best results while EUM shows again the worst results.

## Approach

This experiment tested the target selection and tracking from the different winds producers. The AMVs should have been the best in terms of speed and direction from each producer, as the standard configuration for target and search boxes was used. However, only a single channel was used for cloud height.

This test was similar to Study 1 in many ways (Genkova et al. 2008):

- Only SEVIRI 10.8 µm channel was used for the height assignment.

- Producers used their algorithm and operational settings

- ECMWF grids were used (although, in the first part of Study 1 the producers used their usual grids).

- Collocated AMVs in Study 1 were within 0.5° of latitude and longitude; in this study the distance was 55 km. Distance provides a more precise collocation, especially in high latitudes.

The Genkova scripts, now including NWC SAF, were used to do similar comparison and analysis as before.

In addition, two new analysis methods were used to quantify any observed differences between producers:

- The best fit analysis was used to further analyze differences in cloud heights,

- Computed paired t-tests were used to determine if the observed differences in collocated AMVs were statistically significant.

## Parameter distributions

The bulk statistics are presented in both tables and histograms, for QINF >= 50 and QIWF >= 50 (because not all centers reported both QINF and QIWF). Note: Since there were little differences in filtering based on QINF or QIWF, QINF >= 50 will be used for the following discussions.

Table 8-1 and Table 8-2 list basic Experiment 2 statistics for the AMVs for each winds producer, without and with forecast quality respectively. Most of the values in this table are approximately the same, with the largest variation being in the number of AMVs, ranging from 6000 (JMA) to 90000 (NWC).

Table 8-1: Experiment 2 statistical summary of AMV datasets for QINF >= 50.

|  | EUM | BRZ | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- | --- |
| Total AMVs | 30806 | 74100 | 67324 | 47320 | 60027 | 146780 |
| QI>=50 | 26011 | 67930 | 47232 | 45782 | 55253 | 136999 |
| SPD_min | 2.50 | 3.00 | 2.50 | 3.00 | 2.50 | 2.50 |
| SPD_max | 95.22 | 70.06 | 85.94 | 94.16 | 91.32 | 84.03 |
| SPD_mean | 13.64 | 11.86 | 14.29 | 13.97 | 14.01 | 13.14 |
| P_min | 57.58 | 100.00 | 100.00 | 105.72 | 106.81 | 101.00 |
| P_max | 1009.78 | 1000.00 | 1016.04 | 987.20 | 1000.00 | 971.00 |
| P_mean | 432.39 | 486.23 | 493.32 | 432.01 | 438.53 | 407.66 |
| Low_winds | 26.87 | 29.71 | 30.80 | 33.55 | 18.47 | 25.75 |
| Mid_winds | 13.04 | 21.50 | 20.09 | 2.56 | 29.57 | 10.86 |
| High_winds | 60.09 | 48.79 | 49.11 | 63.89 | 51.96 | 63.38 |
| Low_SPD_min | 2.50 | 3.00 | 2.50 | 3.01 | 2.50 | 2.50 |
| Low_SPD_max | 38.52 | 46.46 | 44.40 | 37.59 | 70.27 | 36.22 |
| Low_SPD_mean | 8.90 | 9.38 | 9.70 | 9.77 | 10.76 | 8.82 |
| Low_P_min | 700.06 | 700.00 | 700.04 | 700.08 | 700.00 | 700.00 |
| Low_P_max | 1009.78 | 1000.00 | 1016.04 | 987.20 | 1000.00 | 971.00 |
| Low_P_mean | 818.82 | 870.10 | 883.95 | 835.02 | 770.58 | 787.16 |
| Mid_SPD_min | 2.51 | 3.07 | 2.50 | 3.04 | 2.50 | 2.54 |
| Mid_SPD_max | 69.67 | 69.46 | 72.05 | 69.25 | 78.19 | 68.28 |
| Mid_SPD_mean | 14.39 | 13.51 | 13.86 | 21.30 | 14.03 | 15.32 |
| Mid_P_min | 400.06 | 400.03 | 400.04 | 400.06 | 400.02 | 400.50 |
| Mid_P_max | 699.73 | 699.97 | 699.97 | 449.98 | 699.98 | 699.50 |
| Mid_P_mean | 526.94 | 528.13 | 527.57 | 426.70 | 547.85 | 534.10 |
| High_SPD_min | 2.50 | 3.04 | 2.51 | 3.00 | 2.50 | 2.50 |
| High_SPD_max | 95.22 | 70.06 | 85.94 | 94.16 | 91.32 | 84.03 |
| High_SPD_mean | 15.60 | 12.65 | 17.35 | 15.88 | 15.14 | 14.53 |
| High_P_min | 57.58 | 100.00 | 100.00 | 105.72 | 106.81 | 101.00 |
| High_P_max | 399.94 | 400.00 | 400.00 | 399.97 | 400.00 | 400.00 |
| High_P_mean | 239.10 | 234.05 | 234.31 | 220.57 | 258.29 | 231.78 |

Table 8-2: Experiment 2 statistical summary of AMV datasets for QIWF  >= 50.

|  | EUM | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- |
| Total AMVs | 30806 | 67324 | 47320 | 60027 | 146780 |
| QI>=50 | 25479 | 46243 | 45477 | 53608 | 136504 |
| SPD_min | 2.50 | 2.50 | 3.00 | 2.50 | 2.50 |
| SPD_max | 95.22 | 85.94 | 94.16 | 91.32 | 84.03 |
| SPD_mean | 13.96 | 14.07 | 14.02 | 14.13 | 13.22 |
| P_min | 57.58 | 100.00 | 105.72 | 106.81 | 101.00 |
| P_max | 1009.78 | 1016.04 | 987.20 | 1000.00 | 971.00 |
| P_mean | 434.01 | 493.84 | 433.40 | 438.99 | 407.82 |
| Low_winds | 27.05 | 30.81 | 33.78 | 18.70 | 25.76 |
| Mid_winds | 13.10 | 20.20 | 2.55 | 29.36 | 10.87 |
| High_winds | 59.85 | 48.98 | 63.67 | 51.93 | 63.36 |
| Low_SPD_min | 2.50 | 2.50 | 3.01 | 2.50 | 2.51 |
| Low_SPD_max | 38.52 | 81.40 | 37.59 | 70.27 | 36.22 |
| Low_SPD_mean | 8.96 | 9.53 | 9.76 | 10.78 | 8.84 |
| Low_P_min | 700.06 | 700.04 | 700.08 | 700.00 | 700.00 |
| Low_P_max | 1009.78 | 1016.04 | 987.20 | 1000.00 | 971.00 |
| Low_P_mean | 818.11 | 884.80 | 835.08 | 770.76 | 787.07 |
| Mid_SPD_min | 2.51 | 2.50 | 3.04 | 2.50 | 2.54 |
| Mid_SPD_max | 69.67 | 72.05 | 69.25 | 74.59 | 68.28 |
| Mid_SPD_mean | 14.60 | 13.57 | 21.42 | 14.15 | 15.41 |
| Mid_P_min | 400.06 | 400.00 | 400.06 | 400.02 | 400.50 |
| Mid_P_max | 699.73 | 699.97 | 449.98 | 699.98 | 699.50 |
| Mid_P_mean | 527.65 | 526.84 | 426.71 | 548.14 | 533.99 |
| High_SPD_min | 2.50 | 2.51 | 3.00 | 2.50 | 2.50 |
| High_SPD_max | 95.22 | 85.94 | 94.16 | 91.32 | 84.03 |
| High_SPD_mean | 16.07 | 17.13 | 15.98 | 15.33 | 14.63 |
| High_P_min | 57.58 | 100.00 | 105.72 | 106.81 | 101.00 |
| High_P_max | 399.94 | 400.00 | 399.97 | 400.00 | 400.00 |
| High_P_mean | 239.93 | 234.29 | 220.60 | 257.80 | 231.94 |

Table 8-3: Experiment 2 statistical summary of AMV datasets for CQI  >= 50

|  | EUM | BRZ | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- | --- |
| Total AMVs | 30806 | 74100 | 67324 | 47320 | 60027 | 146780 |
| QI>=50 | 12117 | 65673 | 27057 | 45782 | 53355 | 146496 |
| SPD_min | 2.50 | 2.55 | 2.51 | 3.00 | 2.50 | 2.50 |
| SPD_max | 90.64 | 70.06 | 85.94 | 94.16 | 97.01 | 84.03 |
| SPD_mean | 12.91 | 12.02 | 14.00 | 13.97 | 14.18 | 12.97 |
| P_min | 61.71 | 100.00 | 125.02 | 105.72 | 106.81 | 101.00 |
| P_max | 999.57 | 1000.00 | 996.47 | 987.20 | 1000.00 | 972.50 |
| P_mean | 443.80 | 487.06 | 497.74 | 432.01 | 436.33 | 409.12 |
| Low_winds | 28.30 | 29.96 | 32.27 | 33.55 | 18.53 | 25.63 |
| Mid_winds | 14.08 | 21.19 | 17.51 | 2.56 | 28.82 | 11.52 |
| High_winds | 57.62 | 48.85 | 50.22 | 63.89 | 52.65 | 62.86 |
| Low_SPD_min | 2.50 | 2.55 | 2.51 | 3.01 | 2.50 | 2.50 |
| Low_SPD_max | 33.78 | 46.46 | 32.67 | 37.59 | 70.27 | 36.22 |
| Low_SPD_mean | 9.24 | 9.39 | 9.14 | 9.77 | 10.79 | 8.76 |
| Low_P_min | 700.06 | 700.00 | 700.04 | 700.08 | 700.00 | 700.00 |
| Low_P_max | 999.57 | 1000.00 | 996.47 | 987.20 | 1000.00 | 972.50 |
| Low_P_mean | 819.08 | 870.56 | 887.37 | 835.02 | 770.50 | 788.38 |
| Mid_SPD_min | 2.56 | 2.55 | 2.51 | 3.04 | 2.50 | 2.51 |
| Mid_SPD_max | 66.96 | 69.46 | 72.05 | 69.25 | 80.01 | 62.00 |
| Mid_SPD_mean | 15.27 | 13.73 | 14.46 | 21.30 | 14.27 | 14.95 |
| Mid_P_min | 400.12 | 400.03 | 400.00 | 400.06 | 400.02 | 400.50 |
| Mid_P_max | 699.71 | 699.97 | 699.97 | 449.98 | 699.98 | 699.50 |
| Mid_P_mean | 526.52 | 528.14 | 533.05 | 426.70 | 548.08 | 535.47 |
| High_SPD_min | 2.51 | 2.59 | 2.52 | 3.00 | 2.50 | 2.50 |
| High_SPD_max | 90.64 | 70.06 | 85.94 | 94.16 | 97.01 | 84.03 |
| High_SPD_mean | 14.14 | 12.88 | 16.97 | 15.88 | 15.31 | 14.33 |
| High_P_min | 61.71 | 100.00 | 125.02 | 105.72 | 106.81 | 101.00 |
| High_P_max | 399.94 | 400.00 | 399.98 | 399.97 | 400.00 | 400.00 |
| High_P_mean | 239.28 | 233.99 | 235.09 | 220.57 | 257.54 | 231.35 |

Table 8-4: Experiment 2 statistical summary of AMV datasets for QINF >= 80.

|  | EUM | BRZ | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- | --- |
| Total AMVs | 30806 | 74100 | 67324 | 47320 | 60027 | 146780 |
| QI>=80 | 20435 | 48916 | 26409 | 40476 | 45280 | 104553 |
| SPD_min | 2.50 | 3.00 | 2.51 | 3.00 | 2.50 | 2.54 |
| SPD_max | 95.22 | 70.06 | 85.66 | 94.16 | 79.89 | 83.89 |
| SPD_mean | 14.71 | 11.49 | 15.57 | 14.51 | 14.76 | 13.97 |
| P_min | 59.63 | 100.00 | 100.00 | 107.49 | 106.81 | 101.00 |
| P_max | 997.16 | 1000.00 | 1011.73 | 970.34 | 1000.00 | 971.00 |
| P_mean | 441.08 | 501.47 | 491.41 | 439.04 | 443.28 | 414.05 |
| Low_winds | 28.74 | 32.53 | 31.71 | 34.66 | 19.90 | 27.12 |
| Mid_winds | 12.06 | 20.71 | 17.03 | 2.45 | 28.69 | 10.14 |
| High_winds | 59.20 | 46.76 | 51.26 | 62.89 | 51.40 | 62.74 |
| Low_SPD_min | 2.50 | 3.00 | 2.54 | 3.01 | 2.50 | 2.54 |
| Low_SPD_max | 31.42 | 46.46 | 32.67 | 36.03 | 70.27 | 36.22 |
| Low_SPD_mean | 9.20 | 9.32 | 9.67 | 9.93 | 11.01 | 9.03 |
| Low_P_min | 700.06 | 700.00 | 700.04 | 700.40 | 700.00 | 700.00 |
| Low_P_max | 997.16 | 1000.00 | 1011.73 | 970.34 | 1000.00 | 971.00 |
| Low_P_mean | 816.96 | 872.41 | 888.71 | 835.62 | 771.78 | 785.81 |
| Mid_SPD_min | 2.54 | 3.07 | 2.51 | 3.04 | 2.50 | 2.60 |
| Mid_SPD_max | 66.96 | 69.46 | 72.05 | 69.25 | 74.59 | 68.28 |
| Mid_SPD_mean | 15.67 | 13.11 | 16.26 | 22.34 | 15.01 | 16.56 |
| Mid_P_min | 400.06 | 400.40 | 400.13 | 400.06 | 400.02 | 400.50 |
| Mid_P_max | 699.73 | 699.97 | 699.97 | 449.94 | 699.98 | 699.50 |
| Mid_P_mean | 527.93 | 528.95 | 529.52 | 426.37 | 550.14 | 534.20 |
| High_SPD_min | 2.51 | 3.07 | 2.52 | 3.00 | 2.50 | 2.60 |
| High_SPD_max | 95.22 | 70.06 | 85.66 | 94.16 | 79.89 | 83.89 |
| High_SPD_mean | 17.19 | 12.28 | 18.99 | 16.73 | 16.08 | 15.68 |
| High_P_min | 59.63 | 100.00 | 100.00 | 107.49 | 106.81 | 101.00 |
| High_P_max | 399.94 | 400.00 | 400.00 | 399.97 | 399.99 | 400.00 |
| High_P_mean | 240.92 | 231.28 | 232.98 | 221.02 | 256.44 | 233.94 |

Table 8-5: Experiment 2 statistical summary of AMV datasets for QIWF  >= 80.

|  | EUM | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- |
| Total AMVs | 30806 | 67324 | 47320 | 60027 | 146780 |
| QI>=80 | 16526 | 27486 | 38293 | 24207 | 94580 |
| SPD_min | 2.50 | 2.51 | 3.00 | 2.50 | 2.60 |
| SPD_max | 95.22 | 85.66 | 94.16 | 76.02 | 83.89 |
| SPD_mean | 15.88 | 15.14 | 14.87 | 16.26 | 14.57 |
| P_min | 59.63 | 100.00 | 107.49 | 107.58 | 102.00 |
| P_max | 997.16 | 1011.73 | 970.34 | 1000.00 | 971.00 |
| P_mean | 453.91 | 488.98 | 445.39 | 483.34 | 417.80 |
| Low_winds | 30.84 | 31.25 | 35.58 | 27.43 | 27.55 |
| Mid_winds | 11.83 | 17.29 | 2.50 | 27.77 | 10.25 |
| High_winds | 57.33 | 51.46 | 61.93 | 44.80 | 62.20 |
| Low_SPD_min | 2.50 | 2.51 | 3.01 | 2.50 | 2.69 |
| Low_SPD_max | 31.42 | 32.67 | 36.03 | 35.05 | 36.22 |
| Low_SPD_mean | 9.47 | 9.49 | 10.01 | 11.43 | 9.27 |
| Low_P_min | 700.06 | 700.04 | 700.40 | 700.03 | 700.00 |
| Low_P_max | 997.16 | 1011.73 | 970.34 | 1000.00 | 971.00 |
| Low_P_mean | 814.85 | 888.82 | 835.80 | 777.77 | 785.72 |
| Mid_SPD_min | 2.58 | 2.51 | 3.05 | 2.52 | 2.77 |
| Mid_SPD_max | 66.96 | 72.05 | 69.25 | 71.93 | 68.28 |
| Mid_SPD_mean | 16.83 | 15.60 | 22.78 | 16.23 | 17.18 |
| Mid_P_min | 400.06 | 400.13 | 400.06 | 400.05 | 400.50 |
| Mid_P_max | 699.73 | 699.97 | 449.94 | 699.98 | 699.50 |
| Mid_P_mean | 527.41 | 529.02 | 426.52 | 548.49 | 532.55 |
| High_SPD_min | 2.51 | 2.51 | 3.00 | 2.50 | 2.60 |
| High_SPD_max | 95.22 | 85.66 | 94.16 | 76.02 | 83.89 |
| High_SPD_mean | 19.13 | 18.41 | 17.34 | 19.23 | 16.49 |
| High_P_min | 59.63 | 100.00 | 107.49 | 107.58 | 102.00 |
| High_P_max | 399.94 | 400.00 | 399.97 | 399.97 | 400.00 |
| High_P_mean | 244.63 | 232.72 | 221.87 | 262.70 | 235.95 |

Table 8-6: Experiment 2 statistical summary of AMV datasets for CQI >= 80.

|  | EUM | BRZ | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- | --- |
| Total AMVs | 30806 | 74100 | 67324 | 47320 | 60027 | 146780 |
| QI>=80 | 2277 | 46166 | 22823 | 40476 | 43505 | 139290 |
| SPD_min | 2.52 | 2.55 | 2.51 | 3.00 | 2.50 | 2.50 |
| SPD_max | 70.69 | 70.06 | 85.66 | 94.16 | 79.89 | 84.03 |
| SPD_mean | 15.52 | 11.89 | 14.64 | 14.51 | 14.90 | 13.11 |
| P_min | 91.30 | 100.00 | 125.02 | 107.49 | 107.58 | 101.00 |
| P_max | 992.62 | 1000.00 | 996.47 | 970.34 | 1000.00 | 971.00 |
| P_mean | 466.63 | 498.29 | 500.55 | 439.04 | 433.33 | 411.08 |
| Low_winds | 27.71 | 32.53 | 32.94 | 34.66 | 19.35 | 26.10 |
| Mid_winds | 21.04 | 19.51 | 16.96 | 2.45 | 26.68 | 11.28 |
| High_winds | 51.25 | 47.96 | 50.10 | 62.89 | 53.97 | 62.62 |
| Low_SPD_min | 2.66 | 2.58 | 2.54 | 3.01 | 2.50 | 2.50 |
| Low_SPD_max | 31.42 | 43.42 | 32.67 | 36.03 | 70.08 | 36.22 |
| Low_SPD_mean | 10.57 | 9.45 | 9.43 | 9.93 | 11.00 | 8.79 |
| Low_P_min | 700.32 | 700.00 | 700.04 | 700.40 | 700.00 | 700.00 |
| Low_P_max | 992.62 | 1000.00 | 996.47 | 970.34 | 1000.00 | 971.00 |
| Low_P_mean | 817.13 | 875.54 | 886.68 | 835.62 | 770.39 | 787.77 |
| Mid_SPD_min | 3.14 | 2.55 | 2.51 | 3.04 | 2.50 | 2.52 |
| Mid_SPD_max | 54.93 | 69.46 | 72.05 | 69.25 | 74.59 | 62.00 |
| Mid_SPD_mean | 18.44 | 13.73 | 15.46 | 22.34 | 15.30 | 15.26 |
| Mid_P_min | 400.68 | 400.26 | 400.13 | 400.06 | 400.02 | 400.50 |
| Mid_P_max | 699.71 | 699.97 | 699.97 | 449.94 | 699.95 | 699.50 |
| Mid_P_mean | 518.48 | 526.36 | 534.75 | 426.37 | 549.98 | 535.79 |
| High_SPD_min | 2.52 | 2.79 | 2.52 | 3.00 | 2.50 | 2.50 |
| High_SPD_max | 70.69 | 70.06 | 85.66 | 94.16 | 79.89 | 84.03 |
| High_SPD_mean | 16.99 | 12.80 | 17.79 | 16.73 | 16.10 | 14.53 |
| High_P_min | 91.30 | 100.00 | 125.02 | 107.49 | 107.58 | 101.00 |
| High_P_max | 399.58 | 400.00 | 399.98 | 399.97 | 400.00 | 400.00 |
| High_P_mean | 255.84 | 230.95 | 235.05 | 221.02 | 254.79 | 231.63 |

Appendix A: Parameter Distribution Histograms shows the bulk histograms for each center.  Major observations of the AMVs for each centre are listed below:

- BRZ (Figure 14-18): The wind direction (lower-left) is not a smooth distribution. Conversely, it has two very sharp peaks. The AMV pressure distribution (lower-right) has peaks at 300 and 770 hPa. Upper-level winds at 300 hPa are reasonable, but the peak at 770 hPa is most likely too high for low-level clouds.

- EUM (Figure 14-19 and Figure 14-24): The AMV pressure distribution has a peak at 500 and 800 hPa. The upper level winds are too low, while the low-level winds are too high.

- JMA (Figure 14-20 and Figure 14-25): The AMV pressure distribution has a peak at 500 and 850 hPa. The upper level winds are too low, while the low-level winds are placed well. There is also a noticeable gap in mid-level winds.

- KMA (Figure 14-21 and Figure 14-26): The AMV pressure distribution has a peak at 450 and 800 hPa. The upper level winds are too low, while the low-level winds are too high.

- NOA (Figure 14-22 and Figure 14-27): The AMV pressure distribution has a peak at 400 and 780 hPa. The upper level winds are too low, while the low-level winds are too high.

- NWC (Figure 14-23 and Figure 14-28): The AMV pressure distribution has a peak at 400 and 780 hPa. The upper level winds are too low, while the low-level winds are too high.

Since only the IR brightness temperature was used in this experiment, the wide variation in cloud heights can be attributed to different techniques and thresholds in determining a representative TB

## Collocation plots

AMVs are first quality controlled, retaining only those with a QINF >= 50. For collocation the distance threshold was 55 km, resulting in 7050 AMVs. The following three figures depict the collocated vectors in terms of parameters (from top to bottom: speed direction, pressure, QI, maximum pressure difference between collocated AMVs, and a scatter plot of AMV pressure from each centre pressure vs. EUM AMV pressure.

Note: There are points plotted on the y-axis in Figure 8-3 and

Figure 8-6, indicating zero pressure values for EUM AMVs. These zero values are also in the original text files from EUMETSAT.

Figure 8-1: Plots of collocated AMVs of speed (top), direction (2nd), pressure (3rd), and QI (bottom) are color-coded based on legend in upper-right. The x-axis is AMV number.

Figure 8-2: The maximum pressure difference between any two collocated AMVs.

Figure 8-3: Scatter plot of AMV pressure for each center vs. EUM pressure.

A similar comparison was done for AMVs with QI with forecast, retaining only those with a QIWF >= 50. Again, the collocation distance threshold was set to 55 km, resulting in 7050 AMVs.

Figure 8-4: Plots of collocated AMVs of speed (top), direction (2nd), pressure (3rd), and QI (bottom) are color-coded based on legend in upper-right. The x-axis is AMV number.

Figure 8-5: The maximum pressure difference between any two collocated AMVs.

Figure 8-6: Scatter plot of AMV pressure for each center vs. EUM pressure.

CQI

Figure 8-7: Plots of collocated AMVs of speed (top), direction (2nd), pressure (3rd), and QI (bottom) are color-coded based on legend in upper-right. The x-axis is AMV number.

Figure 8-8: The maximum pressure difference between any two collocated AMVs.

Figure 8-9: Scatter plot of AMV pressure for each center vs. EUM pressure.

## Rawinsonde comparison

The comparison of all AMVs to nearby rawinsondes is summarized in Table 8-7 (QINF >= 50) and Table 8-16 (QIWF >= 50). Also, the AMVs are grouped into three layers (high, medium, low) for both QINF (Table 8-9 to

Table 8-13) and QIWF (Table 8-18 to Table 8-22), and as collocated AMVs for all levels (Table 8-15 and Table 8-24).

Note: The sample was rather small for some AMV producers (the range was from 60 to 2500 AMV matches to rawinsondes).

The vector RMS ranges from 6 ms-1 (NWC) to 9 ms-1 (BRZ, EUM, CMA) and the speed RMS ranges from 4 ms-1 (NWC) to 7 ms-1 (EUM, CMA). A similar behaviour is also seen when the AMVs are grouped by height. Since the tracking differences in Experiment 1 were small, these large RMS differences are probably due to the substantial variation in height assignment (see Figure 8-2 and Figure 8-5). This height variation can be explained by the technique used to determine a representative brightness temperature, which is detailed in Error! Reference source not found.. The coldest temperature will be computed by NWC, followed by JMA, BRZ, KMA, and NOA. We do not have enough detail of the technique used by EUM and CMA to define the brightness temperature used in the height assignment.

There are not substantial differences when collocated data are considered. Vector RMS ranges from 6 ms-1 (with NWC, JMA, KMA showing the best results) to 9 ms-1 (with EUM showing the worst results). Correspondingly, the speed RMS ranges from 3-4 ms-1 (for NWC, JMA, KMA) to 6 ms-1 (for EUM). This ranking agrees with the coldest temperatures found for NWC, JMA, and KMA. Meanwhile NOA has the warmest (therefore, the lowest) high-level clouds. It is suspected that EUM will also have a very warm temperature based on the comparison statistics and histogram of heights.

Because this is a small sample of data (only one time period), the centers with the best and worst comparison to rawinsondes vary depending on considering all AMVs, grouped by height, or collocated winds. However, closer examination reveals some consistency. For example, JMA has a very good speed bias as compared to rawinsondes in all categories in the tables below, except for the mid-level winds due to a very low count in AMVs. Similarly for EUM, which has very good direction bias, except in cases of very few AMVs.

Table 8-7: Experiment 2: All AMVs (QI no forecast >= 50) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 971 | 1.40 | 14.43 | -2.58 | 11.60 | -11.31 | 13.47 |
| EUM | 460 | -0.58 | 15.71 | -2.47 | 7.05 | 8.87 | 8.71 |
| JMA | 701 | -0.53 | 14.38 | -1.08 | 4.43 | -0.28 | 6.20 |
| KMA | 886 | -1.23 | 14.91 | -1.25 | 7.84 | -5.43 | 10.53 |
| NOA | 691 | -1.58 | 13.93 | -0.90 | 5.44 | 1.89 | 7.62 |
| NWC | 1954 | -1.17 | 16.16 | -1.96 | 5.85 | -0.75 | 7.82 |

Table 8-8: Experiment 2: All AMVs (QI no forecast >= 80) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 676 | 0.72 | 13.34 | -0.09 | 6.82 | -12.06 | 9.36 |
| EUM | 334 | -0.83 | 15.09 | -2.37 | 6.28 | 7.80 | 7.88 |
| JMA | 355 | 0.21 | 13.94 | -1.71 | 4.64 | 2.38 | 6.42 |
| KMA | 689 | -1.02 | 14.51 | -0.78 | 7.50 | -5.36 | 9.76 |
| NOA | 599 | -1.69 | 13.98 | -0.88 | 5.25 | 0.39 | 7.48 |
| NWC | 1450 | -1.04 | 16.06 | -2.03 | 6.01 | -0.88 | 8.07 |

Table 8-9: Experiment 2: High-level AMVs (QI no forecast >= 50) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 459 | 2.06 | 13.71 | -6.96 | 15.79 | 3.88 | 17.11 |
| EUM | 239 | -0.19 | 15.04 | -2.59 | 7.47 | -0.52 | 9.24 |
| JMA | 450 | 0.40 | 10.83 | -1.88 | 4.97 | -0.69 | 6.95 |
| KMA | 518 | -0.32 | 12.53 | -2.75 | 6.84 | -0.16 | 9.53 |
| NOA | 471 | -0.81 | 11.77 | -1.42 | 5.82 | 3.70 | 8.39 |
| NWC | 1303 | -0.11 | 13.77 | -2.19 | 6.13 | -2.78 | 8.40 |

Table 8-10: Experiment 2: High-level AMVs (QI no forecast >= 80) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 288 | 0.53 | 11.28 | -2.12 | 7.94 | 3.60 | 10.03 |
| EUM | 189 | -0.56 | 14.74 | -2.33 | 6.20 | -3.20 | 8.14 |
| JMA | 266 | 1.09 | 11.78 | -2.53 | 4.97 | 0.54 | 6.89 |
| KMA | 418 | -0.27 | 12.36 | -2.42 | 6.04 | -0.41 | 8.34 |
| NOA | 431 | -0.93 | 12.06 | -1.33 | 5.69 | 1.94 | 8.22 |
| NWC | 1037 | -0.18 | 13.82 | -2.16 | 6.11 | -1.91 | 8.45 |

Table 8-11: Experiment 2: Mid-level AMVs (QI no forecast >= 50) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 228 | 1.42 | 11.70 | 1.58 | 7.10 | -45.66 | 11.92 |
| EUM | 25 | 5.65 | 18.27 | -4.80 | 11.48 | 0.61 | 13.10 |
| JMA | 83 | 0.20 | 19.26 | -1.12 | 4.08 | -4.60 | 5.54 |
| KMA | 211 | -4.87 | 16.94 | 1.28 | 11.22 | -21.50 | 14.51 |
| NOA | 19 | 3.27 | 15.59 | -1.75 | 10.79 | -0.15 | 11.26 |
| NWC | 131 | -0.73 | 14.86 | -6.06 | 9.51 | 10.38 | 10.76 |

Table 8-12: Experiment 2: Mid-level AMVs (QI no forecast >= 80) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 169 | -1.66 | 10.32 | 1.53 | 7.62 | -45.42 | 11.72 |
| EUM | 20 | 5.96 | 18.92 | -5.13 | 11.00 | -0.23 | 12.76 |
| JMA | 20 | -1.82 | 17.90 | -3.10 | 4.81 | 0.85 | 6.82 |
| KMA | 165 | -4.68 | 16.20 | 2.57 | 11.19 | -23.11 | 13.76 |
| NOA | 13 | 2.75 | 15.26 | 1.16 | 8.35 | -0.13 | 8.95 |
| NWC | 96 | 0.20 | 15.16 | -6.78 | 10.50 | 10.27 | 11.78 |

Table 8-13: Experiment 2: Low-level AMVs (QI no forecast >= 50) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 284 | 2.61 | 17.26 | 1.16 | 4.08 | -8.29 | 5.74 |
| EUM | 196 | -1.85 | 16.16 | -2.02 | 5.64 | 21.38 | 7.20 |
| JMA | 168 | -3.38 | 19.13 | 1.08 | 2.76 | 2.98 | 3.96 |
| KMA | 157 | 0.67 | 18.73 | 0.27 | 4.81 | -1.19 | 6.57 |
| NOA | 201 | -3.84 | 17.87 | 0.38 | 3.33 | -2.16 | 4.77 |
| NWC | 520 | -3.95 | 21.23 | -0.34 | 3.44 | 1.54 | 4.92 |

Table 8-14: Experiment 2: Low-level AMVs (QI no forecast >= 80) comparison to rawinsondes within 150 km. N= number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS= speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 219 | 2.80 | 17.32 | 1.34 | 4.01 | -6.90 | 5.70 |
| EUM | 125 | -2.32 | 14.93 | -1.99 | 5.28 | 25.71 | 6.32 |
| JMA | 69 | -2.59 | 19.29 | 1.82 | 2.96 | 9.89 | 3.89 |
| KMA | 106 | 1.74 | 18.90 | 0.44 | 5.20 | 2.77 | 7.03 |
| NOA | 155 | -4.17 | 18.21 | 0.20 | 3.22 | -3.86 | 4.68 |
| NWC | 317 | -4.24 | 22.04 | -0.18 | 3.12 | -0.89 | 4.71 |

Table 8-15: Experiment 2: Collocated AMVs (QI no forecast >= 50) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 1573 | 1.12 | 14.22 | -3.00 | 12.31 | -21.34 | 14.34 |
| EUM | 1559 | -0.67 | 15.14 | -1.98 | 6.12 | -4.31 | 7.73 |
| JMA | 1566 | -1.79 | 14.80 | -1.05 | 4.05 | -0.07 | 5.50 |
| KMA | 1565 | -0.14 | 12.99 | -1.69 | 5.79 | -9.54 | 7.71 |
| NOA | 1570 | -2.57 | 13.19 | -0.86 | 5.01 | -2.08 | 6.46 |
| NWC | 1522 | -1.63 | 16.24 | -1.86 | 5.68 | -4.07 | 7.26 |

Table 8-16: Experiment 2: All AMVs (QI with forecast >= 50) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | NaN | NaN | NaN | NaN | NaN | NaN |
| EUM | 462 | -0.33 | 15.71 | -2.56 | 6.94 | 8.51 | 8.77 |
| JMA | 690 | -0.47 | 14.44 | -1.13 | 4.43 | -1.19 | 6.22 |
| KMA | 855 | -1.06 | 14.96 | -1.00 | 7.35 | -5.32 | 9.85 |
| NOA | 686 | -1.81 | 13.84 | -0.87 | 5.26 | 1.25 | 7.44 |
| NWC | 1940 | -1.21 | 16.14 | -1.92 | 5.86 | -1.24 | 7.82 |

Table 8-17: Experiment 2: All AMVs (QI with forecast >= 80) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | NaN | NaN | NaN | NaN | NaN | NaN |
| EUM | 232 | -0.35 | 15.25 | -2.24 | 5.94 | -0.58 | 7.64 |
| JMA | 371 | -0.20 | 14.00 | -1.58 | 4.51 | 0.92 | 6.30 |
| KMA | 312 | -0.29 | 15.63 | -2.43 | 6.30 | -0.61 | 8.40 |
| NOA | 564 | -1.88 | 14.01 | -1.01 | 5.24 | -2.54 | 7.27 |
| NWC | 1271 | -0.86 | 16.18 | -2.14 | 6.08 | -0.66 | 8.16 |

Table 8-18: Experiment 2: High-level AMVs (QI with forecast >= 50) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | NaN | NaN | NaN | NaN | NaN | NaN |
| EUM | 249 | -0.20 | 14.97 | -2.72 | 7.20 | -0.10 | 9.39 |
| JMA | 423 | 0.78 | 10.76 | -1.98 | 4.99 | -0.75 | 7.02 |
| KMA | 509 | -0.20 | 12.54 | -2.47 | 6.12 | -0.41 | 8.90 |
| NOA | 465 | -0.87 | 11.83 | -1.44 | 5.80 | 2.83 | 8.31 |
| NWC | 1306 | -0.15 | 13.78 | -2.15 | 6.12 | -2.68 | 8.36 |

Table 8-19: Experiment 2: High-level AMVs (QI with forecast >= 80) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | NaN | NaN | NaN | NaN | NaN | NaN |
| EUM | 164 | -0.64 | 15.63 | -2.43 | 6.29 | -3.30 | 8.10 |
| JMA | 272 | 0.96 | 11.60 | -2.42 | 4.86 | -0.04 | 6.79 |
| KMA | 207 | -0.44 | 14.76 | -3.34 | 6.65 | 2.00 | 8.84 |
| NOA | 410 | -1.11 | 12.18 | -1.35 | 5.69 | -1.03 | 7.95 |
| NWC | 944 | -0.18 | 14.22 | -2.15 | 6.14 | -1.28 | 8.52 |

Table 8-20: Experiment 2: Mid-level AMVs (QI with forecast >= 50) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | NaN | NaN | NaN | NaN | NaN | NaN |
| EUM | 27 | 5.41 | 18.35 | -4.43 | 11.06 | -0.83 | 12.62 |
| JMA | 90 | -0.27 | 18.62 | -1.27 | 4.14 | -8.58 | 5.64 |
| KMA | 197 | -4.92 | 17.12 | 1.77 | 10.97 | -21.48 | 13.57 |
| NOA | 18 | 2.06 | 14.89 | -0.05 | 8.04 | -0.52 | 8.62 |
| NWC | 132 | -1.44 | 14.63 | -6.09 | 9.51 | 9.58 | 10.75 |

Table 8-21: Experiment 2: Mid-level AMVs (QI with forecast >= 80) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | NaN | NaN | NaN | NaN | NaN | NaN |
| EUM | 14 | 5.08 | 15.83 | -6.82 | 9.69 | 2.28 | 11.73 |
| JMA | 21 | -2.01 | 17.77 | -3.07 | 4.80 | -1.90 | 6.79 |
| KMA | 57 | -4.47 | 16.09 | -0.38 | 6.60 | -11.40 | 8.78 |
| NOA | 14 | -0.15 | 15.84 | 0.28 | 7.84 | -1.06 | 8.46 |
| NWC | 85 | 0.69 | 15.78 | -7.04 | 10.54 | 6.29 | 11.69 |

Table 8-22: Experiment 2: Low-level AMVs (QI with forecast >= 50) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | NaN | NaN | NaN | NaN | NaN | NaN |
| EUM | 186 | -1.34 | 16.25 | -2.08 | 5.69 | 21.38 | 7.06 |
| JMA | 177 | -3.57 | 18.96 | 0.95 | 2.83 | 1.51 | 4.14 |
| KMA | 149 | 1.11 | 18.96 | 0.38 | 4.83 | -0.71 | 6.52 |
| NOA | 203 | -4.31 | 17.52 | 0.36 | 3.28 | -2.21 | 4.72 |
| NWC | 502 | -3.92 | 21.38 | -0.22 | 3.38 | -0.33 | 4.88 |

Table 8-23: Experiment 2: Low-level AMVs (QI with forecast >= 80) comparison to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | NaN | NaN | NaN | NaN | NaN | NaN |
| EUM | 54 | -0.84 | 13.87 | -0.49 | 2.65 | 6.96 | 3.94 |
| JMA | 78 | -3.76 | 19.45 | 1.75 | 2.90 | 5.02 | 3.98 |
| KMA | 48 | 5.31 | 18.46 | -0.90 | 4.02 | 0.98 | 5.53 |
| NOA | 140 | -4.32 | 18.21 | -0.13 | 3.06 | -7.11 | 4.55 |
| NWC | 242 | -4.04 | 22.35 | -0.38 | 2.87 | -0.68 | 4.31 |

Table 8-24: Experiment 2 collocated AMVs with QI >= 50 (with forecast) compared to rawinsondes within 150 km. N = number of matches; P bias = pressure bias; P RMS = pressure RMS; SpdBias = speed bias; SpdRMS = speed RMS; DirBias = wind direction bias; VecRMS = vector RMS. The extreme for each category is highlighted: Yellow = high value; cyan = low value.

| Site | N | Pbias | PRMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| EUM | 1671 | -0.48 | 14.49 | -1.96 | 5.79 | -3.07 | 7.59 |
| JMA | 1671 | -1.67 | 14.43 | -1.38 | 4.32 | 0.55 | 5.99 |
| KMA | 1683 | -0.30 | 12.73 | -1.30 | 6.07 | -8.09 | 8.30 |
| NOA | 1683 | -2.05 | 12.85 | -0.93 | 4.63 | -0.36 | 6.78 |
| NWC | 1639 | -1.34 | 15.35 | -1.91 | 5.46 | -2.48 | 7.66 |

Table 8-25: Experiment 2 All AMVs CQIs ( >=50)

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 942 | 1.46 | 14.44 | -2.69 | 11.65 | -9.48 | 15.22 |
| EUM | 185 | -0.31 | 15.59 | -3.40 | 6.92 | 11.33 | 8.96 |
| JMA | 313 | -2.94 | 18.33 | -1.36 | 4.64 | -0.83 | 6.34 |
| KMA | 896 | -0.97 | 14.74 | -1.35 | 7.28 | -3.89 | 9.68 |
| NOA | 691 | -1.58 | 13.93 | -0.90 | 5.44 | 1.89 | 7.62 |
| NWC | 2204 | -1.02 | 16.30 | -2.17 | 6.03 | 0.40 | 7.85 |

Table 8-26: Experiment 2 All AMVs CQIs (>=80)

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 619 | 1.16 | 13.44 | -0.40 | 7.36 | -14.65 | 9.80 |
| EUM | 33 | -5.10 | 15.79 | -4.64 | 8.99 | 14.28 | 10.65 |
| JMA | 270 | -3.43 | 18.67 | -1.40 | 4.64 | -0.83 | 6.42 |
| KMA | 693 | -0.88 | 14.17 | -1.01 | 7.12 | -5.04 | 9.44 |
| NOA | 599 | -1.69 | 13.98 | -0.88 | 5.25 | 0.39 | 7.48 |
| NWC | 2063 | -1.19 | 16.19 | -2.11 | 5.99 | 0.79 | 7.85 |

Table 8-27: Experiment 2 Collocated AMVs CQI

| Site | N | P bias | P RMS | SpdBias | SpdRMS | DirBias | VecRMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BRZ | 894 | 1.14 | 13.36 | -1.78 | 12.43 | -29.93 | 16.27 |
| EUM | 856 | -1.14 | 12.66 | -2.46 | 6.04 | 3.96 | 7.74 |
| JMA | 817 | 0.02 | 13.21 | -1.20 | 3.99 | -4.24 | 5.34 |
| KMA | 894 | 2.43 | 11.90 | -1.79 | 5.59 | -3.80 | 7.59 |
| NOA | 876 | -1.48 | 11.60 | -0.99 | 4.66 | 3.48 | 6.36 |
| NWC | 874 | -1.47 | 12.55 | -1.60 | 5.10 | 2.22 | 6.98 |

## Model Grid comparison

The Python scripts that we used to find the Best Fit also output comparison of all AMVs to the background grid. This comparison was based on 12-hour forecast using QINF >= 80.  The tables are for all AMVs (Table 8-28), all AMVs by height range (high, medium, and low) (Table 8-29 to Table 8-31), and collocated AMVs for all levels (Table 8-32).

The results are very similar to what was found with the rawinsonde comparisons: NWC and JMA have the lowest error, while BRZ and EUM have the highest errors. Considering collocated data, NWC, JMA, KMA show again the best results while EUM shows again the worst results.

Table 8-28: Experiment 2 all AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 48916 | 10675 | 6.86 | 9.66 | 6.12 | 9.27 |
| EUM | 21083 | 6761 | 4.67 | 6.41 | 3.68 | 5.56 |
| JMA | 26492 | 10009 | 2.51 | 3.11 | 2.24 | 2.90 |
| KMA | 44762 | 13505 | 5.54 | 7.54 | 4.66 | 6.96 |
| NOA | 40472 | 14180 | 4.00 | 5.17 | 3.12 | 4.38 |
| NWC | 104722 | 35948 | 4.24 | 5.21 | 3.30 | 4.37 |

Table 8-29: Experiment 2 high-level AMVs (<400 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 22743 | 7863 | 7.64 | 10.50 | 6.34 | 9.81 |
| EUM | 12247 | 5329 | 5.40 | 7.12 | 4.03 | 5.99 |
| JMA | 13535 | 7228 | 2.94 | 3.62 | 2.58 | 3.34 |
| KMA | 22962 | 10206 | 5.21 | 6.82 | 3.98 | 5.88 |
| NOA | 25446 | 11635 | 4.94 | 6.09 | 3.75 | 5.14 |
| NWC | 65575 | 29434 | 4.64 | 5.63 | 3.45 | 4.58 |

Table 8-30: Experiment 2 mid-level AMVs (400-700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 10089 | 734 | 10.46 | 13.08 | 10.22 | 13.00 |
| EUM | 2477 | 529 | 5.83 | 7.62 | 5.10 | 7.19 |
| JMA | 4488 | 1109 | 2.66 | 3.13 | 2.45 | 2.97 |
| KMA | 12643 | 2090 | 7.18 | 9.50 | 6.57 | 9.19 |
| NOA | 992 | 387 | 4.77 | 5.74 | 3.79 | 4.96 |
| NWC | 10562 | 2436 | 5.14 | 6.25 | 4.42 | 5.76 |

Table 8-31: Experiment 2 low-level AMVs (>700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 15952 | 2073 | 3.40 | 4.36 | 3.15 | 4.18 |
| EUM | 6349 | 900 | 2.82 | 3.97 | 2.45 | 3.63 |
| JMA | 8448 | 1660 | 1.73 | 2.05 | 1.59 | 1.93 |
| KMA | 9108 | 1191 | 4.08 | 5.99 | 3.69 | 5.77 |
| NOA | 14027 | 2155 | 2.22 | 2.72 | 1.93 | 2.43 |
| NWC | 28516 | 4055 | 2.96 | 3.48 | 2.53 | 3.08 |

Table 8-32: Experiment 2 collocated AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 70114 | 17242 | 5.90 | 8.24 | 5.22 | 7.87 |
| EUM | 70089 | 23823 | 3.94 | 4.90 | 3.11 | 4.17 |
| JMA | 70200 | 25587 | 2.50 | 2.99 | 2.24 | 2.78 |
| KMA | 70265 | 22163 | 4.42 | 5.63 | 3.66 | 5.06 |
| NOA | 70213 | 24435 | 3.50 | 4.28 | 2.81 | 3.66 |
| NWC | 70620 | 24503 | 3.97 | 4.72 | 3.12 | 3.99 |

QIWF

Table 8-33: Experiment 2 all AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | 0 | NaN | NaN | NaN | NaN |
| EUM | 16693 | 5623 | 3.73 | 4.96 | 2.88 | 4.13 |
| JMA | 27610 | 10013 | 2.67 | 3.32 | 2.40 | 3.11 |
| KMA | 23866 | 7122 | 3.86 | 4.78 | 3.20 | 4.25 |
| NOA | 38291 | 13621 | 3.78 | 4.89 | 2.94 | 4.11 |
| NWC | 94605 | 32960 | 4.00 | 4.96 | 3.12 | 4.17 |

Table 8-34: Experiment 2 high-level AMVs (<400 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | 0 | NaN | NaN | NaN | NaN |
| EUM | 9492 | 4464 | 4.41 | 5.66 | 3.21 | 4.57 |
| JMA | 14149 | 7251 | 3.12 | 3.84 | 2.76 | 3.57 |
| KMA | 10650 | 4963 | 3.91 | 4.63 | 2.99 | 3.85 |
| NOA | 23706 | 11170 | 4.69 | 5.78 | 3.53 | 4.83 |
| NWC | 58769 | 27199 | 4.38 | 5.37 | 3.25 | 4.36 |

Table 8-35: Experiment 2 mid-level AMVs (400-700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | 0 | NaN | NaN | NaN | NaN |
| EUM | 1937 | 477 | 4.79 | 6.02 | 4.00 | 5.45 |
| JMA | 4755 | 1085 | 2.83 | 3.32 | 2.63 | 3.18 |
| KMA | 6527 | 1288 | 4.74 | 5.89 | 4.16 | 5.48 |
| NOA | 957 | 381 | 4.63 | 5.58 | 3.68 | 4.86 |
| NWC | 9652 | 2333 | 4.94 | 6.05 | 4.22 | 5.54 |

Table 8-36: Experiment 2 low-level AMVs (>700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | 0 | NaN | NaN | NaN | NaN |
| EUM | 5255 | 679 | 2.12 | 2.60 | 1.87 | 2.33 |
| JMA | 8684 | 1665 | 1.84 | 2.20 | 1.70 | 2.09 |
| KMA | 6669 | 865 | 2.94 | 3.69 | 2.58 | 3.37 |
| NOA | 13623 | 2068 | 2.14 | 2.61 | 1.86 | 2.32 |
| NWC | 26116 | 3406 | 2.79 | 3.28 | 2.41 | 2.93 |

Table 8-37: Experiment 2 collocated AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | n/a | n/a | n/a | n/a | n/a | n/a |
| EUM | 55639 | 19127 | 3.64 | 4.73 | 2.81 | 3.89 |
| JMA | 55792 | 19684 | 2.51 | 3.00 | 2.25 | 2.77 |
| KMA | 55790 | 18402 | 3.52 | 4.23 | 2.85 | 3.67 |
| NOA | 55835 | 19340 | 3.44 | 4.35 | 2.70 | 3.61 |
| NWC | 56260 | 19115 | 3.83 | 4.75 | 3.00 | 3.96 |

CQI

Table 8-38: Experiment 2 all AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 46229 | 10571 | 6.79 | 9.65 | 6.02 | 9.24 |
| EUM | 2283 | 582 | 4.56 | 6.20 | 3.81 | 5.63 |
| JMA | 22823 | 8130 | 2.63 | 3.28 | 2.37 | 3.08 |
| KMA | 44019 | 13676 | 5.31 | 7.16 | 4.42 | 6.55 |
| NOA | 40472 | 14180 | 4.00 | 5.17 | 3.12 | 4.38 |
| NWC | 142279 | 48491 | 4.35 | 5.33 | 3.37 | 4.46 |

Table 8-39: Experiment 2 high-level AMVs (<400 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 22055 | 7900 | 7.52 | 10.43 | 6.18 | 9.71 |
| EUM | 1169 | 428 | 4.89 | 6.49 | 3.82 | 5.66 |
| JMA | 11421 | 5858 | 3.11 | 3.85 | 2.75 | 3.60 |
| KMA | 23511 | 10538 | 5.05 | 6.56 | 3.84 | 5.60 |
| NOA | 25446 | 11635 | 4.94 | 6.09 | 3.75 | 5.14 |
| NWC | 88348 | 39160 | 4.78 | 5.78 | 3.52 | 4.68 |

Table 8-40: Experiment 2 mid-level AMVs (400-700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 8979 | 645 | 10.79 | 13.42 | 10.56 | 13.34 |
| EUM | 469 | 97 | 6.36 | 8.03 | 5.65 | 7.66 |
| JMA | 3856 | 863 | 2.76 | 3.23 | 2.56 | 3.08 |
| KMA | 11534 | 1960 | 6.98 | 9.23 | 6.34 | 8.90 |
| NOA | 992 | 387 | 4.77 | 5.74 | 3.79 | 4.96 |
| NWC | 15867 | 3423 | 5.03 | 6.10 | 4.38 | 5.66 |

Table 8-41: Experiment 2 low-level AMVs (>700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 15070 | 2022 | 3.28 | 4.17 | 3.02 | 3.98 |
| EUM | 641 | 55 | 2.64 | 3.53 | 2.44 | 3.35 |
| JMA | 7533 | 1399 | 1.83 | 2.18 | 1.69 | 2.06 |
| KMA | 8924 | 1162 | 3.84 | 5.35 | 3.45 | 5.10 |
| NOA | 14027 | 2155 | 2.22 | 2.72 | 1.93 | 2.43 |
| NWC | 37978 | 5883 | 3.07 | 3.59 | 2.61 | 3.17 |

Table 8-42: Experiment 2 collocated AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 14329 | 2944 | 5.34 | 7.00 | 4.66 | 6.51 |
| EUM | 14145 | 3618 | 3.74 | 4.74 | 3.12 | 4.20 |
| JMA | 14339 | 4226 | 2.49 | 2.93 | 2.27 | 2.75 |
| KMA | 14394 | 3795 | 4.20 | 5.24 | 3.54 | 4.73 |
| NOA | 14369 | 3885 | 3.52 | 4.34 | 2.93 | 3.80 |
| NWC | 14585 | 4075 | 3.83 | 4.65 | 3.14 | 4.02 |

## Best fit height

The Best Fit height analysis was completed for each wind producer according to the method described by Salonen et al. (2012). This technique finds the background model best-fit pressure associated with the AMV, which is where the vector difference between the observed AMV and model background is at a minimum.

It does this by first locating the model pressure levels within the troposphere up to 150 hPa, which is a tunable parameter, above and below the AMV.  Using a parabolic fit, it then analyzes the located model pressure levels and detects the single model pressure level which has the minimum vector difference, which must be both less than or equal to 4 ms-1 and at least 2 ms-1 smaller than the vector differences +/- 100 hPa from the best-fit pressure level.  Therefore, this method is dependent on the model vertical resolution.  It is possible using a requirement of at least 2 ms-1 smaller than the vector differences +/- 100 hPa from the best-fit pressure level is too demanding of a requirement.

Similar to previous studies, the number of best-fit matches was generally less than 30% of the AMVs (Salonen et al. 2012).

Figure 8-10 to Figure 8-26 show the:

- Distribution of Best Fit minus AMV pressure differences, color-coded by low, medium, and high clouds (upper-left),

- Spatial distribution with same color coding (upper-right),

- Relationship between AMV pressure and latitude, color-coded to indicate if the Best fit moved the AMV higher (red) or lower (blue) (lower-left),

- Relationship between AMV pressure and speed, color-coded to indicate if the Best fit moved the AMV higher (red) or lower (blue) (lower-right).

Figure 8-10: BRZ (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-11: BRZ (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-12: EUM (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-13: EUM (QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-14: EUM (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-15: JMA (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-16: JMA (QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-17: JMA (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-18: KMA (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-19: Figure 8-20: KMA (QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-21: Figure 8-22: KMA (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-23: NOA (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-24: NOA (QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-25: NOA (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-26: NWC (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-27: NWC (QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 8-28: NWC (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

The depiction of the distribution of Best Fit statistics is located in Figure 8-29 through Figure 8-43.  Depending on the site, 16% to 24% of the AMVs are adjusted to a Best Fit pressure (lower-left in each figure). This pie chart shows the fraction of AMVs with found best-fit, not constrained (another minimum was found close by), or did not meet minimum vector difference limits (which was 4 ms-1). Results of the Best Fit algorithm show an approximate Gaussian distribution of the pressure difference centered near zero (upper-right panel in figures) extending ±200 hPa. There does not appear to be a relationship between latitude or longitude and adjusted pressure (upper-left and middle-left panels in figures).

The lower-right panels depict the geographic distribution where red dots indicate the AMV should be higher and blue where it should be lower in the atmosphere. JMA (Figure 8-34) shows a spatial pattern in the Best Fit shift of the AMVs: high clouds are moved higher (cyclone northwest of Africa) and low clouds are moved lower (marine stratus) in the mid-South Atlantic Ocean.

In the upper-right corner of the figures is the distribution of the pressure difference (AMV Best Fit pressure minus the Original Pressure). For sites BRZ, CMA, EUM, NOA, and KMA the pressure difference is centered near zero. However, JMA has two peaks (-50 and  +100) with a minimum near zero and NWC is slightly skewed to the right of zero. For experiment 2, since the AMV heights were assigned using only the IR brightness temperature, these offset and skewed distributions may be the result of the specific implementation of the IR brightness temperature height assignment, shown to be erroneous.

Figure 8-29: BRZ (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-30: BRZ (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-31: EUM (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-32: EUM (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-33: EUM (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-34: JMA (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-35: JMA (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-36: JMA (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-37: KMA (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-38: KMA (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-39: KMA (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-40: NOA (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-41: NOA (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-42: NOA (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-43: NWC (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-44: NWC (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 8-45: NWC (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Additional figures were generated to show the distribution of the differences between the AMVs and the background grid, before and after applying the Best Fit algorithm. The lower half of Figure 17-35 to Figure 17-49 shows the change in the speed difference distribution before (left) and after (right) Best Fit adjustment. As expected, the speed bias is usually reduced, as the standard deviation.

Similarly, the lower half of Figure 17-52 to Figure 17-66 shows the change in the vector difference distribution before (left) and after (right) Best Fit adjustment and as expected, the vector difference and standard deviation is usually reduced.  BRZ and CMA have the largest deviation before the best fit, with a vector difference of over 7 ms-1, as compared to the background grid. NWC has the smallest deviation before the best fit, with a vector difference of less than 4 ms-1.

## Statistical comparison

To quantify the observed differences in the previous plots, a paired t-test is once again used with all combinations of producers and parameters.  Further information about the test is located in the section for Experiment 2 Statistical comparison. Error! Reference source not found. to Error! Reference source not found. summarize the results in terms of statistical significance for each of the five variables; green signifies not statistically different while red is statistically different.

AMVs were first quality controlled, retaining only those with a QINF >= 50. For collocation the distance threshold is 35 km, resulting in 7050 AMVs.

NWC SAF and EUMETSAT are the only combinations very close in terms of both speed and direction, despite having a cloud height bias of 130 hPa.

Table 8-43: Experiment 1 QINF 50 speed t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-44: Experiment 1 QINF 50 direction t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-45: Experiment 1 QINF 50 pressure t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA did not report speed.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-46: Experiment 1 QINF 50 QI t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA was not included.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-47: Experiment 1 QINF 80 speed t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-48: Experiment 1 QINF 80 direction t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-49: Experiment 1 QINF 80 pressure t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA did not report speed.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-50: Experiment 1 QINF 80 QI t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA was not included.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-51: Experiment 1 QIWF 50 speed t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 8-52: Experiment 1 QIWF 50 direction t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 8-53: Experiment 1 QIWF 50 pressure t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA did not report speed.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 8-54: Experiment 1 QIWF 50 QI t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA was not included.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 8-55: Experiment 1 QIWF 80 speed t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 8-56: Experiment 1 QIWF 80 direction t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 8-57: Experiment 1 QIWF 80 pressure t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA did not report speed.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 8-58: Experiment 1 QIWF 80 QI t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA was not included.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 8-59: Experiment 1 CQI 50 speed t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-60: Experiment 1 CQI 50 direction t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-61: Experiment 1 CQI 50 pressure t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA did not report speed.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-62: Experiment 1 CQI 50 QI t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA was not included.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-63: Experiment 1 CQI 80 speed t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-64: Experiment 1 CQI 80 direction t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-65: Experiment 1 CQI 80 pressure t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA did not report speed.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

Table 8-66: Experiment 1 CQI 80 QI t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different. JMA was not included.

|  | EUM | KMA | NOA | NWC | JMA | BRZ |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |

# Experiment 3

AMV producers extract IR10.8 channel AMVs considering a prescribed AMV algorithm configuration, but only using the MSG/SEVIRI IR10.8 images and the ECMWF model data for the height assignment. This experiment will be used to test the Tracking and Quality control steps in all AMV algorithms, considering similar targets.

The prescribed AMV configuration defines target scene size, search scene size, etc. to be the same for all AMV producers. Something similar was done in the 2nd study and it was found that:

“Winds data sets retrieved using common target and search box sizes revealed that each producer’s algorithm is finely tuned to a specific imagery temporal and spatial resolution, as well as target and search box sizes (Genkova et al. 2010).”

For each one of the AMV producer’s datasets, a distribution of AMV speed, direction, vector height, and QI will be generated. Differences of these quantities between AMV producers will be made. Also, differences in AMV coverage and number of vectors will be presented as bulk statistics and geographic plots.

Experiment 3 Highlights

Graphs of bulk distributions are similar to Experiment 2, since the height assignment options are restricted to IR BT.

Collocated vectors only number 370, due to the lower overall numbers of AMVs when using prescribed target and search box sizes. Considering this configuration, there are more similarities between centers: speed and direction differences are not statistically different, although, pressure and QI values are significantly different.

Collocated AMVs from the different algorithms will be used to measure the differences. Also, a comparison of the AMVs with the NWP model winds and height assignment investigations using NWP model best-fit pressure will be used for verification.

## Approach

By using a prescribed AMV configuration, we were able to better quantify differences in AMV density between the producers. In cases with similar densities among producers, the QI can also be compared quantitatively.

This was similar to Study 2 by Genkova et al. (2010):

- Only SEVIRI 10.8 µm channel was used for the height assignment.

- Producers used their algorithm .

- Prescribed target and search box sizes were used.

- ECMWF grids were used as NWP data.

- A 55 km distance to define collocation was used, while Genkova et al. (2010) used 0.5°.

Genkova’s scripts were used to do same comparison and analysis as done before, but now including NWC SAF.

In addition:

- The best fit analysis was used to further analyze differences in cloud heights.

- The paired t-test was computed to determine if the observed differences in collocated AMVs were statistically significant.

This was also an opportunity to compare AMVs from Experiments 2 and 3 from the same producer and quantify differences in AMVs between the standard and the prescribed configuration.

EUMETSAT’s ‘standard’ configuration was the same as their ‘prescribed’ one.

## Parameter distributions

The bulk statistics are presented in both tables and histograms for QINF >= 50 and QIWF >= 50 (because not all centers reported both QINF and QIWF).

Table 9-1 and Table 9-2 list basic Experiment 3 statistics for the AMVs for each winds producer, without and with forecast quality respectively. Most of the values in Table 9-1 are approximately the same, with the biggest difference in the number of AMVs: Ranging from 3000 (NWC) to 12000 (EUM).

Table 9-1: Experiment 3 statistical summary of AMV datasets for QI without forecast >= 50.

|  | EUM | BRZ | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- | --- |
| Total AMVs | 30941 | 74298 | 67920 | 50003 | 65256 | 152459 |
| QI>=50 | 26632 | 67084 | 48573 | 48627 | 61075 | 143451 |
| SPD_min | 2.50 | 3.00 | 2.50 | 3.00 | 2.50 | 2.50 |
| SPD_max | 87.16 | 68.07 | 87.47 | 87.49 | 95.67 | 84.80 |
| SPD_mean | 13.23 | 11.72 | 13.94 | 13.58 | 13.30 | 13.08 |
| P_min | 57.49 | 100.00 | 100.00 | 101.84 | 105.18 | 101.00 |
| P_max | 1003.18 | 1000.00 | 1016.10 | 996.76 | 1000.00 | 971.00 |
| P_mean | 450.80 | 497.34 | 503.66 | 454.09 | 474.55 | 416.48 |
| Low_winds | 30.04 | 31.60 | 32.91 | 36.64 | 26.98 | 26.93 |
| Mid_winds | 13.47 | 21.23 | 18.77 | 2.63 | 24.64 | 11.54 |
| High_winds | 56.48 | 47.18 | 48.32 | 60.73 | 48.39 | 61.53 |
| Low_SPD_min | 2.50 | 3.00 | 2.50 | 3.00 | 2.50 | 2.51 |
| Low_SPD_max | 51.62 | 47.50 | 42.92 | 35.11 | 79.65 | 33.41 |
| Low_SPD_mean | 8.70 | 9.12 | 9.43 | 9.66 | 9.62 | 8.51 |
| Low_P_min | 700.07 | 700.00 | 700.02 | 700.01 | 700.02 | 700.00 |
| Low_P_max | 1003.18 | 1000.00 | 1016.10 | 996.76 | 1000.00 | 971.00 |
| Low_P_mean | 821.30 | 883.75 | 891.13 | 842.91 | 804.47 | 798.10 |
| Mid_SPD_min | 2.50 | 3.05 | 2.50 | 3.03 | 2.51 | 2.51 |
| Mid_SPD_max | 67.05 | 68.07 | 67.28 | 81.72 | 81.31 | 65.77 |
| Mid_SPD_mean | 14.60 | 13.40 | 13.76 | 24.42 | 13.81 | 15.46 |
| Mid_P_min | 400.05 | 400.02 | 400.01 | 400.06 | 400.00 | 400.50 |
| Mid_P_max | 699.94 | 699.96 | 699.97 | 449.97 | 699.99 | 699.50 |
| Mid_P_mean | 534.11 | 521.01 | 529.42 | 424.46 | 547.30 | 537.73 |
| High_SPD_min | 2.50 | 3.04 | 2.50 | 3.00 | 2.50 | 2.50 |
| High_SPD_max | 87.16 | 60.61 | 87.47 | 87.49 | 95.67 | 84.80 |
| High_SPD_mean | 15.31 | 12.71 | 17.09 | 15.48 | 15.09 | 14.63 |
| High_P_min | 57.49 | 100.00 | 100.00 | 101.84 | 105.18 | 101.00 |
| High_P_max | 399.66 | 400.00 | 399.93 | 399.94 | 399.99 | 400.00 |
| High_P_mean | 233.86 | 227.88 | 229.73 | 220.80 | 253.58 | 226.75 |

Table 9-2: Experiment 3 statistical summary of AMV datasets for QI with forecast >= 50.

|  | EUM | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- |
| Total AMVs | 30941 | 67920 | 50003 | 65256 | 152459 |
| QI>=50 | 26194 | 47720 | 48403 | 59543 | 142835 |
| SPD_min | 2.50 | 2.50 | 3.00 | 2.50 | 2.50 |
| SPD_max | 87.16 | 87.47 | 87.49 | 95.67 | 84.80 |
| SPD_mean | 13.46 | 13.74 | 13.61 | 13.39 | 13.16 |
| P_min | 57.67 | 100.00 | 104.52 | 105.18 | 101.00 |
| P_max | 1003.18 | 1016.10 | 996.76 | 1000.00 | 971.00 |
| P_mean | 452.07 | 504.04 | 455.17 | 475.05 | 416.77 |
| Low_winds | 30.22 | 32.92 | 36.81 | 27.20 | 26.96 |
| Mid_winds | 13.49 | 18.86 | 2.61 | 24.43 | 11.54 |
| High_winds | 56.29 | 48.22 | 60.58 | 48.37 | 61.50 |
| Low_SPD_min | 2.50 | 2.50 | 3.00 | 2.50 | 2.51 |
| Low_SPD_max | 51.62 | 85.24 | 35.11 | 79.65 | 33.41 |
| Low_SPD_mean | 8.73 | 9.33 | 9.66 | 9.62 | 8.54 |
| Low_P_min | 700.07 | 700.02 | 700.01 | 700.02 | 700.00 |
| Low_P_max | 1003.18 | 1016.10 | 996.76 | 1000.00 | 971.00 |
| Low_P_mean | 820.79 | 891.66 | 842.93 | 804.60 | 798.06 |
| Mid_SPD_min | 2.50 | 2.50 | 3.03 | 2.51 | 2.51 |
| Mid_SPD_max | 69.79 | 67.28 | 81.72 | 81.31 | 65.77 |
| Mid_SPD_mean | 14.97 | 13.47 | 24.67 | 13.92 | 15.54 |
| Mid_P_min | 400.05 | 400.01 | 400.06 | 400.00 | 400.50 |
| Mid_P_max | 699.94 | 699.97 | 449.97 | 699.99 | 699.50 |
| Mid_P_mean | 534.32 | 529.88 | 424.48 | 547.39 | 537.57 |
| High_SPD_min | 2.50 | 2.50 | 3.00 | 2.50 | 2.50 |
| High_SPD_max | 87.16 | 87.47 | 87.49 | 95.67 | 84.80 |
| High_SPD_mean | 15.63 | 16.86 | 15.54 | 15.24 | 14.73 |
| High_P_min | 57.67 | 100.00 | 104.52 | 105.18 | 101.00 |
| High_P_max | 399.66 | 399.93 | 399.94 | 399.99 | 400.00 |
| High_P_mean | 234.38 | 229.32 | 220.85 | 253.16 | 226.97 |

Table 9-3: Experiment 3 statistical summary of AMV datasets for CQI >= 50

|  | EUM | BRZ | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- | --- |
| Total AMVs | 30941 | 74298 | 67920 | 50003 | 65256 | 152459 |
| QI>=50 | 11920 | 66647 | 28238 | 48627 | 59223 | 152200 |
| SPD_min | 2.50 | 2.51 | 2.50 | 3.00 | 2.50 | 2.50 |
| SPD_max | 109.35 | 68.07 | 84.72 | 87.49 | 95.67 | 84.80 |
| SPD_mean | 12.59 | 11.91 | 13.95 | 13.58 | 13.44 | 12.97 |
| P_min | 64.20 | 100.00 | 125.00 | 101.84 | 105.18 | 101.00 |
| P_max | 992.21 | 1000.00 | 999.94 | 996.76 | 1000.00 | 971.00 |
| P_mean | 468.02 | 497.13 | 497.80 | 454.09 | 472.65 | 420.91 |
| Low_winds | 32.79 | 31.57 | 33.33 | 36.64 | 27.06 | 27.17 |
| Mid_winds | 13.73 | 21.14 | 15.91 | 2.63 | 23.95 | 12.44 |
| High_winds | 53.47 | 47.29 | 50.75 | 60.73 | 48.99 | 60.39 |
| Low_SPD_min | 2.50 | 2.55 | 2.50 | 3.00 | 2.50 | 2.50 |
| Low_SPD_max | 109.35 | 47.50 | 39.59 | 35.11 | 79.65 | 33.65 |
| Low_SPD_mean | 9.15 | 9.13 | 9.33 | 9.66 | 9.65 | 8.49 |
| Low_P_min | 700.18 | 700.00 | 700.15 | 700.01 | 700.02 | 700.00 |
| Low_P_max | 992.21 | 1000.00 | 999.94 | 996.76 | 1000.00 | 971.00 |
| Low_P_mean | 823.05 | 884.03 | 895.39 | 842.91 | 804.34 | 798.49 |
| Mid_SPD_min | 2.50 | 2.51 | 2.50 | 3.03 | 2.51 | 2.51 |
| Mid_SPD_max | 94.62 | 68.07 | 67.28 | 81.72 | 81.31 | 65.77 |
| Mid_SPD_mean | 15.67 | 13.67 | 14.63 | 24.42 | 14.02 | 15.34 |
| Mid_P_min | 400.05 | 400.02 | 400.01 | 400.06 | 400.00 | 400.50 |
| Mid_P_max | 699.94 | 699.96 | 699.69 | 449.97 | 699.99 | 699.50 |
| Mid_P_mean | 537.05 | 521.16 | 529.67 | 424.46 | 547.39 | 538.57 |
| High_SPD_min | 2.50 | 2.52 | 2.50 | 3.00 | 2.50 | 2.50 |
| High_SPD_max | 84.99 | 60.61 | 84.72 | 87.49 | 95.67 | 84.80 |
| High_SPD_mean | 13.91 | 12.97 | 16.77 | 15.48 | 15.25 | 14.49 |
| High_P_min | 64.20 | 100.00 | 125.00 | 101.84 | 105.18 | 101.00 |
| High_P_max | 399.66 | 400.00 | 399.88 | 399.94 | 399.99 | 400.00 |
| High_P_mean | 232.56 | 228.08 | 226.71 | 220.80 | 252.90 | 226.83 |

Table 9-4: Experiment 3 statistical summary of AMV datasets for QI without forecast >= 80

|  | EUM | BRZ | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- | --- |
| Total AMVs | 30941 | 74298 | 67920 | 50003 | 65256 | 152459 |
| QI>=80 | 21594 | 48858 | 27475 | 43765 | 51609 | 110878 |
| SPD_min | 2.50 | 3.00 | 2.51 | 3.00 | 2.50 | 2.62 |
| SPD_max | 87.16 | 64.82 | 85.21 | 87.48 | 95.67 | 84.80 |
| SPD_mean | 14.04 | 11.23 | 15.83 | 14.06 | 13.85 | 13.92 |
| P_min | 62.76 | 100.00 | 100.00 | 104.52 | 105.23 | 101.00 |
| P_max | 1003.18 | 1000.00 | 1014.57 | 996.76 | 1000.00 | 971.00 |
| P_mean | 453.52 | 509.73 | 490.09 | 455.00 | 476.03 | 413.69 |
| Low_winds | 31.23 | 33.94 | 32.12 | 36.81 | 27.86 | 26.85 |
| Mid_winds | 12.17 | 20.63 | 16.01 | 2.44 | 23.68 | 10.67 |
| High_winds | 56.60 | 45.43 | 51.87 | 60.76 | 48.45 | 62.48 |
| Low_SPD_min | 2.51 | 3.00 | 2.51 | 3.00 | 2.50 | 2.64 |
| Low_SPD_max | 49.25 | 32.92 | 39.59 | 35.11 | 69.51 | 33.41 |
| Low_SPD_mean | 8.86 | 8.94 | 9.96 | 9.87 | 9.87 | 8.73 |
| Low_P_min | 700.07 | 700.00 | 700.09 | 700.01 | 700.02 | 700.00 |
| Low_P_max | 1003.18 | 1000.00 | 1014.57 | 996.76 | 1000.00 | 971.00 |
| Low_P_mean | 819.14 | 885.86 | 896.62 | 843.91 | 805.89 | 797.03 |
| Mid_SPD_min | 2.54 | 3.05 | 2.52 | 3.22 | 2.51 | 2.68 |
| Mid_SPD_max | 67.05 | 64.82 | 67.28 | 81.72 | 73.25 | 65.69 |
| Mid_SPD_mean | 15.90 | 11.23 | 16.09 | 26.53 | 14.59 | 16.59 |
| Mid_P_min | 400.05 | 100.00 | 400.01 | 400.06 | 400.02 | 400.50 |
| Mid_P_max | 699.94 | 1000.00 | 699.71 | 449.97 | 699.99 | 699.50 |
| Mid_P_mean | 536.24 | 509.73 | 529.28 | 424.02 | 547.32 | 540.27 |
| High_SPD_min | 2.50 | 33.94 | 2.51 | 3.00 | 2.50 | 2.62 |
| High_SPD_max | 87.16 | 20.63 | 85.21 | 87.48 | 95.67 | 84.80 |
| High_SPD_mean | 16.49 | 45.43 | 18.52 | 16.09 | 15.77 | 15.69 |
| High_P_min | 62.76 | 3.00 | 100.00 | 104.52 | 105.23 | 101.00 |
| High_P_max | 399.66 | 32.92 | 399.88 | 399.94 | 399.99 | 400.00 |
| High_P_mean | 234.03 | 8.94 | 226.24 | 220.64 | 251.52 | 227.37 |

Table 9-5: Experiment 3 statistical summary of AMV datasets for QI with forecast >= 80.

|  | EUM | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- |
| Total AMVs | 30941 | 67920 | 50003 | 65256 | 152459 |
| QI>=80 | 17351 | 47720 | 41617 | 27228 | 100097 |
| SPD_min | 2.50 | 2.50 | 3.00 | 2.50 | 2.68 |
| SPD_max | 87.16 | 85.21 | 87.48 | 79.13 | 84.80 |
| SPD_mean | 15.25 | 14.92 | 14.37 | 14.81 | 14.55 |
| P_min | 64.20 | 100.00 | 104.52 | 107.60 | 101.00 |
| P_max | 992.21 | 1014.57 | 996.76 | 1000.00 | 971.00 |
| P_mean | 466.20 | 487.67 | 460.42 | 528.78 | 415.72 |
| Low_winds | 33.36 | 31.65 | 37.59 | 37.23 | 27.03 |
| Mid_winds | 11.90 | 16.35 | 2.46 | 22.62 | 10.64 |
| High_winds | 54.74 | 52.00 | 59.96 | 40.14 | 62.33 |
| Low_SPD_min | 2.51 | 2.50 | 3.00 | 2.50 | 2.73 |
| Low_SPD_max | 34.40 | 39.59 | 35.11 | 41.62 | 33.41 |
| Low_SPD_mean | 9.12 | 9.78 | 9.96 | 10.03 | 8.97 |
| Low_P_min | 700.07 | 700.09 | 700.01 | 700.06 | 700.00 |
| Low_P_max | 992.21 | 1014.57 | 996.76 | 1000.00 | 971.00 |
| Low_P_mean | 818.10 | 896.44 | 844.41 | 815.08 | 797.61 |
| Mid_SPD_min | 2.56 | 2.52 | 3.22 | 2.51 | 2.68 |
| Mid_SPD_max | 67.05 | 67.28 | 81.72 | 73.25 | 65.69 |
| Mid_SPD_mean | 17.53 | 15.32 | 27.27 | 16.18 | 17.34 |
| Mid_P_min | 400.05 | 400.01 | 400.06 | 400.02 | 400.50 |
| Mid_P_max | 699.94 | 699.71 | 449.97 | 699.98 | 699.50 |
| Mid_P_mean | 535.30 | 528.49 | 423.77 | 542.57 | 539.50 |
| High_SPD_min | 2.50 | 2.50 | 3.01 | 2.50 | 2.70 |
| High_SPD_max | 87.16 | 85.21 | 87.48 | 79.13 | 84.80 |
| High_SPD_mean | 18.49 | 17.93 | 16.60 | 18.47 | 16.49 |
| High_P_min | 64.20 | 100.00 | 104.52 | 107.60 | 101.00 |
| High_P_max | 399.61 | 399.88 | 399.94 | 399.99 | 400.00 |
| High_P_mean | 236.71 | 225.99 | 221.22 | 255.46 | 228.99 |

Table 9-6: Experiment 3 statistical summary of AMV datasets for CQI >= 80

|  | EUM | BRZ | JMA | NOA | KMA | NWC |
| --- | --- | --- | --- | --- | --- | --- |
| Total AMVs | 30941 | 74298 | 217744 | 50003 | 65256 | 100869 |
| QI>=80 | 1879 | 46932 | 58446 | 43765 | 49539 | 95479 |
| SPD_min | 2.51 | 2.51 | 2.50 | 3.00 | 2.50 | 2.50 |
| SPD_max | 109.35 | 64.82 | 84.79 | 87.48 | 79.13 | 84.82 |
| SPD_mean | 14.86 | 11.71 | 13.48 | 14.06 | 13.98 | 12.81 |
| P_min | 105.19 | 100.00 | 125.00 | 104.52 | 105.23 | 102.00 |
| P_max | 975.00 | 1000.00 | 1002.10 | 996.76 | 1000.00 | 974.00 |
| P_mean | 481.49 | 504.57 | 489.74 | 455.00 | 467.63 | 483.67 |
| Low_winds | 31.24 | 33.46 | 32.34 | 36.81 | 27.52 | 35.58 |
| Mid_winds | 19.74 | 20.01 | 15.71 | 2.44 | 21.87 | 13.25 |
| High_winds | 49.02 | 46.53 | 51.95 | 60.76 | 50.61 | 51.16 |
| Low_SPD_min | 2.51 | 2.55 | 2.50 | 3.00 | 2.50 | 2.51 |
| Low_SPD_max | 109.35 | 39.01 | 29.02 | 35.11 | 69.51 | 34.52 |
| Low_SPD_mean | 10.27 | 9.05 | 9.12 | 9.87 | 9.79 | 9.30 |
| Low_P_min | 700.44 | 700.00 | 700.04 | 700.01 | 700.02 | 700.00 |
| Low_P_max | 975.00 | 1000.00 | 1002.10 | 996.76 | 1000.00 | 974.00 |
| Low_P_mean | 821.28 | 889.38 | 882.58 | 843.91 | 804.50 | 819.76 |
| Mid_SPD_min | 2.68 | 2.51 | 2.51 | 3.22 | 2.51 | 2.50 |
| Mid_SPD_max | 62.03 | 64.82 | 74.78 | 81.72 | 73.25 | 82.38 |
| Mid_SPD_mean | 16.92 | 13.64 | 14.69 | 26.53 | 14.98 | 15.62 |
| Mid_P_min | 400.51 | 400.02 | 400.03 | 400.06 | 400.02 | 400.50 |
| Mid_P_max | 699.94 | 699.96 | 700.00 | 449.97 | 699.99 | 699.50 |
| Mid_P_mean | 528.96 | 516.99 | 541.65 | 424.02 | 547.98 | 530.79 |
| High_SPD_min | 3.13 | 2.60 | 2.51 | 3.00 | 2.50 | 2.50 |
| High_SPD_max | 67.29 | 60.61 | 84.79 | 87.48 | 79.13 | 84.82 |
| High_SPD_mean | 16.96 | 12.79 | 15.83 | 16.09 | 15.82 | 14.53 |
| High_P_min | 105.19 | 100.00 | 125.00 | 104.52 | 105.23 | 102.00 |
| High_P_max | 399.23 | 400.00 | 399.96 | 399.94 | 399.99 | 400.00 |
| High_P_mean | 245.80 | 222.52 | 229.48 | 220.64 | 249.77 | 237.71 |

Figure 14-35 to Figure 14-45 in Appendix A: Parameter Distribution Histograms show bulk histograms and unique characteristics of the AMVs for each centre, which are very similar to Experiment 2 since the height assignment method was unchanged.  The AMV wind direction and pressure distributions for each center (lower-left and lower-right respectively) have peaks in the same locations as in Experiment 2:

- BRZ (Figure 14-35): The wind direction (lower-left) is not a smooth distribution. Conversely, it has two very sharp peaks. The AMV pressure distribution (lower-right) has peaks at 300 and 770 hPa. Upper-level winds at 300 hPa are reasonable, but the 770 hPa peak is most likely too high for low-level clouds.

EUM (Figure 14-36 and

- Figure 14-41): The AMV pressure distribution has a peak at 500 and 800 hPa. The upper level winds are too low, while the low-level winds are too high.

- JMA (Figure 14-37 and Figure 14-42): The AMV pressure distribution has a peak at 500 and 850 hPa. The upper level winds are too low, while the low-level winds are placed well. There is also a noticeable gap in mid-level winds.

- KMA (Figure 14-38 and Figure 14-43): The AMV pressure distribution has a peak at 450 and 800 hPa. The upper level winds are too low, while the low-level winds are too high.

- NOA (Figure 14-39 and Figure 14-44): The AMV pressure distribution has a peak at 400 and 780 hPa. The upper level winds are too low, while the low-level winds are too high.

NWC (

- Figure 14-40 and Figure 14-45): The AMV pressure distribution has a peak at 400 and 780 hPa. The upper level winds are too low, while the low-level winds are too high.

As in Experiment 2, since only the IR brightness temperature was used in this experiment, the wide variation in cloud heights can be attributed to different techniques and thresholds in determining a representative TB.

## Collocation plots

AMVs were first quality controlled, retaining only those with QINF >= 50. For collocation the distance threshold was 55 km, resulting in only 370 AMVs.

Figure 9-1: Plots of collocated AMVs of speed (top), direction (2nd), pressure (3rd), and QI (bottom) are color-coded based on legend in upper-right. The x-axis is AMV number.

Figure 9-2: The maximum pressure difference between any two collocated AMVs.

Figure 9-3: Scatter plot of AMV pressure for each center vs. EUM pressure.

A similar comparison was done for AMVs with QI with forecast, retaining only those with a QIWF >= 50. For collocation the distance threshold was 55 km, resulting in 409 AMVs.

Figure 9-4: Plots of collocated AMVs of speed (top), direction (2nd), pressure (3rd), and QI (bottom) are color-coded based on legend in upper-right. The x-axis is AMV number.

Figure 9-5: The maximum pressure difference between any two collocated AMVs.

Figure 9-6: Scatter plot of AMV pressure for each center vs. EUM pressure.

CQI

Figure 9-7: Plots of collocated AMVs of speed (top), direction (2nd), pressure (3rd), and QI (bottom) are color-coded based on legend in upper-right. The x-axis is AMV number.

Figure 9-8: The maximum pressure difference between any two collocated AMVs.

Figure 9-9: Scatter plot of AMV pressure for each center vs. EUM pressure.

## Model Grid comparison

Table 9-7 through Table 9-11 show the output comparison of all AMVs to the NWP background grid, considering the Best Fit Python scripts discussed in the corresponding section of Experiment 2. They are very similar to the results in Experiment 2, with NWC and JMA having the best fit to the background while BRZ having the highest errors.

Table 9-7: Experiment 3 all AMV compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 48858 | 11502 | 6.81 | 9.51 | 6.03 | 9.10 |
| EUM | 22657 | 7918 | 4.52 | 5.97 | 3.50 | 5.12 |
| JMA | 27556 | 11765 | 2.52 | 3.11 | 2.21 | 2.85 |
| KMA | 51219 | 16379 | 5.35 | 7.36 | 4.45 | 6.79 |
| NOA | 41617 | 16754 | 3.59 | 4.52 | 2.73 | 3.73 |
| NWC | 111195 | 43496 | 4.15 | 5.02 | 3.13 | 4.12 |

Table 9-8: Experiment 3 high-level AMVs (<400 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean;         RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 22082 | 8571 | 7.43 | 10.11 | 5.99 | 9.35 |
| EUM | 12382 | 6234 | 5.23 | 6.55 | 3.74 | 5.35 |
| JMA | 14246 | 8730 | 2.97 | 3.61 | 2.53 | 3.27 |
| KMA | 24676 | 12423 | 5.30 | 6.88 | 3.92 | 5.84 |
| NOA | 26573 | 14004 | 4.64 | 5.62 | 3.44 | 4.68 |
| NWC | 69249 | 36103 | 4.52 | 5.38 | 3.21 | 4.24 |

Table 9-9: Experiment 3 mid-level AMVs (400-700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 10059 | 712 | 11.18 | 13.58 | 10.93 | 13.45 |
| EUM | 2749 | 620 | 5.68 | 7.41 | 4.94 | 6.94 |
| JMA | 4388 | 1165 | 2.72 | 3.19 | 2.50 | 3.02 |
| KMA | 11894 | 2109 | 7.26 | 9.53 | 6.69 | 9.26 |
| NOA | 1067 | 499 | 4.87 | 6.01 | 3.67 | 4.91 |
| NWC | 11914 | 2880 | 4.91 | 5.92 | 4.19 | 5.41 |

Table 9-10: Experiment 3 low-level AMVs (>700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean;            RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 16602 | 2212 | 3.31 | 4.24 | 3.06 | 4.06 |
| EUM | 7520 | 1062 | 2.93 | 4.07 | 2.59 | 3.79 |
| JMA | 8899 | 1857 | 1.69 | 2.00 | 1.54 | 1.86 |
| KMA | 14599 | 1835 | 3.86 | 5.98 | 3.53 | 5.80 |
| NOA | 16108 | 2776 | 2.29 | 2.83 | 1.93 | 2.44 |
| NWC | 29964 | 4483 | 3.00 | 3.58 | 2.54 | 3.13 |

Table 9-11: Experiment 3 collocated AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast .

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 75057 | 20876 | 6.18 | 8.63 | 5.38 | 8.21 |
| EUM | 74964 | 29469 | 4.05 | 4.99 | 3.06 | 4.13 |
| JMA | 75059 | 31590 | 2.52 | 3.04 | 2.21 | 2.79 |
| KMA | n/a | n/a | n/a | n/a | n/a | n/a |
| NOA | 75243 | 30908 | 3.44 | 4.16 | 2.62 | 3.42 |
| NWC | 75631 | 30021 | 3.95 | 4.71 | 2.99 | 3.88 |

QIWF

Table 9-12: Experiment 3 all AMV compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | 0 | NaN | NaN | NaN | NaN |
| EUM | 17608 | 6618 | 3.65 | 4.66 | 2.75 | 3.80 |
| JMA | 28860 | 11844 | 2.68 | 3.32 | 2.37 | 3.07 |
| KMA | 26891 | 8570 | 3.62 | 4.61 | 2.96 | 4.09 |
| NOA | n/a | n/a | n/a | n/a | n/a | n/a |
| NWC | 100177 | 40050 | 3.91 | 4.75 | 2.95 | 3.90 |

Table 9-13: Experiment 3 high-level AMVs (<400 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean;         RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | 0 | NaN | NaN | NaN | NaN |
| EUM | 9500 | 5290 | 4.30 | 5.25 | 2.96 | 4.05 |
| JMA | 14949 | 8792 | 3.15 | 3.83 | 2.70 | 3.49 |
| KMA | 10704 | 5934 | 3.83 | 4.54 | 2.78 | 3.66 |
| NOA | 24935 | 13551 | 4.41 | 5.30 | 3.23 | 4.34 |
| NWC | 62308 | 33552 | 4.26 | 5.09 | 3.02 | 4.01 |

Table 9-14: Experiment 3 mid-level AMVs (400-700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | 0 | NaN | NaN | NaN | NaN |
| EUM | 2044 | 541 | 4.71 | 5.99 | 3.94 | 5.41 |
| JMA | 4695 | 1160 | 2.92 | 3.43 | 2.70 | 3.28 |
| KMA | 5960 | 1395 | 4.82 | 6.03 | 4.20 | 5.63 |
| NOA | 1022 | 486 | 4.70 | 5.80 | 3.57 | 4.77 |
| NWC | 10639 | 2688 | 4.65 | 5.66 | 3.95 | 5.15 |

Table 9-15: Experiment 3 low-level AMVs (>700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean;            RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 0 | 0 | NaN | NaN | NaN | NaN |
| EUM | 6060 | 786 | 2.27 | 2.77 | 2.02 | 2.53 |
| JMA | 9193 | 1879 | 1.81 | 2.18 | 1.65 | 2.04 |
| KMA | 10207 | 1236 | 2.70 | 3.61 | 2.43 | 3.40 |
| NOA | 15642 | 2702 | 2.22 | 2.74 | 1.87 | 2.35 |
| NWC | 27165 | 3780 | 2.81 | 3.35 | 2.40 | 2.96 |

Table 9-16: Experiment 3 collocated AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | n/a | n/a | n/a | n/a | n/a | n/a |
| EUM | 59068 | 23039 | 3.55 | 4.37 | 2.67 | 3.56 |
| JMA | 59175 | 24209 | 2.48 | 2.97 | 2.17 | 2.72 |
| KMA | 59216 | 22550 | 3.39 | 4.20 | 2.70 | 3.63 |
| NOA | 59328 | 23863 | 3.18 | 3.88 | 2.43 | 3.17 |
| NWC | 59757 | 23292 | 3.65 | 4.42 | 2.78 | 3.63 |

CQI

Table 9-17: Experiment 3 all AMV compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 47026 | 11470 | 6.83 | 9.60 | 6.02 | 9.18 |
| EUM | 1898 | 587 | 4.55 | 6.53 | 3.65 | 5.91 |
| JMA | 23941 | 9826 | 2.63 | 3.24 | 2.33 | 3.01 |
| KMA | 50834 | 16601 | 5.12 | 6.99 | 4.21 | 6.38 |
| NOA | 43765 | 17293 | 3.78 | 4.79 | 2.89 | 4.01 |
| NWC | 150451 | 56044 | 4.27 | 5.16 | 3.24 | 4.26 |

Table 9-18: Experiment 3 high-level AMVs (<400 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean;         RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 21764 | 8681 | 7.37 | 10.11 | 5.90 | 9.33 |
| EUM | 923 | 428 | 5.15 | 6.54 | 3.77 | 5.53 |
| JMA | 12271 | 7283 | 3.11 | 3.78 | 2.69 | 3.48 |
| KMA | 25169 | 12769 | 5.18 | 6.66 | 3.80 | 5.61 |
| NOA | 26573 | 14004 | 4.64 | 5.62 | 3.44 | 4.68 |
| NWC | 89571 | 45368 | 4.68 | 5.58 | 3.32 | 4.41 |

Table 9-19: Experiment 3 mid-level AMVs (400-700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 9376 | 665 | 11.58 | 13.94 | 11.33 | 13.86 |
| EUM | 360 | 86 | 5.32 | 6.83 | 4.59 | 6.33 |
| JMA | 3719 | 925 | 2.82 | 3.27 | 2.60 | 3.11 |
| KMA | 10885 | 1968 | 7.09 | 9.30 | 6.51 | 9.02 |
| NOA | 1067 | 499 | 4.87 | 6.01 | 3.67 | 4.91 |
| NWC | 18607 | 4191 | 4.94 | 5.92 | 4.26 | 5.44 |

Table 9-20: Experiment 3 low-level AMVs (>700 hPa) compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean;            RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 15774 | 2117 | 3.21 | 4.11 | 2.97 | 3.94 |
| EUM | 614 | 73 | 3.20 | 6.32 | 2.90 | 6.17 |
| JMA | 7930 | 1606 | 1.79 | 2.14 | 1.63 | 2.01 |
| KMA | 14737 | 1853 | 3.54 | 5.32 | 3.20 | 5.12 |
| NOA | 16108 | 2776 | 2.29 | 2.83 | 1.93 | 2.44 |
| NWC | 42185 | 6449 | 3.08 | 3.67 | 2.62 | 3.23 |

Table 9-21: Experiment 3 collocated AMVs compared to background grid: a 12-hour forecast. N = total number of AMVs; BFN = Best Fit number of AMVs; V_O = VD OMB mean; RAF = RMSE after Best Fit; VAF = Vector difference after Best Fit; RMSE = root mean square error. QI = 80-100, without forecast.

| EXP | N | BFN | VO | RMSE | VAF | RAF |
| --- | --- | --- | --- | --- | --- | --- |
| BRZ | 13577 | 3884 | 5.72 | 7.73 | 4.73 | 7.12 |
| EUM | 13363 | 4516 | 3.81 | 4.67 | 3.02 | 4.01 |
| JMA | 13581 | 5331 | 2.46 | 2.91 | 2.14 | 2.64 |
| KMA | 13647 | 4872 | 4.20 | 5.34 | 3.30 | 4.64 |
| NOA | 13666 | 5418 | 3.41 | 4.08 | 2.61 | 3.39 |
| NWC | 13887 | 5332 | 3.72 | 4.45 | 2.84 | 3.68 |

## Best fit height

The Best Fit height analysis was completed using the same method as in Experiment 2, described in section Experiment 2 Best fit height.

Figure 9-10: BRZ (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-11: BRZ (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-12: EUM (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-13: EUM (QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-14: EUM (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-15: JMA (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-16: JMA (QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-17: JMA (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-18: KMA (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-19: KMA (QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-20: KMA (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-21: NOA( QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-22: NOA( QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-23: NOA( CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-24: NWC (QINF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-25: NWC (QIWF): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

Figure 9-26: NWC (CQI): Distribution of Best Fit – AMV pressure by height (upper-left); Best Fit AMV location (color coded by height); AMV pressure vs. Latitude (color coded by Best Fit height adjustment); AMV pressure vs. Speed (color coded by Best Fit height adjustment).

The depiction of the distribution of Best Fit statistics is in Figure 9-27 through Figure 9-41.  Depending on the site, 12% to 28% (lower-left in each figure) of the AMVs were adjusted to a Best Fit pressure. There does not appear to be a relationship between latitude or longitude and adjusted pressure (upper-left and middle-left in figures).

In the upper-right corner of the figures is the distribution of the pressure difference (AMV Best Fit pressure minus the Original Pressure), all of which are extremely similar to Experiment 2. For sites BRZ, CMA, EUM, NOA, and KMA the pressure difference was centered near zero, JMA had two peaks (-50 and +100) with a minimum near zero, and NWC was slightly skewed to the right of zero. As similar to Experiment 2, since the AMV heights were assigned using only the IR brightness temperature, these offset and skewed distributions may be the result of the specific implementation of the IR brightness temperature height assignment.

Figure 9-27: BRZ (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-28: BRZ (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-29: EUM (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-30: EUM (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-31: EUM (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-32: JMA (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-33 : JMA (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-34: JMA (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-35: KMA (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-36: KMA (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-37: KMA (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-38: NOA (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-39: NOA (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-40: NOA (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-41: NWC (QINF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-42: NWC (QIWF): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Figure 9-43: NWC (CQI): Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Latitude (upper-left); Distribution of all AMVs (green) and Best Fit AMVs (gold) vs. Longitude (middle-left); Percent of AMVs used in Best Fit (Found) and those that didn’t meet criteria (Not Constrained, No sufficient minimum) (lower-left).  Frequency of pressure difference, Best Fit – original AMV pressure (upper-right); spatial distribution of Best Fit modified AMVs (color code by Best Fit height adjustment (lower-right).

Ancillary figures were generated, located in Appendix D: Best Fit Speed and Vector Difference, to show how the statistics improved when the AMV height was adjusted to the Best Fit level.

The lower half of Figure 17-69 to Figure 17-86 shows the change in the speed difference distribution before (left) and after (right) Best Fit adjustment. As expected, the speed bias and standard deviation was usually reduced. Likewise, the lower half of Figure 17-86 to Figure 17-100 shows the change in the vector difference distribution before (left) and after (right) Best Fit adjustment with the same result: the vector difference and standard deviation were usually reduced. BRZ and CMA had the largest deviation before the best fit, with a vector difference of about 8 ms-1, as compared to the background grid, and NWC had the smallest deviation before the best fit, with a vector difference of about 4 ms-1, s compared to the background grid. These results are very similar to Experiment 2.

## Statistical comparison

Paired t-tests were used between all combinations of producers and parameters to determine if the differences were statistically significant in collocated AMVs. Using a QI no forecast >= 50 and distance threshold of 55 km, the number of collocations was very low at 370.

As compared to Experiment 2, there were many more instances of agreement between the winds producers in terms of wind speed (Table 9-22) and wind direction (Table 9-23). In fact, for direction, differences between all centers are not statistically significant. Table 9-22 to Table 9-29 summarize the results in terms of statistical significance for each of the five variables; green signifies not statistically different while red is statistically different.

As with Experiment 2, there was very little similarity in AMV pressure and QI values, at least as measured by the paired t-test.

The output from the paired t-test for this experiment can be found in Appendix C: t-test Results Experiment 3.

Table 9-22: Experiment 3 speed t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | BRZ | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |

Table 9-23: Experiment 3 direction t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | BRZ | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |

Table 9-24: Experiment 3 pressure t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | BRZ | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |

Table 9-25: Experiment 3 QI without forecast t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | BRZ | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |  |
| KMA |  |  |  |  |  |  |
| BRZ |  |  |  |  |  |  |
| NOA |  |  |  |  |  |  |
| NWC |  |  |  |  |  |  |
| JMA |  |  |  |  |  |  |

The paired t-tests were also used between all combinations of producers and parameters using a QIWF >= 50 and a distance threshold 55 km; the number of collocations was also low at 245.

There was no statistical difference in direction (

Table 9-27) and many speed differences were also not statistically different (Table 9-26).

Although many differences between AMV pressure and QI values were still statistically different, the similarities between AMV pressures were noticeably improved when compared to the t-tests completed on the data with QI without forecast.

The output from the paired t-test for this experiment can be found in Appendix C: t-test Results Experiment 3.

Table 9-26: Experiment 3 speed t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 9-27: Experiment 3 direction t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 9-28: Experiment 3 pressure t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

Table 9-29: Experiment 3 QI with forecast t-test for each paired combination of winds producers. Green indicates the parameter is not statistically different at the 95% level; red is statistically different.

|  | EUM | KMA | NOA | NWC | JMA |
| --- | --- | --- | --- | --- | --- |
| EUM |  |  |  |  |  |
| KMA |  |  |  |  |  |
| NOA |  |  |  |  |  |
| NWC |  |  |  |  |  |
| JMA |  |  |  |  |  |

# Comparison to CALIPSO

Observations from NASA’s Cloud-Aerosol Lidar and Infrared Pathfinder Satellite Observation (CALIPSO) provide and independent measurement of cloud top heights, which can be used to validate and compare to the satellite-derived AMVs.

Since CALIPSO is a line-of-site measurement, there are few co-located observations with AMVs. Therefore, this evaluation is qualitative as illustrsted in the following figures.

The results are as expected and very consistent among the different centres, with the AMV heigths generally near the cloud base for the high-level, thin cirrus clouds and near the cloud top for low- and mid-level clouds. This is in general agreement with the Best Fit analysis (see Section ???).

Figure 10-1: BRZ: CALIPSO comparison with collocated AMVs at 0540 UTC on 21 July 2016. Asterisks represent the AMV heights.

Figure 10-2: EUM: CALIPSO comparison with collocated AMVs at 0540 UTC on 21 July 2016. Asterisks represent the AMV heights.

Figure 10-3: JMA: CALIPSO comparison with collocated AMVs at 0540 UTC on 21 July 2016. Asterisks represent the AMV heights.

Figure 10-4: KMA: CALIPSO comparison with collocated AMVs at 0540 UTC on 21 July 2016. Asterisks represent the AMV heights.

Figure 10-5: NOA: CALIPSO comparison with collocated AMVs at 0540 UTC on 21 July 2016. Asterisks represent the AMV heights.

Figure 10-6: NWC: CALIPSO comparison with collocated AMVs at 0540 UTC on 21 July 2016. Asterisks represent the AMV heights.

# Summary and Conclusions

Four experiments have been conducted by each of the AMV producers:

- AMV producers extract IR 10.8 µ AMVs considering a triplet of images with a known displacement. This experiment tests the tracking step in all AMV algorithms.

- AMV producers extract IR 10.8 µ AMVs considering their standard AMV algorithm configuration, but only using the MSG/SEVIRI IR 10.8 µ images and the ECMWF model data for the Height assignment. This experiment tests the target selection, tracking and quality control steps in all AMV algorithms.

- This experiment is the same as Experiment 2, except a prescribed AMV algorithm configuration is used. The experiment is used to test the tracking and quality control steps in all AMV algorithms, considering similar targets.

- This experiment is the same as Experiment 3, except the AMV producer can use the height assignment method of their choosing. This experiment is used to test the height assignment and quality control steps in all AMV algorithms, considering similar targets.

The following sections detail the findings from the experiments, in terms of each AMV producer, independently. This includes the strengths and weaknesses as determined from the results of the experiments.

## EUMETSAT

In this study, the EUMETSAT AMVs have been used as the comparison to all the other centers to detect differences in the datasets, as the EUMETSAT algorithm has been specifically developed and tuned for Meteosat data.

The strengths of the algorithm have been especially noted in Experiments 1 and 4.  In Experiment 1, all vector displacements are correct. In Experiment 4, the statistical comparison of the EUM AMVs to rawinsondes and the background forecast wind field is performing best together with NWC SAF AMVs. The AMV coverage at high and low levels is very dense in Experiment 4, as evidenced in the spatial plots.

However, the use of only the IR BT for cloud height (Experiments 2 and 3) results in AMVs being placed several hundred hPa different than when other techniques could be used (Experiment 4). This conclusion is confirmed with the high error in the rawinsonde comparison statistics, and is likely due to a brightness temperature that is too warm.

## Japan Meteorological Agency

JMA algorithm performs very well in Experiments 2 and 3. Results from Experiment 4 show instead a relative degradation of validation statistics when measuring performance both on comparisons to rawinsondes and the background wind field.

More in detail, AMV coverage for JMA is very dense in low levels in Experiment 4, and the comparison to rawinsondes and the background grid is good. However, the upper level winds are few and did not compare well to rawinsondes nor to background grid, while results in Experiment 2 did compare well, likely due to the cold brightness temperature assigned to the cloud features.

JMA developers should verify the reasons because of which Experiment 4 statistics degrade respect to Experiment 2 and 3 statistics.

## NOAA

The strength of the NOAA algorithm is in its cloud height determination, as evidenced in Experiment 4: A substantial number of heights are adjusted (as compared to IR-only BT) resulting in an improvement in a statistical comparison to rawinsondes and the background forecast wind field.

The spatial distribution in Experiment 4 is very good at high levels, but very few winds are tracked in low levels. Unfortunately, a high vertical resolution background grid cloud not be used to better detect temperature inversions and the height of low-level clouds, which may have impacted the low-cloud density.

## Korea Meteorological Administration

Results from Experiments 2 and 3 show that KMA AMVs perform rather well against many centers.

Results from Experiment 4 show at the same time that the KMA algorithm is in the middle (statistically) when measuring performance, based on comparisons to rawinsondes and the background wind field.

Coverage in Experiment 4 is very good for the high-level winds. However, low-level winds are assigned higher in the atmosphere, compared to EUM.

## NWC SAF

Among all the centers in this study, the NWC SAF/HRW algorithm has the best statistics as compared to rawinsondes and the background forecast wind field. This is the case for both Experiment 3 (IR BT only cloud height) and Experiment 4 (any cloud height technique). Moreover, NWC AMVs with IR-only cloud height performs better than several other centers using other cloud height techniques.

The spatial distribution of the low-level winds using the prescribed configuration is dense and with very good comparison to rawinsondes and the background grid. The high-levels winds are less dense that EUM; however the comparison to rawinsondes and the background grid is usually the best among the centers.

There are two areas noted for suggestions to improve the NWC SAF algorithm. First, to investigate increasing the coverage of the high-level winds since it is less dense than several other centers (e.g., EUM, NOA, KMA) when the prescribed configuration is used. Second, the IR BT technique used by NWC SAF will result in the coldest temperature compared to the other centers. This is good for the high-level winds (and the statistics confirm that); however, this may not be the best method for warmer clouds as the low-level clouds are placed too high in Experiment 2.

## Brazilian Meteorological Center

The performance of the BRZ AMV algorithm could not be evaluated because the results of Experiment 1 indicate an error in determining wind speed up to 10 ms-1 depending on the distance from the satellite subpoint.

However, the Best Fit analysis indicates that there are good AMVs in this dataset as the Best Fit height adjustment and corresponding improvement in statistics (compared to the background) are very similar to other centers.

In addition, the coverage in Experiment 4 is very good for the high-level winds. However, low-level winds are assigned higher in the atmosphere, compared to EUM.

# References

Genkova, I., R. Borde, J. Schmetz, J. Daniels, C. Velden, K. Holmlund, 2008: Global atmospheric motion vectors intercomparison study, 9th Int. Winds Workshop, Annapolis, MD USA, April 2008.

Genkova, I., R. Borde, J. Schmetz, C. Velden, K. Holmlund, N. Bormann, P. Bauer, 2010: Global atmospheric motion vector intercomparison study, 10th Int. Winds Workshop, Tokyo, Japan, February 2010.

Salonen, K., J. Cotton, N. Bormann, M. Forsythe, 2012: Characterising AMV height assignment error by comparing best-fit pressure statistics from the Met Office and ECMWF System, 11th International Winds Workshop, Auckland, New Zealand, February 2012.

Xu J. and Q. Zhang, 1996: Calculation of Cloud motion wind with GMS-5 images in China. Proceedings 3rd international winds workshop, EUMETSAT Pub.18, pp.45-52.

# Acknowledgements

We would like to thank EUMETSAT, the Satellite Application Facility on support to Nowcasting (NWC SAF) and Agencia Estatal de Meteorología (AEMET) for the funding and support of this project. Also, we appreciate the comments and suggestions from the International Winds Working Group (IWWG) co-chairs, Jaime Daniels and Mary Forsythe.

# Appendix A: Parameter Distribution Histograms

## Experiment 1: QINF Parameter Distribution Histograms

Figure 14-1: Experiment 1 parameter distributions for BRZ: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-2: Experiment 1 parameter distributions for EUM: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-3: Experiment 1 parameter distributions for JMA: latitude/longitude spatial      distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-4: Experiment 1 parameter distributions for KMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-5: Experiment 1 parameter distributions for NOA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-6: Experiment 1 parameter distributions for NWC: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

## Experiment 1: QIWF Parameter Distribution Histograms

Figure 14-7: Experiment 1 parameter distributions for EUM: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-8: Experiment 1 parameter distributions for JMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-9: Experiment 1 parameter distributions for KMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-10: Experiment 1 parameter distributions for NOA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-11: Experiment 1 parameter distributions for NWC: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

## Experiment 1: CQI Parameter Distribution Histograms

Figure 14-12: Experiment 1 parameter distributions for BRZ: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-13: Experiment 1 parameter distributions for EUM: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-14: Experiment 1 parameter distributions for JMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-15: Experiment 1 parameter distributions for KMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-16: Experiment 1 parameter distributions for NOA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-17: Experiment 1 parameter distributions for NWC	: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

## Experiment 2: QINF Parameter Distribution Histograms

Figure 14-18: Experiment 2 parameter distributions for BZR: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-19: Experiment 2 parameter distributions for EUM: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-20: Experiment 2 parameter distributions for JMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-21: Experiment 2 parameter distributions for KMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-22: Experiment 2 parameter distributions for NOA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-23: Experiment 2 parameter distributions for NWC: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

## Experiment 2: QIWF Parameter Distribution Histograms

Figure 14-24: Experiment 2 parameter distributions for EUM: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-25: Experiment 2 parameter distributions for JMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-26: Experiment 2 parameter distributions for KMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-27: Experiment 2 parameter distributions for NOA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-28: Experiment 2 parameter distributions for NWC: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

## Experiment 2: CQI Parameter Distribution Histogram

Figure 14-29: Experiment 2 parameter distributions for BRZ: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-30: Experiment 2 parameter distributions for EUM: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-31: Experiment 2 parameter distributions for JMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-32: Experiment 2 parameter distributions for KMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-33: Experiment 2 parameter distributions for NOA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-34: Experiment 2 parameter distributions for NWC: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

## Experiment 3: QINF Parameter Distribution Histogram

Figure 14-35: Experiment 3 parameter distributions for BRZ: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-36: Experiment 3 parameter distributions for EUM: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-37: Experiment 3 parameter distributions for JMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-38: Experiment 3 parameter distributions for KMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-39: Experiment 3 parameter distributions for NOA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-40: Experiment 3 parameter distributions for NWC: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

## Experiment 3: QIWF Parameter Distribution Histograms

Figure 14-41: Experiment 3 parameter distributions for EUM: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-42: Experiment 3 parameter distributions for JMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-43: Experiment 3 parameter distributions for KMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-44: Experiment 3 parameter distributions for NOA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-45: Experiment 3 parameter distributions for NWC: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

## Experiment 3: CQI Parameter Distribution Histograms

Figure 14-46: Experiment 3 parameter distributions for BRZ: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-47: Experiment 3 parameter distributions for EUM: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-48: Experiment 3 parameter distributions for JMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-49: Experiment 3 parameter distributions for KMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-50: Experiment 3 parameter distributions for NOA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-51: Experiment 3 parameter distributions for NWC: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

## Experiment 4: QINF Parameter Distribution Histograms

Figure 14-52: Experiment 4 parameter distributions for BRZ: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-53: Experiment 4 parameter distributions for EUM: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-54: Experiment 4 parameter distributions for JMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-55: Experiment 4 parameter distributions for KMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-56: Experiment 4 parameter distributions for NOA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-57: Experiment 4 parameter distributions for NWC: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

## Experiment 4: QIWF Parameter Distribution Histograms

Figure 14-58: Experiment 4 parameter distributions for EUM: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-59: Experiment 4 parameter distributions for JMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-60: Experiment 4 parameter distributions for KMA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-61: Experiment 4 parameter distributions for NOA: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

Figure 14-62: Experiment 4 parameter distributions for NWC: latitude/longitude spatial distribution and histograms of QI, wind speed, direction, and AMV pressure.

# Appendix B: Matlab t-test Documentation

ttest  One-sample and paired-sample t-test.

H = ttest(X) performs a t-test of the hypothesis that the data in the

vector X come from a distribution with mean zero, and returns the

result of the test in H.  H=0 indicates that the null hypothesis

(“mean is zero”) cannot be rejected at the 5% significance leve

H=1 indicates that the null hypothesis can be rejected at the 5%

level. The data are assumed to come from a normal distribution

with unknown variance.

X can also be a matrix or an N-D array. For matrices, ttest performs

separate t-tests along each column of X, and returns a vector of

results. For N-D arrays, ttest works along the first non-singleton

dimension of X.

ttest treats NaNs as missing values, and ignores them.

H = ttest(X,M) performs a t-test of the hypothesis that the data in

X come from a distribution with mean M. M must be a scalar.

H = ttest(X,Y) performs a paired t-test of the hypothesis that two

matched samples, in the vectors X and Y, come from distributions with

equal means. The difference X-Y is assumed to come from a normal

distribution with unknown variance. X and Y must have the same

length. X and Y can also be matrices or N-D arrays of the same size.

[H,P] = ttest(...) returns the p-value, i.e., the probability of

observing the given result, or one more extreme, by chance if the

null hypothesis is true.  Small values of P cast doubt on the

validity of the null hypothesis.

[H,P,CI] = ttest(...) returns a 100*(1-ALPHA)% confidence interval

For the true mean of X, or of X-Y for a paired test.

[H,P,CI,STATS] = ttest(...) returns a structure with the following

fields:

‘tstat’ – the value of the test statistic

‘df’    -- the degrees of freedom of the test

‘sd’    -- the estimated population standard deviation.  For a

paired test, this is the std. dev. Of X-Y.

# Appendix C: t-test Results

Text output from Matlab script Stats_QIF_One.m. Text files used as input:

EUM:	EUMETSATDatasetOne.csv

JMA:	JapanDatasetOneNew.csv

NOA:	NOAADatasetOne.csv

KMA:	KoreaDatasetOne.csv

NWC:	NWCSAFDatasetOne_O_C_S_E.csv

BRZ:	BrazilDatasetOne.csv

All QI values were used, distance threshold was set to 35km, there were 10876 collocated matches, and a Statistical confidence 95% was used.

See Appendix B: Matlab t-test Documentation for description of h, p, ci, Mean and Appendix G: Matlab Scripts the script for script Stats_QIF_One.m

## Experiment 1 t-test Results

QINF 50 collocated AMVs: ???? File: statsout_exp1.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.07 0.14, Mean: 0.10

Direction: h = 0, p = 0.80, ci = -0.60 0.77, Mean: 0.09

Pressure:  h = 1, p = 0.00, ci = -14.22 -8.78, Mean: -11.50

QI:        h = 1, p = 0.00, ci = -2.72 -2.22, Mean: -2.47

Japan "VS" NOAA

Speed:     h = 0, p = 0.25, ci = -0.06 0.02, Mean: -0.02

Direction: h = 0, p = 0.15, ci = -0.18 1.19, Mean: 0.51

Pressure:  h = 1, p = 0.00, ci = 11.38 16.19, Mean: 13.79

QI:        h = 1, p = 0.00, ci = -6.95 -6.41, Mean: -6.68

Japan "VS" EUMETSAT

Speed:     h = 0, p = 0.12, ci = -0.02 0.15, Mean: 0.07

Direction: h = 1, p = 0.04, ci = -1.98 -0.07, Mean: -1.03

Pressure:  h = 1, p = 0.00, ci = 22.19 27.06, Mean: 24.63

QI:        h = 1, p = 0.00, ci = -3.69 -3.07, Mean: -3.38

Japan "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.85 1.02, Mean: 0.94

Direction: h = 1, p = 0.00, ci = 2.35 4.11, Mean: 3.23

Pressure:  h = 1, p = 0.00, ci = -53.07 -46.38, Mean: -49.72

QI:        h = 1, p = 0.00, ci = -2.67 -1.90, Mean: -2.29

Japan "VS" NWCSAF

Speed:     h = 0, p = 0.78, ci = -0.03 0.03, Mean: 0.00

Direction: h = 1, p = 0.01, ci = 0.24 1.52, Mean: 0.88

Pressure:  h = 1, p = 0.00, ci = 25.29 29.71, Mean: 27.50

QI:        h = 1, p = 0.00, ci = 5.47 6.06, Mean: 5.77

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.17 -0.08, Mean: -0.13

Direction: h = 0, p = 0.27, ci = -0.33 1.17, Mean: 0.42

Pressure:  h = 1, p = 0.00, ci = 22.95 27.63, Mean: 25.29

QI:        h = 1, p = 0.00, ci = -4.42 -4.00, Mean: -4.21

Korea "VS" EUMETSAT

Speed:     h = 0, p = 0.36, ci = -0.12 0.04, Mean: -0.04

Direction: h = 1, p = 0.02, ci = -2.08 -0.15, Mean: -1.11

Pressure:  h = 1, p = 0.00, ci = 33.91 38.36, Mean: 36.13

QI:        h = 1, p = 0.00, ci = -1.17 -0.65, Mean: -0.91

Korea "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.75 0.91, Mean: 0.83

Direction: h = 1, p = 0.00, ci = 2.29 3.99, Mean: 3.14

Pressure:  h = 1, p = 0.00, ci = -41.38 -35.06, Mean: -38.22

QI:        h = 0, p = 0.27, ci = -0.14 0.49, Mean: 0.18

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.13 -0.07, Mean: -0.10

Direction: h = 1, p = 0.03, ci = 0.09 1.49, Mean: 0.79

Pressure:  h = 1, p = 0.00, ci = 37.09 40.93, Mean: 39.01

QI:        h = 1, p = 0.00, ci = 7.97 8.50, Mean: 8.23

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.05, ci = 0.00 0.18, Mean: 0.09

Direction: h = 1, p = 0.00, ci = -2.54 -0.52, Mean: -1.53

Pressure:  h = 1, p = 0.00, ci = 9.15 12.54, Mean: 10.84

QI:        h = 1, p = 0.00, ci = 3.02 3.59, Mean: 3.30

NOAA "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.87 1.04, Mean: 0.96

Direction: h = 1, p = 0.00, ci = 1.85 3.60, Mean: 2.72

Pressure:  h = 1, p = 0.00, ci = -66.33 -60.69, Mean: -63.51

QI:        h = 1, p = 0.00, ci = 4.04 4.74, Mean: 4.39

NOAA "VS" NWCSAF

Speed:     h = 0, p = 0.20, ci = -0.01 0.07, Mean: 0.03

Direction: h = 0, p = 0.32, ci = -0.35 1.10, Mean: 0.37

Pressure:  h = 1, p = 0.00, ci = 12.46 14.98, Mean: 13.72

QI:        h = 1, p = 0.00, ci = 12.17 12.73, Mean: 12.45

EUMETSAT "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.76 0.98, Mean: 0.87

Direction: h = 1, p = 0.00, ci = 3.15 5.36, Mean: 4.25

Pressure:  h = 1, p = 0.00, ci = -77.16 -71.55, Mean: -74.35

QI:        h = 1, p = 0.00, ci = 0.71 1.47, Mean: 1.09

EUMETSAT "VS" NWCSAF

Speed:     h = 0, p = 0.15, ci = -0.15 0.02, Mean: -0.06

Direction: h = 1, p = 0.00, ci = 0.89 2.91, Mean: 1.90

Pressure:  h = 1, p = 0.00, ci = 1.61 4.13, Mean: 2.87

QI:        h = 1, p = 0.00, ci = 8.83 9.47, Mean: 9.15

Brazil "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -1.01 -0.85, Mean: -0.93

Direction: h = 1, p = 0.00, ci = -3.16 -1.54, Mean: -2.35

Pressure:  h = 1, p = 0.00, ci = 74.59 79.86, Mean: 77.23

QI:        h = 1, p = 0.00, ci = 7.67 8.44, Mean: 8.06

QINF 80 collocated AMVs: ???? File: statsout_exp1.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.01, ci = 0.01 0.09, Mean: 0.05

Direction: h = 1, p = 0.04, ci = 0.05 1.62, Mean: 0.84

Pressure:  h = 1, p = 0.00, ci = -11.85 -4.76, Mean: -8.30

QI:        h = 1, p = 0.00, ci = 2.27 2.56, Mean: 2.42

Japan "VS" NOAA

Speed:     h = 0, p = 0.58, ci = -0.03 0.05, Mean: 0.01

Direction: h = 1, p = 0.01, ci = 0.18 1.61, Mean: 0.89

Pressure:  h = 1, p = 0.00, ci = 3.50 9.52, Mean: 6.51

QI:        h = 1, p = 0.00, ci = -4.00 -3.71, Mean: -3.86

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.08 0.19, Mean: 0.14

Direction: h = 0, p = 0.48, ci = -1.46 0.68, Mean: -0.39

Pressure:  h = 1, p = 0.00, ci = 12.95 19.26, Mean: 16.10

QI:        h = 1, p = 0.00, ci = -2.45 -2.11, Mean: -2.28

Japan "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.39 0.55, Mean: 0.47

Direction: h = 1, p = 0.00, ci = 1.70 3.73, Mean: 2.72

Pressure:  h = 1, p = 0.00, ci = -63.35 -54.52, Mean: -58.93

QI:        h = 1, p = 0.00, ci = -5.08 -4.71, Mean: -4.90

Japan "VS" NWCSAF

Speed:     h = 0, p = 0.50, ci = -0.04 0.02, Mean: -0.01

Direction: h = 1, p = 0.00, ci = 0.36 1.78, Mean: 1.07

Pressure:  h = 1, p = 0.00, ci = 19.70 25.38, Mean: 22.54

QI:        h = 1, p = 0.00, ci = 2.40 2.75, Mean: 2.58

Korea "VS" NOAA

Speed:     h = 0, p = 0.08, ci = -0.08 0.00, Mean: -0.04

Direction: h = 0, p = 0.89, ci = -0.72 0.84, Mean: 0.06

Pressure:  h = 1, p = 0.00, ci = 11.76 17.86, Mean: 14.81

QI:        h = 1, p = 0.00, ci = -6.40 -6.15, Mean: -6.27

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.03 0.15, Mean: 0.09

Direction: h = 1, p = 0.03, ci = -2.31 -0.14, Mean: -1.22

Pressure:  h = 1, p = 0.00, ci = 21.44 27.37, Mean: 24.41

QI:        h = 1, p = 0.00, ci = -4.84 -4.55, Mean: -4.70

Korea "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.34 0.50, Mean: 0.42

Direction: h = 1, p = 0.00, ci = 0.88 2.88, Mean: 1.88

Pressure:  h = 1, p = 0.00, ci = -54.93 -46.33, Mean: -50.63

QI:        h = 1, p = 0.00, ci = -7.45 -7.17, Mean: -7.31

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.10 -0.03, Mean: -0.06

Direction: h = 0, p = 0.55, ci = -0.54 1.01, Mean: 0.23

Pressure:  h = 1, p = 0.00, ci = 28.35 33.34, Mean: 30.85

QI:        h = 0, p = 0.06, ci = -0.00 0.32, Mean: 0.16

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.06 0.19, Mean: 0.13

Direction: h = 1, p = 0.03, ci = -2.42 -0.15, Mean: -1.28

Pressure:  h = 1, p = 0.00, ci = 7.42 11.77, Mean: 9.59

QI:        h = 1, p = 0.00, ci = 1.42 1.73, Mean: 1.58

NOAA "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.38 0.54, Mean: 0.46

Direction: h = 1, p = 0.00, ci = 0.82 2.82, Mean: 1.82

Pressure:  h = 1, p = 0.00, ci = -69.11 -61.78, Mean: -65.44

QI:        h = 1, p = 0.00, ci = -1.20 -0.88, Mean: -1.04

NOAA "VS" NWCSAF

Speed:     h = 0, p = 0.30, ci = -0.06 0.02, Mean: -0.02

Direction: h = 0, p = 0.65, ci = -0.58 0.93, Mean: 0.18

Pressure:  h = 1, p = 0.00, ci = 14.35 17.71, Mean: 16.03

QI:        h = 1, p = 0.00, ci = 6.27 6.60, Mean: 6.43

EUMETSAT "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.24 0.42, Mean: 0.33

Direction: h = 1, p = 0.00, ci = 1.83 4.37, Mean: 3.10

Pressure:  h = 1, p = 0.00, ci = -78.79 -71.28, Mean: -75.04

QI:        h = 1, p = 0.00, ci = -2.79 -2.44, Mean: -2.62

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.21 -0.09, Mean: -0.15

Direction: h = 1, p = 0.01, ci = 0.32 2.59, Mean: 1.46

Pressure:  h = 1, p = 0.00, ci = 4.76 8.12, Mean: 6.44

QI:        h = 1, p = 0.00, ci = 4.67 5.04, Mean: 4.86

Brazil "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.56 -0.40, Mean: -0.48

Direction: h = 1, p = 0.00, ci = -2.57 -0.73, Mean: -1.65

Pressure:  h = 1, p = 0.00, ci = 77.97 84.98, Mean: 81.48

QI:        h = 1, p = 0.00, ci = 7.28 7.67, Mean: 7.47

QIWF 50 collocated AMVs: ???? File: statsout_exp1.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.05 0.12, Mean: 0.08

Direction: h = 0, p = 0.86, ci = -0.60 0.71, Mean: 0.06

Pressure:  h = 1, p = 0.00, ci = -15.16 -9.76, Mean: -12.46

QI:        h = 1, p = 0.00, ci = 5.94 6.49, Mean: 6.22

Japan "VS" NOAA

Speed:     h = 0, p = 0.92, ci = -0.04 0.03, Mean: -0.00

Direction: h = 0, p = 0.23, ci = -0.26 1.08, Mean: 0.41

Pressure:  h = 1, p = 0.00, ci = 10.29 15.12, Mean: 12.71

QI:        h = 1, p = 0.00, ci = -0.95 -0.40, Mean: -0.68

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.09 0.20, Mean: 0.14

Direction: h = 1, p = 0.02, ci = -1.99 -0.18, Mean: -1.09

Pressure:  h = 1, p = 0.00, ci = 21.41 26.29, Mean: 23.85

QI:        h = 1, p = 0.00, ci = 3.63 4.24, Mean: 3.94

Japan "VS" NWCSAF

Speed:     h = 0, p = 0.70, ci = -0.04 0.02, Mean: -0.01

Direction: h = 1, p = 0.01, ci = 0.25 1.43, Mean: 0.84

Pressure:  h = 1, p = 0.00, ci = 24.26 28.68, Mean: 26.47

QI:        h = 1, p = 0.00, ci = 9.85 10.46, Mean: 10.16

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.13 -0.05, Mean: -0.09

Direction: h = 0, p = 0.34, ci = -0.37 1.07, Mean: 0.35

Pressure:  h = 1, p = 0.00, ci = 22.84 27.49, Mean: 25.16

QI:        h = 1, p = 0.00, ci = -7.09 -6.69, Mean: -6.89

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.04, ci = 0.00 0.11, Mean: 0.06

Direction: h = 1, p = 0.01, ci = -2.04 -0.25, Mean: -1.14

Pressure:  h = 1, p = 0.00, ci = 34.11 38.50, Mean: 36.31

QI:        h = 1, p = 0.00, ci = -2.50 -2.06, Mean: -2.28

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.12 -0.06, Mean: -0.09

Direction: h = 1, p = 0.02, ci = 0.10 1.46, Mean: 0.78

Pressure:  h = 1, p = 0.00, ci = 37.04 40.82, Mean: 38.93

QI:        h = 1, p = 0.00, ci = 3.70 4.18, Mean: 3.94

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.08 0.20, Mean: 0.14

Direction: h = 1, p = 0.00, ci = -2.45 -0.54, Mean: -1.50

Pressure:  h = 1, p = 0.00, ci = 9.45 12.83, Mean: 11.14

QI:        h = 1, p = 0.00, ci = 4.37 4.86, Mean: 4.61

NOAA "VS" NWCSAF

Speed:     h = 0, p = 0.84, ci = -0.04 0.04, Mean: -0.00

Direction: h = 0, p = 0.23, ci = -0.27 1.12, Mean: 0.43

Pressure:  h = 1, p = 0.00, ci = 12.50 15.02, Mean: 13.76

QI:        h = 1, p = 0.00, ci = 10.58 11.08, Mean: 10.83

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.20 -0.09, Mean: -0.15

Direction: h = 1, p = 0.00, ci = 0.99 2.86, Mean: 1.92

Pressure:  h = 1, p = 0.00, ci = 1.37 3.87, Mean: 2.62

QI:        h = 1, p = 0.00, ci = 5.95 6.50, Mean: 6.22

QIWF 80 collocated AMVs: ???? File: statsout_exp1.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.05 0.13, Mean: 0.09

Direction: h = 0, p = 0.31, ci = -0.35 1.09, Mean: 0.37

Pressure:  h = 1, p = 0.00, ci = -10.03 -3.26, Mean: -6.65

QI:        h = 1, p = 0.00, ci = 8.40 8.74, Mean: 8.57

Japan "VS" NOAA

Speed:     h = 0, p = 0.64, ci = -0.03 0.05, Mean: 0.01

Direction: h = 1, p = 0.02, ci = 0.12 1.52, Mean: 0.82

Pressure:  h = 0, p = 0.59, ci = -2.32 4.07, Mean: 0.87

QI:        h = 1, p = 0.00, ci = 1.27 1.59, Mean: 1.43

Japan "VS" EUMETSAT

Speed:     h = 0, p = 0.80, ci = -0.05 0.07, Mean: 0.01

Direction: h = 0, p = 0.17, ci = -1.59 0.28, Mean: -0.66

Pressure:  h = 1, p = 0.00, ci = 10.61 17.13, Mean: 13.87

QI:        h = 1, p = 0.00, ci = 4.06 4.42, Mean: 4.24

Japan "VS" NWCSAF

Speed:     h = 0, p = 0.40, ci = -0.02 0.05, Mean: 0.02

Direction: h = 1, p = 0.01, ci = 0.27 1.58, Mean: 0.93

Pressure:  h = 1, p = 0.00, ci = 15.56 21.50, Mean: 18.53

QI:        h = 1, p = 0.00, ci = 6.44 6.78, Mean: 6.61

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.13 -0.03, Mean: -0.08

Direction: h = 0, p = 0.23, ci = -0.28 1.17, Mean: 0.45

Pressure:  h = 1, p = 0.00, ci = 4.60 10.44, Mean: 7.52

QI:        h = 1, p = 0.00, ci = -7.28 -6.98, Mean: -7.13

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.01, ci = -0.14 -0.02, Mean: -0.08

Direction: h = 1, p = 0.04, ci = -2.00 -0.05, Mean: -1.03

Pressure:  h = 1, p = 0.00, ci = 17.73 23.31, Mean: 20.52

QI:        h = 1, p = 0.00, ci = -4.49 -4.16, Mean: -4.33

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.12 -0.03, Mean: -0.07

Direction: h = 0, p = 0.13, ci = -0.15 1.26, Mean: 0.55

Pressure:  h = 1, p = 0.00, ci = 22.87 27.48, Mean: 25.17

QI:        h = 1, p = 0.00, ci = -2.13 -1.78, Mean: -1.95

NOAA "VS" EUMETSAT

Speed:     h = 0, p = 0.93, ci = -0.07 0.06, Mean: -0.00

Direction: h = 1, p = 0.00, ci = -2.49 -0.46, Mean: -1.48

Pressure:  h = 1, p = 0.00, ci = 10.81 15.20, Mean: 13.00

QI:        h = 1, p = 0.00, ci = 2.65 2.96, Mean: 2.81

NOAA "VS" NWCSAF

Speed:     h = 0, p = 0.83, ci = -0.04 0.05, Mean: 0.01

Direction: h = 0, p = 0.77, ci = -0.61 0.83, Mean: 0.11

Pressure:  h = 1, p = 0.00, ci = 15.94 19.37, Mean: 17.66

QI:        h = 1, p = 0.00, ci = 5.01 5.34, Mean: 5.18

EUMETSAT "VS" NWCSAF

Speed:     h = 0, p = 0.79, ci = -0.05 0.07, Mean: 0.01

Direction: h = 1, p = 0.00, ci = 0.56 2.60, Mean: 1.58

Pressure:  h = 1, p = 0.00, ci = 2.98 6.33, Mean: 4.65

QI:        h = 1, p = 0.00, ci = 2.20 2.54, Mean: 2.37

CQI 50 collocated AMVs: ???? File: statsout_exp1.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.08 0.15, Mean: 0.11

Direction: h = 0, p = 0.37, ci = -0.44 1.18, Mean: 0.37

Pressure:  h = 1, p = 0.00, ci = -10.83 -4.80, Mean: -7.81

QI:        h = 0, p = 0.46, ci = -0.15 0.34, Mean: 0.09

Japan "VS" NOAA

Speed:     h = 0, p = 0.13, ci = -0.08 0.01, Mean: -0.04

Direction: h = 1, p = 0.01, ci = 0.22 1.92, Mean: 1.07

Pressure:  h = 1, p = 0.00, ci = 21.37 26.56, Mean: 23.96

QI:        h = 1, p = 0.00, ci = 0.47 0.99, Mean: 0.73

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.12 0.26, Mean: 0.19

Direction: h = 0, p = 0.80, ci = -1.16 0.90, Mean: -0.13

Pressure:  h = 1, p = 0.00, ci = 28.85 34.07, Mean: 31.46

QI:        h = 1, p = 0.00, ci = 29.27 29.92, Mean: 29.60

Japan "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.59 0.74, Mean: 0.67

Direction: h = 1, p = 0.00, ci = 3.26 5.30, Mean: 4.28

Pressure:  h = 1, p = 0.00, ci = -39.60 -32.43, Mean: -36.01

QI:        h = 1, p = 0.00, ci = 5.90 6.60, Mean: 6.25

Japan "VS" NWCSAF

Speed:     h = 1, p = 0.01, ci = 0.02 0.09, Mean: 0.05

Direction: h = 1, p = 0.01, ci = 0.31 1.86, Mean: 1.09

Pressure:  h = 1, p = 0.00, ci = 31.63 36.47, Mean: 34.05

QI:        h = 1, p = 0.00, ci = 1.53 2.03, Mean: 1.78

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.20 -0.10, Mean: -0.15

Direction: h = 0, p = 0.10, ci = -0.14 1.54, Mean: 0.70

Pressure:  h = 1, p = 0.00, ci = 29.15 34.40, Mean: 31.78

QI:        h = 1, p = 0.00, ci = 0.38 0.91, Mean: 0.64

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.04, ci = 0.00 0.15, Mean: 0.08

Direction: h = 0, p = 0.33, ci = -1.51 0.51, Mean: -0.50

Pressure:  h = 1, p = 0.00, ci = 36.82 41.72, Mean: 39.27

QI:        h = 1, p = 0.00, ci = 29.17 29.84, Mean: 29.51

Korea "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.47 0.63, Mean: 0.55

Direction: h = 1, p = 0.00, ci = 2.95 4.86, Mean: 3.91

Pressure:  h = 1, p = 0.00, ci = -31.65 -24.75, Mean: -28.20

QI:        h = 1, p = 0.00, ci = 5.80 6.51, Mean: 6.15

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.10 -0.02, Mean: -0.06

Direction: h = 0, p = 0.07, ci = -0.05 1.49, Mean: 0.72

Pressure:  h = 1, p = 0.00, ci = 39.70 44.02, Mean: 41.86

QI:        h = 1, p = 0.00, ci = 1.43 1.94, Mean: 1.69

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.15 0.30, Mean: 0.23

Direction: h = 1, p = 0.03, ci = -2.26 -0.14, Mean: -1.20

Pressure:  h = 1, p = 0.00, ci = 5.66 9.32, Mean: 7.49

QI:        h = 1, p = 0.00, ci = 28.52 29.21, Mean: 28.87

NOAA "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.62 0.78, Mean: 0.70

Direction: h = 1, p = 0.00, ci = 2.23 4.18, Mean: 3.21

Pressure:  h = 1, p = 0.00, ci = -63.04 -56.91, Mean: -59.98

QI:        h = 1, p = 0.00, ci = 5.15 5.88, Mean: 5.51

NOAA "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.04 0.14, Mean: 0.09

Direction: h = 0, p = 0.97, ci = -0.80 0.84, Mean: 0.02

Pressure:  h = 1, p = 0.00, ci = 8.69 11.49, Mean: 10.09

QI:        h = 1, p = 0.00, ci = 0.78 1.32, Mean: 1.05

EUMETSAT "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.38 0.57, Mean: 0.47

Direction: h = 1, p = 0.00, ci = 3.22 5.60, Mean: 4.41

Pressure:  h = 1, p = 0.00, ci = -70.51 -64.43, Mean: -67.47

QI:        h = 1, p = 0.00, ci = -23.77 -22.93, Mean: -23.35

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.21 -0.06, Mean: -0.14

Direction: h = 1, p = 0.02, ci = 0.16 2.28, Mean: 1.22

Pressure:  h = 1, p = 0.00, ci = 1.21 3.98, Mean: 2.59

QI:        h = 1, p = 0.00, ci = -28.16 -27.47, Mean: -27.82

Brazil "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.69 -0.53, Mean: -0.61

Direction: h = 1, p = 0.00, ci = -4.09 -2.29, Mean: -3.19

Pressure:  h = 1, p = 0.00, ci = 67.20 72.93, Mean: 70.06

QI:        h = 1, p = 0.00, ci = -4.82 -4.11, Mean: -4.47

CQI 80 collocated AMVs: ???? File: statsout_exp1.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.07 0.18, Mean: 0.13

Direction: h = 0, p = 0.30, ci = -1.66 0.52, Mean: -0.57

Pressure:  h = 0, p = 0.06, ci = -10.08 0.22, Mean: -4.93

QI:        h = 1, p = 0.00, ci = -0.58 -0.18, Mean: -0.38

Japan "VS" NOAA

Speed:     h = 0, p = 0.85, ci = -0.05 0.06, Mean: 0.01

Direction: h = 0, p = 0.83, ci = -0.94 1.17, Mean: 0.11

Pressure:  h = 1, p = 0.00, ci = 19.61 28.65, Mean: 24.13

QI:        h = 0, p = 0.29, ci = -0.32 0.10, Mean: -0.11

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.16 0.33, Mean: 0.25

Direction: h = 0, p = 0.47, ci = -2.01 0.93, Mean: -0.54

Pressure:  h = 1, p = 0.00, ci = 23.66 32.80, Mean: 28.23

QI:        h = 1, p = 0.00, ci = 6.36 6.91, Mean: 6.64

Japan "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.07 0.26, Mean: 0.17

Direction: h = 1, p = 0.01, ci = 0.49 3.02, Mean: 1.76

Pressure:  h = 1, p = 0.00, ci = -38.92 -26.44, Mean: -32.68

QI:        h = 1, p = 0.00, ci = -0.79 -0.26, Mean: -0.52

Japan "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.03 0.14, Mean: 0.09

Direction: h = 0, p = 0.92, ci = -0.83 0.92, Mean: 0.05

Pressure:  h = 1, p = 0.00, ci = 27.53 36.10, Mean: 31.82

QI:        h = 1, p = 0.00, ci = 0.32 0.73, Mean: 0.53

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.18 -0.06, Mean: -0.12

Direction: h = 0, p = 0.20, ci = -0.37 1.74, Mean: 0.68

Pressure:  h = 1, p = 0.00, ci = 24.58 33.55, Mean: 29.06

QI:        h = 1, p = 0.01, ci = 0.05 0.48, Mean: 0.26

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.04 0.20, Mean: 0.12

Direction: h = 0, p = 0.96, ci = -1.48 1.55, Mean: 0.03

Pressure:  h = 1, p = 0.00, ci = 28.81 37.51, Mean: 33.16

QI:        h = 1, p = 0.00, ci = 6.74 7.29, Mean: 7.02

Korea "VS" Brazil

Speed:     h = 0, p = 0.41, ci = -0.06 0.13, Mean: 0.04

Direction: h = 1, p = 0.00, ci = 1.13 3.53, Mean: 2.33

Pressure:  h = 1, p = 0.00, ci = -33.87 -21.62, Mean: -27.75

QI:        h = 0, p = 0.27, ci = -0.40 0.11, Mean: -0.14

Korea "VS" NWCSAF

Speed:     h = 0, p = 0.14, ci = -0.09 0.01, Mean: -0.04

Direction: h = 0, p = 0.23, ci = -0.40 1.63, Mean: 0.62

Pressure:  h = 1, p = 0.00, ci = 32.99 40.51, Mean: 36.75

QI:        h = 1, p = 0.00, ci = 0.70 1.11, Mean: 0.90

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.15 0.33, Mean: 0.24

Direction: h = 0, p = 0.41, ci = -2.18 0.88, Mean: -0.65

Pressure:  h = 1, p = 0.00, ci = 1.35 6.84, Mean: 4.09

QI:        h = 1, p = 0.00, ci = 6.47 7.04, Mean: 6.75

NOAA "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.06 0.26, Mean: 0.16

Direction: h = 1, p = 0.02, ci = 0.26 3.03, Mean: 1.65

Pressure:  h = 1, p = 0.00, ci = -61.77 -51.85, Mean: -56.81

QI:        h = 1, p = 0.00, ci = -0.69 -0.13, Mean: -0.41

NOAA "VS" NWCSAF

Speed:     h = 1, p = 0.01, ci = 0.02 0.14, Mean: 0.08

Direction: h = 0, p = 0.90, ci = -1.05 0.92, Mean: -0.07

Pressure:  h = 1, p = 0.00, ci = 5.51 9.86, Mean: 7.68

QI:        h = 1, p = 0.00, ci = 0.42 0.86, Mean: 0.64

EUMETSAT "VS" Brazil

Speed:     h = 0, p = 0.14, ci = -0.19 0.03, Mean: -0.08

Direction: h = 1, p = 0.01, ci = 0.56 4.03, Mean: 2.29

Pressure:  h = 1, p = 0.00, ci = -65.97 -55.85, Mean: -60.91

QI:        h = 1, p = 0.00, ci = -7.48 -6.84, Mean: -7.16

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.25 -0.07, Mean: -0.16

Direction: h = 0, p = 0.45, ci = -0.93 2.10, Mean: 0.58

Pressure:  h = 1, p = 0.00, ci = 1.44 5.74, Mean: 3.59

QI:        h = 1, p = 0.00, ci = -6.40 -5.83, Mean: -6.11

Brazil "VS" NWCSAF

Speed:     h = 0, p = 0.10, ci = -0.17 0.01, Mean: -0.08

Direction: h = 1, p = 0.01, ci = -2.91 -0.51, Mean: -1.71

Pressure:  h = 1, p = 0.00, ci = 59.74 69.25, Mean: 64.50

QI:        h = 1, p = 0.00, ci = 0.78 1.32, Mean: 1.05

## Experiment 2 t-test Results

QINF 50 collocated AMVs: ???? File: statsout_exp2.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.21 0.24, Mean: 0.23

Direction: h = 1, p = 0.00, ci = -1.03 -0.48, Mean: -0.76

Pressure:  h = 1, p = 0.00, ci = 1.34 3.08, Mean: 2.21

QI:        h = 1, p = 0.00, ci = -5.81 -5.64, Mean: -5.73

Japan "VS" NOAA

Speed:     h = 1, p = 0.00, ci = 0.05 0.08, Mean: 0.07

Direction: h = 1, p = 0.02, ci = -0.61 -0.06, Mean: -0.33

Pressure:  h = 1, p = 0.00, ci = 21.29 22.85, Mean: 22.07

QI:        h = 1, p = 0.00, ci = -12.05 -11.88, Mean: -11.97

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.36 0.39, Mean: 0.37

Direction: h = 1, p = 0.00, ci = -2.34 -1.70, Mean: -2.02

Pressure:  h = 1, p = 0.00, ci = 28.62 30.21, Mean: 29.41

QI:        h = 1, p = 0.00, ci = -7.19 -7.00, Mean: -7.10

Japan "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 1.03 1.09, Mean: 1.06

Direction: h = 1, p = 0.00, ci = 3.70 4.37, Mean: 4.04

Pressure:  h = 1, p = 0.00, ci = -38.15 -35.91, Mean: -37.03

QI:        h = 1, p = 0.00, ci = -7.94 -7.69, Mean: -7.81

Japan "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.31 0.33, Mean: 0.32

Direction: h = 0, p = 0.92, ci = -0.27 0.25, Mean: -0.01

Pressure:  h = 1, p = 0.00, ci = 31.37 32.80, Mean: 32.08

QI:        h = 1, p = 0.00, ci = -5.56 -5.37, Mean: -5.47

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.18 -0.15, Mean: -0.16

Direction: h = 1, p = 0.00, ci = 0.17 0.68, Mean: 0.42

Pressure:  h = 1, p = 0.00, ci = 19.12 20.60, Mean: 19.86

QI:        h = 1, p = 0.00, ci = -6.31 -6.18, Mean: -6.24

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.13 0.16, Mean: 0.15

Direction: h = 1, p = 0.00, ci = -1.59 -0.94, Mean: -1.26

Pressure:  h = 1, p = 0.00, ci = 26.49 27.92, Mean: 27.21

QI:        h = 1, p = 0.00, ci = -1.45 -1.29, Mean: -1.37

Korea "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.80 0.86, Mean: 0.83

Direction: h = 1, p = 0.00, ci = 4.47 5.11, Mean: 4.79

Pressure:  h = 1, p = 0.00, ci = -40.26 -38.22, Mean: -39.24

QI:        h = 1, p = 0.00, ci = -2.19 -1.99, Mean: -2.09

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.08 0.11, Mean: 0.09

Direction: h = 1, p = 0.00, ci = 0.49 1.00, Mean: 0.74

Pressure:  h = 1, p = 0.00, ci = 29.27 30.48, Mean: 29.88

QI:        h = 1, p = 0.00, ci = 0.19 0.33, Mean: 0.26

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.29 0.33, Mean: 0.31

Direction: h = 1, p = 0.00, ci = -2.01 -1.37, Mean: -1.69

Pressure:  h = 1, p = 0.00, ci = 6.78 7.90, Mean: 7.34

QI:        h = 1, p = 0.00, ci = 4.79 4.95, Mean: 4.87

NOAA "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.96 1.02, Mean: 0.99

Direction: h = 1, p = 0.00, ci = 4.05 4.69, Mean: 4.37

Pressure:  h = 1, p = 0.00, ci = -60.05 -58.14, Mean: -59.10

QI:        h = 1, p = 0.00, ci = 4.05 4.26, Mean: 4.15

NOAA "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.24 0.27, Mean: 0.25

Direction: h = 1, p = 0.01, ci = 0.06 0.57, Mean: 0.32

Pressure:  h = 1, p = 0.00, ci = 9.62 10.41, Mean: 10.01

QI:        h = 1, p = 0.00, ci = 6.42 6.58, Mean: 6.50

EUMETSAT "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.65 0.71, Mean: 0.68

Direction: h = 1, p = 0.00, ci = 5.68 6.43, Mean: 6.05

Pressure:  h = 1, p = 0.00, ci = -67.39 -65.49, Mean: -66.44

QI:        h = 1, p = 0.00, ci = -0.83 -0.60, Mean: -0.72

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.07 -0.04, Mean: -0.05

Direction: h = 1, p = 0.00, ci = 1.69 2.32, Mean: 2.01

Pressure:  h = 1, p = 0.00, ci = 2.23 3.11, Mean: 2.67

QI:        h = 1, p = 0.00, ci = 1.55 1.72, Mean: 1.63

Brazil "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.77 -0.71, Mean: -0.74

Direction: h = 1, p = 0.00, ci = -4.36 -3.74, Mean: -4.05

Pressure:  h = 1, p = 0.00, ci = 68.21 70.02, Mean: 69.11

QI:        h = 1, p = 0.00, ci = 2.23 2.46, Mean: 2.35

QINF 80 collocated AMVs: ???? File: statsout_exp2.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.15 0.18, Mean: 0.16

Direction: h = 1, p = 0.00, ci = -0.76 -0.17, Mean: -0.46

Pressure:  h = 1, p = 0.00, ci = 4.36 6.38, Mean: 5.37

QI:        h = 1, p = 0.00, ci = 0.67 0.76, Mean: 0.71

Japan "VS" NOAA

Speed:     h = 1, p = 0.00, ci = 0.11 0.14, Mean: 0.12

Direction: h = 0, p = 0.11, ci = -0.51 0.05, Mean: -0.23

Pressure:  h = 1, p = 0.00, ci = 16.60 18.30, Mean: 17.45

QI:        h = 1, p = 0.00, ci = -6.20 -6.11, Mean: -6.15

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.25 0.28, Mean: 0.27

Direction: h = 1, p = 0.00, ci = -2.13 -1.48, Mean: -1.81

Pressure:  h = 1, p = 0.00, ci = 23.02 24.80, Mean: 23.91

QI:        h = 1, p = 0.00, ci = -3.40 -3.30, Mean: -3.35

Japan "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.50 0.55, Mean: 0.52

Direction: h = 1, p = 0.00, ci = 3.44 4.16, Mean: 3.80

Pressure:  h = 1, p = 0.00, ci = -44.85 -42.26, Mean: -43.55

QI:        h = 1, p = 0.00, ci = -7.47 -7.36, Mean: -7.42

Japan "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.23 0.25, Mean: 0.24

Direction: h = 0, p = 0.27, ci = -0.12 0.43, Mean: 0.15

Pressure:  h = 1, p = 0.00, ci = 27.67 29.29, Mean: 28.48

QI:        h = 1, p = 0.00, ci = -2.20 -2.10, Mean: -2.15

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.05 -0.03, Mean: -0.04

Direction: h = 0, p = 0.08, ci = -0.03 0.50, Mean: 0.24

Pressure:  h = 1, p = 0.00, ci = 11.20 12.95, Mean: 12.08

QI:        h = 1, p = 0.00, ci = -6.90 -6.83, Mean: -6.87

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.09 0.12, Mean: 0.10

Direction: h = 1, p = 0.00, ci = -1.68 -1.00, Mean: -1.34

Pressure:  h = 1, p = 0.00, ci = 17.69 19.38, Mean: 18.54

QI:        h = 1, p = 0.00, ci = -4.11 -4.02, Mean: -4.07

Korea "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.34 0.38, Mean: 0.36

Direction: h = 1, p = 0.00, ci = 3.91 4.61, Mean: 4.26

Pressure:  h = 1, p = 0.00, ci = -50.16 -47.69, Mean: -48.93

QI:        h = 1, p = 0.00, ci = -8.17 -8.09, Mean: -8.13

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.07 0.09, Mean: 0.08

Direction: h = 1, p = 0.00, ci = 0.35 0.89, Mean: 0.62

Pressure:  h = 1, p = 0.00, ci = 22.39 23.83, Mean: 23.11

QI:        h = 1, p = 0.00, ci = -2.91 -2.82, Mean: -2.87

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.13 0.16, Mean: 0.14

Direction: h = 1, p = 0.00, ci = -1.90 -1.25, Mean: -1.58

Pressure:  h = 1, p = 0.00, ci = 5.83 7.09, Mean: 6.46

QI:        h = 1, p = 0.00, ci = 2.76 2.84, Mean: 2.80

NOAA "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.38 0.43, Mean: 0.40

Direction: h = 1, p = 0.00, ci = 3.68 4.37, Mean: 4.02

Pressure:  h = 1, p = 0.00, ci = -62.11 -59.89, Mean: -61.00

QI:        h = 1, p = 0.00, ci = -1.31 -1.22, Mean: -1.27

NOAA "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.10 0.13, Mean: 0.12

Direction: h = 1, p = 0.00, ci = 0.12 0.64, Mean: 0.38

Pressure:  h = 1, p = 0.00, ci = 10.59 11.47, Mean: 11.03

QI:        h = 1, p = 0.00, ci = 3.95 4.04, Mean: 4.00

EUMETSAT "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.23 0.28, Mean: 0.26

Direction: h = 1, p = 0.00, ci = 5.20 6.01, Mean: 5.60

Pressure:  h = 1, p = 0.00, ci = -68.58 -66.35, Mean: -67.46

QI:        h = 1, p = 0.00, ci = -4.12 -4.02, Mean: -4.07

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.04 -0.01, Mean: -0.03

Direction: h = 1, p = 0.00, ci = 1.63 2.29, Mean: 1.96

Pressure:  h = 1, p = 0.00, ci = 4.08 5.06, Mean: 4.57

QI:        h = 1, p = 0.00, ci = 1.15 1.25, Mean: 1.20

Brazil "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.31 -0.26, Mean: -0.28

Direction: h = 1, p = 0.00, ci = -3.98 -3.30, Mean: -3.64

Pressure:  h = 1, p = 0.00, ci = 70.97 73.10, Mean: 72.03

QI:        h = 1, p = 0.00, ci = 5.21 5.32, Mean: 5.26

QIWF 50 collocated AMVs: ???? File: statsout_exp2.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.18 0.21, Mean: 0.20

Direction: h = 1, p = 0.00, ci = -0.99 -0.45, Mean: -0.72

Pressure:  h = 1, p = 0.00, ci = 1.40 3.13, Mean: 2.26

QI:        h = 1, p = 0.00, ci = 4.35 4.54, Mean: 4.45

Japan "VS" NOAA

Speed:     h = 1, p = 0.00, ci = 0.04 0.07, Mean: 0.06

Direction: h = 1, p = 0.03, ci = -0.56 -0.03, Mean: -0.30

Pressure:  h = 1, p = 0.00, ci = 21.53 23.08, Mean: 22.30

QI:        h = 1, p = 0.00, ci = -5.75 -5.56, Mean: -5.66

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.29 0.33, Mean: 0.31

Direction: h = 1, p = 0.00, ci = -2.34 -1.71, Mean: -2.03

Pressure:  h = 1, p = 0.00, ci = 28.91 30.48, Mean: 29.70

QI:        h = 1, p = 0.00, ci = 0.42 0.62, Mean: 0.52

Japan "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.29 0.32, Mean: 0.30

Direction: h = 0, p = 0.74, ci = -0.21 0.29, Mean: 0.04

Pressure:  h = 1, p = 0.00, ci = 31.66 33.09, Mean: 32.37

QI:        h = 0, p = 0.49, ci = -0.13 0.06, Mean: -0.03

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.15 -0.13, Mean: -0.14

Direction: h = 1, p = 0.00, ci = 0.18 0.67, Mean: 0.43

Pressure:  h = 1, p = 0.00, ci = 19.32 20.76, Mean: 20.04

QI:        h = 1, p = 0.00, ci = -10.16 -10.04, Mean: -10.10

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.10 0.13, Mean: 0.11

Direction: h = 1, p = 0.00, ci = -1.62 -0.99, Mean: -1.30

Pressure:  h = 1, p = 0.00, ci = 26.73 28.13, Mean: 27.43

QI:        h = 1, p = 0.00, ci = -4.00 -3.85, Mean: -3.92

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.10 0.12, Mean: 0.11

Direction: h = 1, p = 0.00, ci = 0.52 1.01, Mean: 0.76

Pressure:  h = 1, p = 0.00, ci = 29.52 30.70, Mean: 30.11

QI:        h = 1, p = 0.00, ci = -4.55 -4.41, Mean: -4.48

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.24 0.27, Mean: 0.25

Direction: h = 1, p = 0.00, ci = -2.03 -1.43, Mean: -1.73

Pressure:  h = 1, p = 0.00, ci = 6.85 7.94, Mean: 7.39

QI:        h = 1, p = 0.00, ci = 6.11 6.25, Mean: 6.18

NOAA "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.23 0.26, Mean: 0.25

Direction: h = 1, p = 0.01, ci = 0.09 0.58, Mean: 0.34

Pressure:  h = 1, p = 0.00, ci = 9.68 10.46, Mean: 10.07

QI:        h = 1, p = 0.00, ci = 5.55 5.69, Mean: 5.62

EUMETSAT "VS" NWCSAF

Speed:     h = 0, p = 0.40, ci = -0.02 0.01, Mean: -0.01

Direction: h = 1, p = 0.00, ci = 1.77 2.37, Mean: 2.07

Pressure:  h = 1, p = 0.00, ci = 2.25 3.10, Mean: 2.68

QI:        h = 1, p = 0.00, ci = -0.63 -0.48, Mean: -0.55

QIWF 80 collocated AMVs: ???? File: statsout_exp2.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.20 0.24, Mean: 0.22

Direction: h = 1, p = 0.00, ci = -0.78 -0.20, Mean: -0.49

Pressure:  h = 1, p = 0.01, ci = 0.28 2.38, Mean: 1.33

QI:        h = 1, p = 0.00, ci = 8.10 8.21, Mean: 8.15

Japan "VS" NOAA

Speed:     h = 1, p = 0.00, ci = 0.07 0.11, Mean: 0.09

Direction: h = 1, p = 0.00, ci = -0.81 -0.29, Mean: -0.55

Pressure:  h = 1, p = 0.00, ci = 12.56 14.49, Mean: 13.53

QI:        h = 1, p = 0.01, ci = -0.13 -0.02, Mean: -0.07

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.16 0.20, Mean: 0.18

Direction: h = 1, p = 0.00, ci = -1.68 -1.09, Mean: -1.38

Pressure:  h = 1, p = 0.00, ci = 21.58 23.53, Mean: 22.56

QI:        h = 1, p = 0.00, ci = 3.19 3.30, Mean: 3.24

Japan "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.21 0.23, Mean: 0.22

Direction: h = 0, p = 0.12, ci = -0.45 0.05, Mean: -0.20

Pressure:  h = 1, p = 0.00, ci = 25.98 27.81, Mean: 26.89

QI:        h = 1, p = 0.00, ci = 2.80 2.91, Mean: 2.85

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.15 -0.11, Mean: -0.13

Direction: h = 0, p = 0.65, ci = -0.32 0.20, Mean: -0.06

Pressure:  h = 1, p = 0.00, ci = 11.29 13.10, Mean: 12.19

QI:        h = 1, p = 0.00, ci = -8.28 -8.18, Mean: -8.23

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = -0.06 -0.02, Mean: -0.04

Direction: h = 1, p = 0.00, ci = -1.21 -0.58, Mean: -0.89

Pressure:  h = 1, p = 0.00, ci = 20.35 22.10, Mean: 21.23

QI:        h = 1, p = 0.00, ci = -4.96 -4.86, Mean: -4.91

Korea "VS" NWCSAF

Speed:     h = 0, p = 0.99, ci = -0.01 0.01, Mean: -0.00

Direction: h = 1, p = 0.03, ci = 0.02 0.56, Mean: 0.29

Pressure:  h = 1, p = 0.00, ci = 24.82 26.30, Mean: 25.56

QI:        h = 1, p = 0.00, ci = -5.36 -5.25, Mean: -5.30

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.07 0.11, Mean: 0.09

Direction: h = 1, p = 0.00, ci = -1.12 -0.55, Mean: -0.83

Pressure:  h = 1, p = 0.00, ci = 8.38 9.69, Mean: 9.03

QI:        h = 1, p = 0.00, ci = 3.27 3.36, Mean: 3.32

NOAA "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.11 0.15, Mean: 0.13

Direction: h = 1, p = 0.00, ci = 0.11 0.59, Mean: 0.35

Pressure:  h = 1, p = 0.00, ci = 12.89 13.85, Mean: 13.37

QI:        h = 1, p = 0.00, ci = 2.88 2.97, Mean: 2.92

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.02 0.06, Mean: 0.04

Direction: h = 1, p = 0.00, ci = 0.89 1.48, Mean: 1.18

Pressure:  h = 1, p = 0.00, ci = 3.83 4.84, Mean: 4.33

QI:        h = 1, p = 0.00, ci = -0.44 -0.35, Mean: -0.39

CQI 50 collocated AMVs: ???? File: statsout_exp2.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.23 0.27, Mean: 0.25

Direction: h = 0, p = 0.84, ci = -0.36 0.29, Mean: -0.03

Pressure:  h = 1, p = 0.00, ci = 5.90 8.04, Mean: 6.97

QI:        h = 1, p = 0.00, ci = -2.55 -2.35, Mean: -2.45

Japan "VS" NOAA

Speed:     h = 1, p = 0.00, ci = 0.10 0.13, Mean: 0.11

Direction: h = 0, p = 0.69, ci = -0.39 0.26, Mean: -0.07

Pressure:  h = 1, p = 0.00, ci = 27.05 28.91, Mean: 27.98

QI:        h = 1, p = 0.00, ci = -4.38 -4.18, Mean: -4.28

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.50 0.55, Mean: 0.53

Direction: h = 0, p = 0.26, ci = -0.16 0.61, Mean: 0.22

Pressure:  h = 1, p = 0.00, ci = 30.94 32.87, Mean: 31.91

QI:        h = 1, p = 0.00, ci = 26.05 26.31, Mean: 26.18

Japan "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.90 0.97, Mean: 0.93

Direction: h = 1, p = 0.00, ci = 3.70 4.48, Mean: 4.09

Pressure:  h = 1, p = 0.00, ci = -27.12 -24.41, Mean: -25.77

QI:        h = 1, p = 0.00, ci = 1.55 1.82, Mean: 1.68

Japan "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.38 0.42, Mean: 0.40

Direction: h = 1, p = 0.00, ci = 0.36 1.00, Mean: 0.68

Pressure:  h = 1, p = 0.00, ci = 34.66 36.38, Mean: 35.52

QI:        h = 1, p = 0.00, ci = -6.32 -6.14, Mean: -6.23

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.15 -0.12, Mean: -0.13

Direction: h = 0, p = 0.83, ci = -0.33 0.26, Mean: -0.03

Pressure:  h = 1, p = 0.00, ci = 20.09 21.93, Mean: 21.01

QI:        h = 1, p = 0.00, ci = -1.92 -1.74, Mean: -1.83

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.26 0.30, Mean: 0.28

Direction: h = 0, p = 0.19, ci = -0.12 0.63, Mean: 0.25

Pressure:  h = 1, p = 0.00, ci = 24.04 25.83, Mean: 24.93

QI:        h = 1, p = 0.00, ci = 28.51 28.75, Mean: 28.63

Korea "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.65 0.71, Mean: 0.68

Direction: h = 1, p = 0.00, ci = 3.76 4.49, Mean: 4.13

Pressure:  h = 1, p = 0.00, ci = -34.01 -31.47, Mean: -32.74

QI:        h = 1, p = 0.00, ci = 4.00 4.26, Mean: 4.13

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.14 0.16, Mean: 0.15

Direction: h = 1, p = 0.00, ci = 0.42 1.01, Mean: 0.72

Pressure:  h = 1, p = 0.00, ci = 27.79 29.30, Mean: 28.55

QI:        h = 1, p = 0.00, ci = -3.86 -3.70, Mean: -3.78

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.39 0.43, Mean: 0.41

Direction: h = 0, p = 0.13, ci = -0.09 0.66, Mean: 0.29

Pressure:  h = 1, p = 0.00, ci = 3.24 4.61, Mean: 3.92

QI:        h = 1, p = 0.00, ci = 30.34 30.58, Mean: 30.46

NOAA "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.79 0.85, Mean: 0.82

Direction: h = 1, p = 0.00, ci = 3.79 4.53, Mean: 4.16

Pressure:  h = 1, p = 0.00, ci = -54.93 -52.57, Mean: -53.75

QI:        h = 1, p = 0.00, ci = 5.84 6.09, Mean: 5.96

NOAA "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.27 0.30, Mean: 0.28

Direction: h = 1, p = 0.00, ci = 0.45 1.05, Mean: 0.75

Pressure:  h = 1, p = 0.00, ci = 7.06 8.01, Mean: 7.54

QI:        h = 1, p = 0.00, ci = -2.03 -1.88, Mean: -1.95

EUMETSAT "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.37 0.44, Mean: 0.41

Direction: h = 1, p = 0.00, ci = 3.44 4.30, Mean: 3.87

Pressure:  h = 1, p = 0.00, ci = -58.85 -56.49, Mean: -57.67

QI:        h = 1, p = 0.00, ci = -24.65 -24.35, Mean: -24.50

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.15 -0.11, Mean: -0.13

Direction: h = 1, p = 0.01, ci = 0.09 0.83, Mean: 0.46

Pressure:  h = 1, p = 0.00, ci = 3.06 4.17, Mean: 3.61

QI:        h = 1, p = 0.00, ci = -32.52 -32.31, Mean: -32.41

Brazil "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.56 -0.50, Mean: -0.53

Direction: h = 1, p = 0.00, ci = -3.78 -3.04, Mean: -3.41

Pressure:  h = 1, p = 0.00, ci = 60.17 62.40, Mean: 61.29

QI:        h = 1, p = 0.00, ci = -8.03 -7.80, Mean: -7.92

CQI 80 collocated AMVs: ???? File: statsout_exp2.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.13 0.20, Mean: 0.16

Direction: h = 0, p = 0.38, ci = -0.67 0.26, Mean: -0.21

Pressure:  h = 1, p = 0.00, ci = 18.45 23.04, Mean: 20.75

QI:        h = 1, p = 0.00, ci = -1.74 -1.54, Mean: -1.64

Japan "VS" NOAA

Speed:     h = 1, p = 0.00, ci = 0.10 0.17, Mean: 0.14

Direction: h = 1, p = 0.04, ci = -0.89 -0.01, Mean: -0.45

Pressure:  h = 1, p = 0.00, ci = 30.25 34.34, Mean: 32.30

QI:        h = 1, p = 0.00, ci = -2.66 -2.47, Mean: -2.56

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.51 0.59, Mean: 0.55

Direction: h = 1, p = 0.00, ci = 1.59 2.62, Mean: 2.11

Pressure:  h = 1, p = 0.00, ci = 32.18 36.37, Mean: 34.27

QI:        h = 1, p = 0.00, ci = 4.53 4.78, Mean: 4.66

Japan "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.51 0.61, Mean: 0.56

Direction: h = 1, p = 0.00, ci = 0.92 1.99, Mean: 1.46

Pressure:  h = 1, p = 0.00, ci = -17.05 -11.41, Mean: -14.23

QI:        h = 1, p = 0.00, ci = -2.40 -2.16, Mean: -2.28

Japan "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.36 0.43, Mean: 0.40

Direction: h = 1, p = 0.03, ci = 0.04 0.85, Mean: 0.45

Pressure:  h = 1, p = 0.00, ci = 39.87 43.69, Mean: 41.78

QI:        h = 1, p = 0.00, ci = -3.16 -2.98, Mean: -3.07

Korea "VS" NOAA

Speed:     h = 0, p = 0.12, ci = -0.06 0.01, Mean: -0.03

Direction: h = 0, p = 0.18, ci = -0.60 0.11, Mean: -0.24

Pressure:  h = 1, p = 0.00, ci = 9.63 13.47, Mean: 11.55

QI:        h = 1, p = 0.00, ci = -1.01 -0.84, Mean: -0.92

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.35 0.43, Mean: 0.39

Direction: h = 1, p = 0.00, ci = 1.76 2.86, Mean: 2.31

Pressure:  h = 1, p = 0.00, ci = 11.64 15.41, Mean: 13.53

QI:        h = 1, p = 0.00, ci = 6.18 6.41, Mean: 6.30

Korea "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.35 0.44, Mean: 0.40

Direction: h = 1, p = 0.00, ci = 1.16 2.17, Mean: 1.66

Pressure:  h = 1, p = 0.00, ci = -37.62 -32.34, Mean: -34.98

QI:        h = 1, p = 0.00, ci = -0.75 -0.53, Mean: -0.64

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.20 0.26, Mean: 0.23

Direction: h = 1, p = 0.00, ci = 0.26 1.05, Mean: 0.66

Pressure:  h = 1, p = 0.00, ci = 19.46 22.61, Mean: 21.03

QI:        h = 1, p = 0.00, ci = -1.51 -1.36, Mean: -1.43

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.37 0.46, Mean: 0.41

Direction: h = 1, p = 0.00, ci = 2.04 3.08, Mean: 2.56

Pressure:  h = 1, p = 0.00, ci = 0.64 3.31, Mean: 1.98

QI:        h = 1, p = 0.00, ci = 7.10 7.33, Mean: 7.22

NOAA "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.37 0.47, Mean: 0.42

Direction: h = 1, p = 0.00, ci = 1.37 2.44, Mean: 1.91

Pressure:  h = 1, p = 0.00, ci = -48.92 -44.14, Mean: -46.53

QI:        h = 1, p = 0.00, ci = 0.17 0.39, Mean: 0.28

NOAA "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.23 0.29, Mean: 0.26

Direction: h = 1, p = 0.00, ci = 0.49 1.31, Mean: 0.90

Pressure:  h = 1, p = 0.00, ci = 8.53 10.44, Mean: 9.48

QI:        h = 1, p = 0.00, ci = -0.59 -0.43, Mean: -0.51

EUMETSAT "VS" Brazil

Speed:     h = 0, p = 0.77, ci = -0.04 0.06, Mean: 0.01

Direction: h = 1, p = 0.03, ci = -1.23 -0.06, Mean: -0.65

Pressure:  h = 1, p = 0.00, ci = -50.88 -46.13, Mean: -48.51

QI:        h = 1, p = 0.00, ci = -7.07 -6.81, Mean: -6.94

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.19 -0.11, Mean: -0.15

Direction: h = 1, p = 0.00, ci = -2.21 -1.11, Mean: -1.66

Pressure:  h = 1, p = 0.00, ci = 6.43 8.59, Mean: 7.51

QI:        h = 1, p = 0.00, ci = -7.83 -7.62, Mean: -7.73

Brazil "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.20 -0.12, Mean: -0.16

Direction: h = 1, p = 0.00, ci = -1.50 -0.52, Mean: -1.01

Pressure:  h = 1, p = 0.00, ci = 53.71 58.32, Mean: 56.01

QI:        h = 1, p = 0.00, ci = -0.89 -0.69, Mean: -0.79

## Experiment 3 t-test Results

QINF 50 collocated AMVs: ???? File: statsout_exp3.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.16 0.19, Mean: 0.18

Direction: h = 0, p = 0.28, ci = -0.42 0.12, Mean: -0.15

Pressure:  h = 1, p = 0.00, ci = -3.32 -1.61, Mean: -2.47

QI:        h = 1, p = 0.00, ci = -5.99 -5.82, Mean: -5.90

Japan "VS" NOAA

Speed:     h = 1, p = 0.00, ci = 0.07 0.09, Mean: 0.08

Direction: h = 1, p = 0.02, ci = -0.57 -0.06, Mean: -0.31

Pressure:  h = 1, p = 0.00, ci = 12.41 13.93, Mean: 13.17

QI:        h = 1, p = 0.00, ci = -12.28 -12.11, Mean: -12.20

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.32 0.36, Mean: 0.34

Direction: h = 1, p = 0.00, ci = -3.52 -2.87, Mean: -3.19

Pressure:  h = 1, p = 0.00, ci = 31.45 33.04, Mean: 32.25

QI:        h = 1, p = 0.00, ci = -7.62 -7.44, Mean: -7.53

Japan "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 1.07 1.13, Mean: 1.10

Direction: h = 1, p = 0.00, ci = 4.51 5.17, Mean: 4.84

Pressure:  h = 1, p = 0.00, ci = -35.12 -32.94, Mean: -34.03

QI:        h = 1, p = 0.00, ci = -7.67 -7.43, Mean: -7.55

Japan "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.27 0.29, Mean: 0.28

Direction: h = 1, p = 0.01, ci = -0.61 -0.08, Mean: -0.35

Pressure:  h = 1, p = 0.00, ci = 29.65 31.07, Mean: 30.36

QI:        h = 1, p = 0.00, ci = -5.70 -5.51, Mean: -5.61

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.11 -0.08, Mean: -0.09

Direction: h = 0, p = 0.19, ci = -0.42 0.08, Mean: -0.17

Pressure:  h = 1, p = 0.00, ci = 14.95 16.33, Mean: 15.64

QI:        h = 1, p = 0.00, ci = -6.36 -6.24, Mean: -6.30

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.15 0.18, Mean: 0.16

Direction: h = 1, p = 0.00, ci = -3.37 -2.72, Mean: -3.05

Pressure:  h = 1, p = 0.00, ci = 34.04 35.39, Mean: 34.72

QI:        h = 1, p = 0.00, ci = -1.70 -1.56, Mean: -1.63

Korea "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.90 0.95, Mean: 0.93

Direction: h = 1, p = 0.00, ci = 4.66 5.31, Mean: 4.98

Pressure:  h = 1, p = 0.00, ci = -32.54 -30.58, Mean: -31.56

QI:        h = 1, p = 0.00, ci = -1.74 -1.55, Mean: -1.65

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.09 0.11, Mean: 0.10

Direction: h = 0, p = 0.12, ci = -0.45 0.05, Mean: -0.20

Pressure:  h = 1, p = 0.00, ci = 32.25 33.40, Mean: 32.83

QI:        h = 1, p = 0.00, ci = 0.23 0.37, Mean: 0.30

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.24 0.27, Mean: 0.26

Direction: h = 1, p = 0.00, ci = -3.20 -2.55, Mean: -2.88

Pressure:  h = 1, p = 0.00, ci = 18.54 19.62, Mean: 19.08

QI:        h = 1, p = 0.00, ci = 4.59 4.74, Mean: 4.67

NOAA "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.99 1.05, Mean: 1.02

Direction: h = 1, p = 0.00, ci = 4.83 5.48, Mean: 5.15

Pressure:  h = 1, p = 0.00, ci = -48.11 -46.28, Mean: -47.20

QI:        h = 1, p = 0.00, ci = 4.55 4.75, Mean: 4.65

NOAA "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.18 0.21, Mean: 0.20

Direction: h = 0, p = 0.81, ci = -0.29 0.22, Mean: -0.03

Pressure:  h = 1, p = 0.00, ci = 16.82 17.56, Mean: 17.19

QI:        h = 1, p = 0.00, ci = 6.52 6.67, Mean: 6.59

EUMETSAT "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.73 0.79, Mean: 0.76

Direction: h = 1, p = 0.00, ci = 7.66 8.40, Mean: 8.03

Pressure:  h = 1, p = 0.00, ci = -67.20 -65.36, Mean: -66.28

QI:        h = 0, p = 0.74, ci = -0.13 0.09, Mean: -0.02

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.08 -0.04, Mean: -0.06

Direction: h = 1, p = 0.00, ci = 2.53 3.16, Mean: 2.85

Pressure:  h = 1, p = 0.00, ci = -2.31 -1.47, Mean: -1.89

QI:        h = 1, p = 0.00, ci = 1.84 2.01, Mean: 1.92

Brazil "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.85 -0.80, Mean: -0.82

Direction: h = 1, p = 0.00, ci = -5.50 -4.86, Mean: -5.18

Pressure:  h = 1, p = 0.00, ci = 63.52 65.27, Mean: 64.39

QI:        h = 1, p = 0.00, ci = 1.83 2.05, Mean: 1.94

QINF 80 collocated AMVs: ???? File: statsout_exp3.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.12 0.15, Mean: 0.14

Direction: h = 0, p = 0.23, ci = -0.49 0.12, Mean: -0.19

Pressure:  h = 1, p = 0.00, ci = -2.78 -0.81, Mean: -1.79

QI:        h = 1, p = 0.00, ci = 0.87 0.96, Mean: 0.91

Japan "VS" NOAA

Speed:     h = 1, p = 0.00, ci = 0.11 0.14, Mean: 0.13

Direction: h = 1, p = 0.00, ci = -0.72 -0.14, Mean: -0.43

Pressure:  h = 1, p = 0.00, ci = 9.02 10.68, Mean: 9.85

QI:        h = 1, p = 0.00, ci = -6.25 -6.16, Mean: -6.20

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.22 0.25, Mean: 0.24

Direction: h = 1, p = 0.00, ci = -2.75 -2.05, Mean: -2.40

Pressure:  h = 1, p = 0.00, ci = 27.42 29.19, Mean: 28.31

QI:        h = 1, p = 0.00, ci = -3.39 -3.30, Mean: -3.34

Japan "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.45 0.50, Mean: 0.48

Direction: h = 1, p = 0.00, ci = 4.32 5.07, Mean: 4.69

Pressure:  h = 1, p = 0.00, ci = -42.99 -40.44, Mean: -41.71

QI:        h = 1, p = 0.00, ci = -7.37 -7.26, Mean: -7.32

Japan "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.20 0.23, Mean: 0.21

Direction: h = 0, p = 0.64, ci = -0.37 0.23, Mean: -0.07

Pressure:  h = 1, p = 0.00, ci = 26.03 27.64, Mean: 26.84

QI:        h = 1, p = 0.00, ci = -2.11 -2.01, Mean: -2.06

Korea "VS" NOAA

Speed:     h = 0, p = 0.18, ci = -0.02 0.00, Mean: -0.01

Direction: h = 0, p = 0.07, ci = -0.51 0.02, Mean: -0.24

Pressure:  h = 1, p = 0.00, ci = 10.85 12.43, Mean: 11.64

QI:        h = 1, p = 0.00, ci = -7.15 -7.08, Mean: -7.11

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.08 0.12, Mean: 0.10

Direction: h = 1, p = 0.00, ci = -2.57 -1.86, Mean: -2.22

Pressure:  h = 1, p = 0.00, ci = 29.31 30.89, Mean: 30.10

QI:        h = 1, p = 0.00, ci = -4.30 -4.22, Mean: -4.26

Korea "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.32 0.36, Mean: 0.34

Direction: h = 1, p = 0.00, ci = 4.51 5.25, Mean: 4.88

Pressure:  h = 1, p = 0.00, ci = -41.11 -38.74, Mean: -39.92

QI:        h = 1, p = 0.00, ci = -8.27 -8.19, Mean: -8.23

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.07 0.09, Mean: 0.08

Direction: h = 0, p = 0.41, ci = -0.16 0.39, Mean: 0.12

Pressure:  h = 1, p = 0.00, ci = 27.96 29.30, Mean: 28.63

QI:        h = 1, p = 0.00, ci = -3.01 -2.93, Mean: -2.97

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.09 0.13, Mean: 0.11

Direction: h = 1, p = 0.00, ci = -2.33 -1.61, Mean: -1.97

Pressure:  h = 1, p = 0.00, ci = 17.87 19.05, Mean: 18.46

QI:        h = 1, p = 0.00, ci = 2.82 2.90, Mean: 2.86

NOAA "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.33 0.37, Mean: 0.35

Direction: h = 1, p = 0.00, ci = 4.76 5.49, Mean: 5.12

Pressure:  h = 1, p = 0.00, ci = -52.64 -50.48, Mean: -51.56

QI:        h = 1, p = 0.00, ci = -1.16 -1.07, Mean: -1.11

NOAA "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.08 0.10, Mean: 0.09

Direction: h = 1, p = 0.01, ci = 0.08 0.64, Mean: 0.36

Pressure:  h = 1, p = 0.00, ci = 16.58 17.40, Mean: 16.99

QI:        h = 1, p = 0.00, ci = 4.10 4.19, Mean: 4.14

EUMETSAT "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.21 0.26, Mean: 0.24

Direction: h = 1, p = 0.00, ci = 6.67 7.52, Mean: 7.10

Pressure:  h = 1, p = 0.00, ci = -71.12 -68.93, Mean: -70.02

QI:        h = 1, p = 0.00, ci = -4.02 -3.92, Mean: -3.97

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.01, ci = -0.04 -0.00, Mean: -0.02

Direction: h = 1, p = 0.00, ci = 1.98 2.68, Mean: 2.33

Pressure:  h = 1, p = 0.00, ci = -1.93 -1.01, Mean: -1.47

QI:        h = 1, p = 0.00, ci = 1.24 1.33, Mean: 1.29

Brazil "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.28 -0.24, Mean: -0.26

Direction: h = 1, p = 0.00, ci = -5.13 -4.40, Mean: -4.76

Pressure:  h = 1, p = 0.00, ci = 67.51 69.60, Mean: 68.55

QI:        h = 1, p = 0.00, ci = 5.21 5.31, Mean: 5.26

QIWF 50 collocated AMVs: ???? File: statsout_exp3.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.13 0.16, Mean: 0.14

Direction: h = 1, p = 0.01, ci = -0.59 -0.07, Mean: -0.33

Pressure:  h = 1, p = 0.00, ci = -2.76 -1.06, Mean: -1.91

QI:        h = 1, p = 0.00, ci = 4.38 4.56, Mean: 4.47

Japan "VS" NOAA

Speed:     h = 1, p = 0.00, ci = 0.05 0.08, Mean: 0.07

Direction: h = 1, p = 0.00, ci = -0.69 -0.18, Mean: -0.43

Pressure:  h = 1, p = 0.00, ci = 12.70 14.24, Mean: 13.47

QI:        h = 1, p = 0.00, ci = -5.77 -5.59, Mean: -5.68

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.28 0.31, Mean: 0.29

Direction: h = 1, p = 0.00, ci = -3.40 -2.77, Mean: -3.09

Pressure:  h = 1, p = 0.00, ci = 31.76 33.35, Mean: 32.55

QI:        h = 1, p = 0.00, ci = 0.50 0.69, Mean: 0.60

Japan "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.24 0.27, Mean: 0.26

Direction: h = 1, p = 0.00, ci = -0.68 -0.16, Mean: -0.42

Pressure:  h = 1, p = 0.00, ci = 30.24 31.67, Mean: 30.96

QI:        h = 0, p = 0.74, ci = -0.08 0.11, Mean: 0.02

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.09 -0.07, Mean: -0.08

Direction: h = 0, p = 0.39, ci = -0.35 0.14, Mean: -0.11

Pressure:  h = 1, p = 0.00, ci = 14.70 16.06, Mean: 15.38

QI:        h = 1, p = 0.00, ci = -10.21 -10.09, Mean: -10.15

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.13 0.16, Mean: 0.15

Direction: h = 1, p = 0.00, ci = -3.07 -2.44, Mean: -2.76

Pressure:  h = 1, p = 0.00, ci = 33.80 35.13, Mean: 34.46

QI:        h = 1, p = 0.00, ci = -3.94 -3.80, Mean: -3.87

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.10 0.12, Mean: 0.11

Direction: h = 0, p = 0.48, ci = -0.33 0.16, Mean: -0.09

Pressure:  h = 1, p = 0.00, ci = 32.30 33.43, Mean: 32.87

QI:        h = 1, p = 0.00, ci = -4.52 -4.39, Mean: -4.45

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.21 0.24, Mean: 0.23

Direction: h = 1, p = 0.00, ci = -2.97 -2.34, Mean: -2.65

Pressure:  h = 1, p = 0.00, ci = 18.55 19.61, Mean: 19.08

QI:        h = 1, p = 0.00, ci = 6.21 6.35, Mean: 6.28

NOAA "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.18 0.20, Mean: 0.19

Direction: h = 0, p = 0.89, ci = -0.23 0.27, Mean: 0.02

Pressure:  h = 1, p = 0.00, ci = 17.12 17.85, Mean: 17.48

QI:        h = 1, p = 0.00, ci = 5.63 5.76, Mean: 5.70

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.05 -0.02, Mean: -0.04

Direction: h = 1, p = 0.00, ci = 2.36 2.98, Mean: 2.67

Pressure:  h = 1, p = 0.00, ci = -2.01 -1.19, Mean: -1.60

QI:        h = 1, p = 0.00, ci = -0.65 -0.51, Mean: -0.58

QIWF 80 collocated AMVs: ???? File: statsout_exp3.txt

apan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.16 0.19, Mean: 0.18

Direction: h = 0, p = 0.35, ci = -0.46 0.16, Mean: -0.15

Pressure:  h = 1, p = 0.00, ci = -7.72 -5.58, Mean: -6.65

QI:        h = 1, p = 0.00, ci = 8.08 8.19, Mean: 8.13

Japan "VS" NOAA

Speed:     h = 1, p = 0.00, ci = 0.10 0.13, Mean: 0.12

Direction: h = 0, p = 0.10, ci = -0.51 0.04, Mean: -0.23

Pressure:  h = 1, p = 0.00, ci = 2.92 4.83, Mean: 3.88

QI:        h = 1, p = 0.00, ci = -0.13 -0.03, Mean: -0.08

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.21 0.25, Mean: 0.23

Direction: h = 1, p = 0.00, ci = -1.65 -1.02, Mean: -1.34

Pressure:  h = 1, p = 0.00, ci = 24.28 26.24, Mean: 25.26

QI:        h = 1, p = 0.00, ci = 3.45 3.57, Mean: 3.51

Japan "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.17 0.20, Mean: 0.19

Direction: h = 0, p = 0.46, ci = -0.38 0.17, Mean: -0.10

Pressure:  h = 1, p = 0.00, ci = 22.64 24.48, Mean: 23.56

QI:        h = 1, p = 0.00, ci = 2.95 3.05, Mean: 3.00

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.08 -0.05, Mean: -0.06

Direction: h = 0, p = 0.53, ci = -0.35 0.18, Mean: -0.09

Pressure:  h = 1, p = 0.00, ci = 9.65 11.39, Mean: 10.52

QI:        h = 1, p = 0.00, ci = -8.26 -8.16, Mean: -8.21

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.03 0.07, Mean: 0.05

Direction: h = 1, p = 0.00, ci = -1.49 -0.88, Mean: -1.19

Pressure:  h = 1, p = 0.00, ci = 31.08 32.73, Mean: 31.91

QI:        h = 1, p = 0.00, ci = -4.68 -4.57, Mean: -4.62

Korea "VS" NWCSAF

Speed:     h = 0, p = 0.34, ci = -0.01 0.02, Mean: 0.01

Direction: h = 0, p = 0.76, ci = -0.24 0.33, Mean: 0.04

Pressure:  h = 1, p = 0.00, ci = 29.46 30.95, Mean: 30.21

QI:        h = 1, p = 0.00, ci = -5.19 -5.08, Mean: -5.13

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.09 0.13, Mean: 0.11

Direction: h = 1, p = 0.00, ci = -1.41 -0.80, Mean: -1.10

Pressure:  h = 1, p = 0.00, ci = 20.77 22.00, Mean: 21.38

QI:        h = 1, p = 0.00, ci = 3.54 3.63, Mean: 3.59

NOAA "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.06 0.08, Mean: 0.07

Direction: h = 0, p = 0.32, ci = -0.13 0.39, Mean: 0.13

Pressure:  h = 1, p = 0.00, ci = 19.21 20.15, Mean: 19.68

QI:        h = 1, p = 0.00, ci = 3.04 3.12, Mean: 3.08

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.06 -0.02, Mean: -0.04

Direction: h = 1, p = 0.00, ci = 0.93 1.53, Mean: 1.23

Pressure:  h = 1, p = 0.00, ci = -2.16 -1.24, Mean: -1.70

QI:        h = 1, p = 0.00, ci = -0.56 -0.46, Mean: -0.51

CQI 50 collocated AMVs: ???? File: statsout_exp3.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.20 0.23, Mean: 0.21

Direction: h = 0, p = 0.87, ci = -0.36 0.30, Mean: -0.03

Pressure:  h = 1, p = 0.00, ci = 0.60 2.72, Mean: 1.66

QI:        h = 1, p = 0.00, ci = -2.95 -2.77, Mean: -2.86

Japan "VS" NOAA

Speed:     h = 1, p = 0.00, ci = 0.11 0.15, Mean: 0.13

Direction: h = 1, p = 0.02, ci = -0.70 -0.07, Mean: -0.38

Pressure:  h = 1, p = 0.00, ci = 15.19 17.04, Mean: 16.11

QI:        h = 1, p = 0.00, ci = -4.41 -4.23, Mean: -4.32

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.47 0.51, Mean: 0.49

Direction: h = 1, p = 0.00, ci = -1.18 -0.43, Mean: -0.81

Pressure:  h = 1, p = 0.00, ci = 31.33 33.30, Mean: 32.32

QI:        h = 1, p = 0.00, ci = 27.56 27.80, Mean: 27.68

Japan "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 1.04 1.10, Mean: 1.07

Direction: h = 1, p = 0.00, ci = 3.36 4.12, Mean: 3.74

Pressure:  h = 1, p = 0.00, ci = -28.25 -25.55, Mean: -26.90

QI:        h = 1, p = 0.00, ci = 1.82 2.08, Mean: 1.95

Japan "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.33 0.36, Mean: 0.35

Direction: h = 0, p = 0.08, ci = -0.61 0.03, Mean: -0.29

Pressure:  h = 1, p = 0.00, ci = 30.81 32.57, Mean: 31.69

QI:        h = 1, p = 0.00, ci = -6.12 -5.95, Mean: -6.03

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.10 -0.07, Mean: -0.09

Direction: h = 1, p = 0.02, ci = -0.65 -0.06, Mean: -0.36

Pressure:  h = 1, p = 0.00, ci = 13.60 15.31, Mean: 14.45

QI:        h = 1, p = 0.00, ci = -1.54 -1.38, Mean: -1.46

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.25 0.30, Mean: 0.28

Direction: h = 1, p = 0.00, ci = -1.16 -0.40, Mean: -0.78

Pressure:  h = 1, p = 0.00, ci = 29.78 31.53, Mean: 30.66

QI:        h = 1, p = 0.00, ci = 30.43 30.66, Mean: 30.54

Korea "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.82 0.89, Mean: 0.86

Direction: h = 1, p = 0.00, ci = 3.39 4.15, Mean: 3.77

Pressure:  h = 1, p = 0.00, ci = -29.78 -27.35, Mean: -28.56

QI:        h = 1, p = 0.00, ci = 4.69 4.94, Mean: 4.81

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.12 0.15, Mean: 0.13

Direction: h = 0, p = 0.09, ci = -0.56 0.04, Mean: -0.26

Pressure:  h = 1, p = 0.00, ci = 29.31 30.75, Mean: 30.03

QI:        h = 1, p = 0.00, ci = -3.24 -3.10, Mean: -3.17

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.34 0.38, Mean: 0.36

Direction: h = 1, p = 0.03, ci = -0.80 -0.05, Mean: -0.42

Pressure:  h = 1, p = 0.00, ci = 15.51 16.89, Mean: 16.20

QI:        h = 1, p = 0.00, ci = 31.89 32.11, Mean: 32.00

NOAA "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.91 0.97, Mean: 0.94

Direction: h = 1, p = 0.00, ci = 3.74 4.51, Mean: 4.13

Pressure:  h = 1, p = 0.00, ci = -44.17 -41.87, Mean: -43.02

QI:        h = 1, p = 0.00, ci = 6.15 6.39, Mean: 6.27

NOAA "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.20 0.23, Mean: 0.22

Direction: h = 0, p = 0.54, ci = -0.21 0.40, Mean: 0.09

Pressure:  h = 1, p = 0.00, ci = 15.13 16.03, Mean: 15.58

QI:        h = 1, p = 0.00, ci = -1.78 -1.65, Mean: -1.71

EUMETSAT "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.54 0.61, Mean: 0.58

Direction: h = 1, p = 0.00, ci = 4.12 4.97, Mean: 4.55

Pressure:  h = 1, p = 0.00, ci = -60.40 -58.04, Mean: -59.22

QI:        h = 1, p = 0.00, ci = -25.87 -25.58, Mean: -25.73

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.17 -0.12, Mean: -0.15

Direction: h = 1, p = 0.01, ci = 0.15 0.89, Mean: 0.52

Pressure:  h = 1, p = 0.03, ci = -1.19 -0.06, Mean: -0.62

QI:        h = 1, p = 0.00, ci = -33.81 -33.62, Mean: -33.72

Brazil "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.76 -0.69, Mean: -0.72

Direction: h = 1, p = 0.00, ci = -4.40 -3.66, Mean: -4.03

Pressure:  h = 1, p = 0.00, ci = 57.49 59.70, Mean: 58.59

QI:        h = 1, p = 0.00, ci = -8.10 -7.87, Mean: -7.99

CQI 80 collocated AMVs: ???? File: statsout_exp3.txt

Japan "VS" Korea

Speed:     h = 1, p = 0.00, ci = 0.18 0.24, Mean: 0.21

Direction: h = 1, p = 0.00, ci = -2.24 -1.04, Mean: -1.64

Pressure:  h = 1, p = 0.00, ci = 2.20 6.77, Mean: 4.49

QI:        h = 1, p = 0.00, ci = -1.65 -1.46, Mean: -1.56

Japan "VS" NOAA

Speed:     h = 1, p = 0.00, ci = 0.09 0.16, Mean: 0.13

Direction: h = 1, p = 0.00, ci = -2.00 -0.88, Mean: -1.44

Pressure:  h = 1, p = 0.00, ci = 18.90 23.02, Mean: 20.96

QI:        h = 1, p = 0.00, ci = -2.66 -2.46, Mean: -2.56

Japan "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.62 0.71, Mean: 0.67

Direction: h = 1, p = 0.00, ci = -2.68 -1.22, Mean: -1.95

Pressure:  h = 1, p = 0.00, ci = 31.30 35.62, Mean: 33.46

QI:        h = 1, p = 0.00, ci = 5.13 5.39, Mean: 5.26

Japan "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.54 0.66, Mean: 0.60

Direction: h = 1, p = 0.00, ci = 1.81 3.05, Mean: 2.43

Pressure:  h = 1, p = 0.00, ci = -21.53 -15.66, Mean: -18.59

QI:        h = 1, p = 0.00, ci = -2.55 -2.31, Mean: -2.43

Japan "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.31 0.38, Mean: 0.34

Direction: h = 1, p = 0.01, ci = -1.30 -0.16, Mean: -0.73

Pressure:  h = 1, p = 0.00, ci = 32.87 36.77, Mean: 34.82

QI:        h = 1, p = 0.00, ci = -3.08 -2.89, Mean: -2.99

Korea "VS" NOAA

Speed:     h = 1, p = 0.00, ci = -0.11 -0.06, Mean: -0.08

Direction: h = 0, p = 0.45, ci = -0.32 0.72, Mean: 0.20

Pressure:  h = 1, p = 0.00, ci = 14.61 18.34, Mean: 16.48

QI:        h = 1, p = 0.00, ci = -1.09 -0.92, Mean: -1.00

Korea "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.41 0.50, Mean: 0.46

Direction: h = 0, p = 0.39, ci = -1.02 0.40, Mean: -0.31

Pressure:  h = 1, p = 0.00, ci = 27.09 30.86, Mean: 28.98

QI:        h = 1, p = 0.00, ci = 6.69 6.94, Mean: 6.81

Korea "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.33 0.44, Mean: 0.39

Direction: h = 1, p = 0.00, ci = 3.33 4.81, Mean: 4.07

Pressure:  h = 1, p = 0.00, ci = -25.75 -20.41, Mean: -23.08

QI:        h = 1, p = 0.00, ci = -0.99 -0.76, Mean: -0.88

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.10 0.16, Mean: 0.13

Direction: h = 1, p = 0.00, ci = 0.32 1.50, Mean: 0.91

Pressure:  h = 1, p = 0.00, ci = 28.77 31.90, Mean: 30.33

QI:        h = 1, p = 0.00, ci = -1.51 -1.35, Mean: -1.43

NOAA "VS" EUMETSAT

Speed:     h = 1, p = 0.00, ci = 0.50 0.58, Mean: 0.54

Direction: h = 0, p = 0.15, ci = -1.20 0.18, Mean: -0.51

Pressure:  h = 1, p = 0.00, ci = 11.04 13.97, Mean: 12.50

QI:        h = 1, p = 0.00, ci = 7.69 7.94, Mean: 7.82

NOAA "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.42 0.53, Mean: 0.47

Direction: h = 1, p = 0.00, ci = 3.16 4.57, Mean: 3.87

Pressure:  h = 1, p = 0.00, ci = -42.05 -37.06, Mean: -39.56

QI:        h = 1, p = 0.02, ci = 0.02 0.24, Mean: 0.13

NOAA "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = 0.18 0.25, Mean: 0.22

Direction: h = 1, p = 0.01, ci = 0.18 1.24, Mean: 0.71

Pressure:  h = 1, p = 0.00, ci = 12.81 14.90, Mean: 13.86

QI:        h = 1, p = 0.00, ci = -0.50 -0.35, Mean: -0.42

EUMETSAT "VS" Brazil

Speed:     h = 1, p = 0.02, ci = -0.13 -0.01, Mean: -0.07

Direction: h = 1, p = 0.00, ci = 3.60 5.16, Mean: 4.38

Pressure:  h = 1, p = 0.00, ci = -54.63 -49.48, Mean: -52.06

QI:        h = 1, p = 0.00, ci = -7.83 -7.55, Mean: -7.69

EUMETSAT "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.37 -0.29, Mean: -0.33

Direction: h = 1, p = 0.00, ci = 0.52 1.93, Mean: 1.22

Pressure:  h = 1, p = 0.03, ci = 0.16 2.55, Mean: 1.36

QI:        h = 1, p = 0.00, ci = -8.36 -8.13, Mean: -8.24

Brazil "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.31 -0.20, Mean: -0.26

Direction: h = 1, p = 0.00, ci = -3.84 -2.48, Mean: -3.16

Pressure:  h = 1, p = 0.00, ci = 50.97 55.86, Mean: 53.42

QI:        h = 1, p = 0.00, ci = -0.66 -0.45, Mean: -0.55

## Experiment 4 t-test Results

QINF 50 collocated AMVs: ???? File: statsout_exp4.txt

Korea "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.94 1.09, Mean: 1.01

Direction: h = 1, p = 0.00, ci = 3.01 4.70, Mean: 3.86

Pressure:  h = 1, p = 0.00, ci = -42.24 -36.57, Mean: -39.40

QI:        h = 1, p = 0.00, ci = 0.50 1.09, Mean: 0.80

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.01, ci = -0.07 -0.01, Mean: -0.04

Direction: h = 0, p = 0.66, ci = -0.78 0.50, Mean: -0.14

Pressure:  h = 1, p = 0.00, ci = 35.37 38.72, Mean: 37.04

QI:        h = 1, p = 0.00, ci = 8.31 8.79, Mean: 8.55

Brazil "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -1.13 -0.98, Mean: -1.05

Direction: h = 1, p = 0.00, ci = -4.80 -3.20, Mean: -4.00

Pressure:  h = 1, p = 0.00, ci = 74.01 78.88, Mean: 76.45

QI:        h = 1, p = 0.00, ci = 7.38 8.12, Mean: 7.75

QINF 80 collocated AMVs: ???? File: statsout_exp4.txt

Missing

QIWF 50 collocated AMVs: ???? File: statsout_exp4.txt

Korea "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.07 -0.01, Mean: -0.04

Direction: h = 0, p = 0.50, ci = -0.83 0.41, Mean: -0.21

Pressure:  h = 1, p = 0.00, ci = 35.15 38.48, Mean: 36.82

QI:        h = 1, p = 0.00, ci = 4.02 4.46, Mean: 4.24

CQI 50 collocated AMVs: ???? File: statsout_exp4.txt

Missing

CQI 80 collocated AMVs: ???? File: statsout_exp4.txt

Korea "VS" Brazil

Speed:     h = 1, p = 0.00, ci = 0.81 0.95, Mean: 0.88

Direction: h = 1, p = 0.00, ci = 3.25 4.93, Mean: 4.09

Pressure:  h = 1, p = 0.00, ci = -39.16 -33.76, Mean: -36.46

QI:        h = 1, p = 0.00, ci = 6.13 6.70, Mean: 6.42

Korea "VS" NWCSAF

Speed:     h = 0, p = 0.59, ci = -0.04 0.02, Mean: -0.01

Direction: h = 0, p = 0.63, ci = -0.81 0.49, Mean: -0.16

Pressure:  h = 1, p = 0.00, ci = 35.89 39.12, Mean: 37.51

QI:        h = 1, p = 0.00, ci = 2.48 2.89, Mean: 2.68

Brazil "VS" NWCSAF

Speed:     h = 1, p = 0.00, ci = -0.95 -0.82, Mean: -0.89

Direction: h = 1, p = 0.00, ci = -5.05 -3.46, Mean: -4.26

Pressure:  h = 1, p = 0.00, ci = 71.63 76.31, Mean: 73.97

QI:        h = 1, p = 0.00, ci = -4.03 -3.44, Mean: -3.73

# Appendix D: Best Fit Speed and Vector Difference

## Experiment 1 Best Fit speed difference

Figure 17-1: BRZ (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-2: BRZ (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-3: EUM (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-4: EUM (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-5: EUM (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-6: JMA (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-7: JMA (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-8: JMA (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-9: KMA (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-10: KMA (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-11: KMA (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-12: NOA (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-13: NOA (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-14: NOA (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-15: NWC (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-16: NWC (QWIF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-17: NWC (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

## Experiment 1 Best Fit vector difference

Figure 17-18: BRZ (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-19: BRZ (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-20: EUM (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-21: EUM (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-22: EUM (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-23: JMA (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-24: JMA (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-25: JMA (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-26: KMA (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-27: KMA (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-28: KMA (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-29: NOA (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-30: NOA (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-31: NOA (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-32: NWC (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-33: NWC (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-34: NWC (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

## Experiment 2 Best Fit speed difference

Figure 17-35: BRZ (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-36: BRZ (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-37: EUM (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-38: EUM (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-39: EUM (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-40: JMA (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-41: JMA (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-42: JMA (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-43: KMA (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-44: KMA (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-45: KMA (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-46: NOA (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-47: NOA (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-48: NOA (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-49: NWC (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-50: NWC (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-51: NWC (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

## Experiment 2 Best Fit vector difference

Figure 17-52: BRZ (QINF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-53: BRZ (CQI): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-54: EUM (QINF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-55: EUM (QIWF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-56: EUM (CQI): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-57: JMA (QINF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-58: JMA (QIWF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-59: JMA (CQI): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-60: KMA (QINF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-61: KMA (QIWF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-62: KMA (CQI): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-63: NOA (QINF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-64: NOA (QIWF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-65: NOA (CQI): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-66: NWC (QINF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-67: NWC (QIWF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-68: NWC (CQI): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

## Experiment 3 Best Fit speed difference

Figure 17-69: BRZ (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-70: BRZ (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-71: EUM (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-72: EUM (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-73: EUM (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-74: JMA (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-75: JMA (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-76: JMA (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-77: KMA (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-78: KMA (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-79: KMA (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-80: NOA (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-81: NOA (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-82: NOA (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-83: NWC (QINF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-84: NWC (QIWF): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-85: NWC (CQI): AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

## Experiment 3 Best Fit vector difference

Figure 17-86: BRZ (QINF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-87: BRZ (CQI): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-88: EUM (QINF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-89: EUM (QIWF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-90: EUM (CQI): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-91: JMA (QINF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-92: JMA (QIWF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-93: JMA (CQI): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-94: KMA (QINF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-95: KMA (QIWF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-96: KMA (CQI): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-97: NOA (QINF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-98: NOA (QIWF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-99: NOA (CQI): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-100: NWC (QINF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-101: NWC (QIWF): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-102: NWC (CQI): AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

## Experiment 4 Best Fit speed difference

Figure 17-103: BRZ: AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-104: EUM: AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-105: JMA: AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-106: KMA: AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-107: NOA: AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-108: NWC: AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-109: NWC: AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-110: NWC: AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-111: NWC: AMV – background speed distribution of all AMVs (upper-left); AMV – background speed distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background speed distribution of Best Fit AMVs (lower-left); AMV – background speed distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

## Experiment 4 Best Fit vector difference

Figure 17-112: BRZ: AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-113: EUM: AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-114: JMA: AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-115: KMA: AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-116: NOA: AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-117: NWC: AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-118: NWC: AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

Figure 17-119: NWC: AMV – background vector difference distribution of all AMVs (upper-left); AMV – background vector difference distribution of all AMVs, including those that are Best Fit adjusted. (upper-right); AMV – background vector difference distribution of Best Fit AMVs (lower-left); AMV – background vector difference distribution of Best Fit AMVs, after Best Fit adjustment (lower-right).

# Appendix E: Shell Scripts

The following scripts were used to preprocess the text files.

## input_fix.sh

input_fix.sh replaces semi-colons with 5 spaces in the text files.

This script ran on ‘granite’. A copy is stored on ‘tinman’ 1 Oct 2013:

/Users/daves/Desktop/Intercomparison/Max_Mindock/AMVIntercomparisonFinalData16Apr2013/ZMiscellaneous/

#**********************************************************************

# input_fix.sh

# This script does a recursive, case sensitive directory search and replace of files

# To make a case insensitive search replace, use the -i switch in the grep call

# uses a startdirectory parameter so that you can run it outside of specified directory - else this script will modify itself!

#**********************************************************************

!/bin/bash

# **************** Change Variables Here ************

startdirectory="/Users/mmindock/Desktop/AMVIntercomparisonFinalData16Apr2013/Japan/GoodData/*.csv"

searchterm="N/A"

replaceterm="0"

# **********************************************************

echo "******************************************"

echo "* Search and Replace in Files Version .1 *"

echo "******************************************"

for file in $(grep -l -R $searchterm $startdirectory)

do

sed 's/;/     /g' $file > output.txt

mv output.txt $file

echo "Modified: " $file

done

echo " *** Yay! All Done! *** "

# Appendix F: Best-Fit Python Scripts

These scripts compute the Best-Fit pressure.

## test_amv.py

Determines AMV best-fit pressure with respect to model background.

On ‘verdandi’ 16 Jan 2014: /home/snebuda/icomp/python/save/

On tinman 21 Jan 2014: /Users/daves/Desktop/Intercomparison/BestFit

#!/usr/bin/env python

import os, os.path

import sys

import numpy as np

import amv as amv

import matplotlib as mpl

import matplotlib.pyplot as plt

from mpl_toolkits.basemap import Basemap

from matplotlib.colors import LogNorm

from pylab import *

undef = -9999.

# European data

file_amv='EUMETSATDatasetTwo.csv'

file_fig='amv.europe.png'

fig_title='European AMV Data'

# Chinese data

file_amv='ChinaDatasetTwo.csv'

file_fig='amv.china.png'

fig_title='Chinese AMV Data'

print "Reading amv data file: "  + file_amv

amv_data = amv.read_txt(file_amv)

print "Reading forecast file"

file_fcst = '/home/daves/intercomparison/nominal/DecodedForecast_20120917070613Z_20120917120000Z_12_O_MPFS03'

fcst_data = amv.read_DecodedForecast_MSG(file_fcst)

print "Finding grid location"

grid_i,grid_j = amv.locate(amv_data,fcst_data)

print "Finding best fit"

amv_num = amv_data.shape[1]

amv_bfit=np.empty(amv_num)

bfit_flag=np.empty(amv_num)

print 'Number of AMV {0}'.format(amv_num)

n = 0

while (n < amv_num):

amv_single = amv_data[:,n]

fcst_profile = fcst_data[grid_i[n],grid_j[n],:,:]

amv_bfit[n],bfit_flag[n] = amv.bestfit(amv_single,fcst_profile)

n += 1

amv_prs = amv_data[4,:]

amv_lat = amv_data[2,:]

amv_lon = amv_data[3,:]

#Compute change in pressure for AMV best fit

dp = amv_bfit[amv_bfit !=undef] - amv_prs[amv_bfit !=undef]

dp_x = amv_lon[amv_bfit != undef]

dp_y = amv_lat[amv_bfit != undef]

dp_x_none = amv_lon[amv_bfit == undef]

dp_y_none = amv_lat[amv_bfit == undef]

dp_x_down = amv_lon[amv_bfit > amv_prs]

dp_y_down = amv_lat[amv_bfit > amv_prs]

dp_x_up = amv_lon[(amv_bfit != undef) & (amv_bfit < amv_prs)]

dp_y_up = amv_lat[(amv_bfit != undef) & (amv_bfit < amv_prs)]

frac = np.empty(4)

flag_title=['Found','Not Constrained','No sufficient minimum','No forecast pressure match']

for f in range (4):

frac[f] = float((bfit_flag == f).sum()) / float(amv_num)

x=amv_data[3,:]

y=amv_data[2,:]

fig = plt.figure(figsize=(12,12))

plt.subplot(3,2,1)

num_bins=50

n,bins,patches = plt.hist(x,num_bins,facecolor='green',alpha=0.5)

n,bins,patches = plt.hist(dp_x,num_bins,facecolor='orange',alpha=0.5)

plt.xlim(-60.,60.)

plt.ylim(0.,600.)

plt.xlabel('Longitudes')

plt.ylabel('Number')

plt.title(r'Green all AMV, Yellow found best fit')

plt.subplot(3,2,3)

num_bins=50

n,bins,patches = plt.hist(y,num_bins,facecolor='green',alpha=0.5)

n,bins,patches = plt.hist(dp_y,num_bins,facecolor='orange',alpha=0.5)

plt.xlim(-60.,60.)

plt.ylim(0.,600.)

plt.xlabel('Latitudes')

plt.ylabel('Number')

#plt.title(r'Green all AMV, Yellow found best fit')

plt.subplot(4,2,7)

#not enough flag=3 to plot for these test cases

labels=flag_title[0:3]

sizes=frac[0:3]

colors=['lightblue','orange','lightgreen','red']

plt.pie(sizes,labels=labels,colors=colors,autopct='%.0f%%')

plt.axis('equal')

plt.subplot(2,2,2)

num_bins=50

n,bins,patches = plt.hist(dp,num_bins,facecolor='blue',alpha=0.5)

plt.xlim(-300.,300.)

plt.ylim(0.,100.)

plt.xlabel('dp')

plt.ylabel('Number')

plt.title(r'Histogram of AMV Best Fit - Original Pressure')

plt.subplot(2,2,4)

plt.scatter(dp_x_none,dp_y_none,s=1,color='0.8')

plt.scatter(dp_x_up,dp_y_up,s=1,color='r')

plt.scatter(dp_x_down,dp_y_down,s=1,color='b')

m = Basemap(projection='cyl',llcrnrlat=-80.,urcrnrlat=80.,llcrnrlon=-80.,urcrnrlon=80.,resolution='c')

m.drawcoastlines()

plt.title(r'Grey no fit, Red higher, Blue lower ')

plt.figtext(0.5,0.96,fig_title,ha='center',color='black',weight='bold',size='large')

#plt.show()

plt.gcf().set_size_inches(13, 13)

plt.savefig(file_fig)

plt.clf()

print "wrote figure file: "  + file_fig

## amv.py

#!/usr/bin/env python

import os, os.path

import sys

import numpy as np

import math

def read_txt(file):

undef = -9999.0

# count the valid AMV

amv_num = 0

with open(file,'r') as f:

for line in f:

try:

amv_list=parse_amv(line)

if (amv_list[0] != undef):

amv_num +=1

except:

continue

f.closed

# allocate np arrays

amv_spd = np.empty(amv_num)

amv_dir = np.empty(amv_num)

amv_prs = np.empty(amv_num)

amv_lat = np.empty(amv_num)

amv_lon = np.empty(amv_num)

# read again to fill np arrays

count = 0

with open(file,'r') as f:

for line in f:

try:

amv_list=parse_amv(line)

if (amv_list[0] != undef):

amv_spd[count] = amv_list[0]

amv_dir[count] = amv_list[1]

amv_lat[count] = amv_list[2]

amv_lon[count] = amv_list[3]

amv_prs[count] = amv_list[4]

count +=1

except:

continue

f.closed

# place in one variable for convenience

amv_data = np.vstack((amv_spd,amv_dir,amv_lat,amv_lon,amv_prs))

return amv_data

def parse_amv(line):

# contain format of text file and data checking in one function

undef = -9999.0

tlat = 1 ; vlat=[-61.,61.]

tlon = 2 ; vlon=[-61.,61.]

tspd = 5 ; vspd=[0.,150.]

tdir = 6 ; vdir=[0.,361.]

tprs = 7 ; vprs=[10.,1020.]

token=line.split()

lat=undef

lon=undef

spd=undef

dir=undef

prs=undef

#print 'lat {0} lon {1} spd {2} dir {3} prs {4}'.format(token[tlat],token[tlon],token[tspd],token[tdir],token[tprs])

# basic valid data check, could add QI or other flag check here

valid=True

if (float(token[tlat]) < vlat[0]) or (float(token[tlat]) > vlat[1]):

valid=False

if (float(token[tlon]) < vlon[0]) or (float(token[tlon]) > vlon[1]):

valid=False

if (float(token[tspd]) < vspd[0]) or (float(token[tspd]) > vspd[1]):

valid=False

if (float(token[tdir]) < vdir[0]) or (float(token[tdir]) > vdir[1]):

valid=False

if (float(token[tprs]) < vprs[0]) or (float(token[tprs]) > vprs[1]):

valid=False

if (valid):

lat = float(token[tlat])

lon = float(token[tlon])

spd = float(token[tspd])

dir = float(token[tdir])

prs = float(token[tprs])

amv_list = [spd,dir,lat,lon,prs]

return amv_list

def write_txt(file,amv_data):

amv_num  = np.size(amv_data,axis=1)

with open(file,'w') as f:

out_string=file+" spd,dir,lat,lon,prs"

f.write(out_string)

f.write("\n")

i = 0

while (i<amv_num):

out_string="{0} {1} {2} {3} {4}".format(amv_data[0,i],amv_data[1,i],amv_data[2,i],amv_data[3,i],amv_data[4,i])

f.write(out_string)

f.write("\n")

i +=1

f.closed

return

def read_DecodedForecast_MSG(file):

"""Usage: DecFcst = read_DecodedForecast_MSG(file)

where, e.g.:

file = '[...]/DecodedForecast_20120503070606Z_20120503120000Z_12_V_MPFS07'

"""

#   	Decoded forecast header (4,776 bytes)

#	Decoded_Forecast_Header = BYTARR(4776)

#   	Decoded forecast data point (40 bytes)

#	Decoded_Forecast_Point = {Latitude      : FLOAT(0), $

#                             Longitude     : FLOAT(0), $

#                             Pressure      : FLOAT(0), $

#                             Geopotential  : FLOAT(0), $

#                             Temperature   : FLOAT(0), $

#                             WVMixingRatio : FLOAT(0), $

#                             O3MixingRatio : FLOAT(0), $

#                             WindSpeed     : FLOAT(0), $

#                             WindDirection : FLOAT(0), $

#                             DewPointTemp  : FLOAT(0)}

#   Decoded forecast data array (29,160,000 bytes)

#   Decoded_Forecast_Array = REPLICATE(Decoded_Forecast_Point, 40L * 135L * 135L)

# This could input defined

num_x = 135

num_y = 135

num_var = 10

num_lev = 40

num_pt = num_x * num_y

arrout = np.zeros((num_x,num_y,10,40))

#   Open file and skip header

fo = open(file)

fo.seek(4776)

print "Program is reading decoded forecast data:" + file

#   '>f4' is float which is big-endian

forecast=np.fromfile(fo,dtype=('>f4'))

fo.close()

fore=np.reshape(forecast,(num_pt,num_lev,num_var))

latitude      = fore[:,:,0]

longitude     = fore[:,:,1]

pressure      = fore[:,:,2]

geopotential  = fore[:,:,3]

temperature   = fore[:,:,4]

wvmixingratio = fore[:,:,5]

o3mixingratio = fore[:,:,6]

windspeed     = fore[:,:,7]

winddirection = fore[:,:,8]

dewpointtemp  = fore[:,:,9]

for j in range(num_pt):

#hardwired for this input file

x = np.floor(longitude[j,0] + 67)

y = np.floor(latitude[j,0] + 67)

arrout[x,y,0,:] = pressure[j,:]

arrout[x,y,1,:] = geopotential[j,:]

arrout[x,y,2,:] = temperature[j,:]

arrout[x,y,3,:] = wvmixingratio[j,:]

arrout[x,y,4,:] = o3mixingratio[j,:]

arrout[x,y,5,:] = windspeed[j,:]

arrout[x,y,6,:] = winddirection[j,:]

arrout[x,y,7,:] = dewpointtemp[j,:]

arrout[x,y,8,:] = latitude[j,:]

arrout[x,y,9,:] = longitude[j,:]

return arrout

def locate(amv_data,fcst_data):

amv_num  = np.size(amv_data,axis=1)

grid_i = np.zeros(amv_num)

grid_j = np.zeros(amv_num)

fcst_lat  = fcst_data[:,:,8,0]

fcst_lon  = fcst_data[:,:,9,0]

numx = np.size(fcst_data,axis=0)

numy = np.size(fcst_data,axis=1)

n = 0

while (n<amv_num):

amv_lat  = amv_data[2,n]

amv_lon  = amv_data[3,n]

grid_diff = (amv_lat-fcst_lat)**2 + (amv_lon-fcst_lon)**2

indx= np.argmin(grid_diff)

indx_2d=np.unravel_index(indx,(numx,numy))

grid_i[n] = indx_2d[0]

grid_j[n] = indx_2d[1]

n +=1

return grid_i,grid_j

def bestfit(amv_data,fcst_data):

"""Finds the background model best fit pressure associated with the AMV.

The model best-fit pressure is the height (in pressure units) where the

vector difference between the observed AMV and model background is a

minimum.  This calculation may only work approximately 1/3 of the time.

Reference:

Salonen et al (2012), "Characterising AMV height assignment error by

comparing best-fit pressure statistics from the Met Office and ECMWF

System."  Proceedings of the 11th International Winds Workshop,

Auckland, New Zealand, 20-24 February 2012.

Input contained in amv_data and fcst_data:

amv_spd -  AMV speed m/s

amv_dir -  AMV direction deg

amv_prs -  AMV pressure hPa

fcst_spd - (level) forecast speed m/s

fcst_dir - (level) forecast direction (deg)

fcst_prs - (level) forecast pressure (hPa)

Output:

amv_bfit - AMV best fit pressure m/s, unconstrained value is undef

flag - 0 found, 1 not contrained, 2 vec diff minimum not met, 3 failed to find suitable fcst pressure match

History:

10/2012 - Steve Wanzong - Created in Fortran

10/2013 - Sharon Nebuda - rewritten for python

"""

undef = -9999.0

amv_spd = amv_data[0]

amv_dir = amv_data[1]

amv_prs = amv_data[4]

amv_lat = amv_data[2]

amv_lon = amv_data[3]

fcst_spd = fcst_data[5,:]

fcst_dir = fcst_data[6,:]

fcst_prs = fcst_data[0,:]

fcst_lat = fcst_data[8,0]

fcst_lon = fcst_data[9,0]

fcst_num_levels = fcst_spd.shape[0]

#   verbose = True

verbose = False

SatwindBestFitPress = undef

SatwindBestFitU = undef

SatwindBestFitV = undef

PressDiff = 150.                      # pressure above and below AMV to look for fit

TopPress = 50.                        # highest level to allow search

flag = 3

#print " AMV location lat,lon,prs ({0},{1},{2})".format(amv_lat,amv_lon,amv_prs)

if (amv_prs<TopPress):

if (verbose):

print " AMV location lat,lon,prs ({0},{1},{2}) is higher than pressure {3}".format(amv_lat,amv_lon,amv_prs,TopPress)

return undef

#Calculate the pressure +/- 150 hPa from the AMV pressure.

PressMax = amv_prs + PressDiff

PressMin = max((amv_prs-PressDiff),TopPress)

#1d array of indicies to consider for best fit location

kk = np.where((fcst_prs<PressMax) & (fcst_prs>PressMin))

if (len(kk[0]) ==0):

if (verbose):

print " AMV location lat,lon,prs ({0},{1},{2}) failed to find fcst prs around AMV".format(amv_lat,amv_lon,amv_prs)

return undef

#Diagnostic field: Find the model minimum speed and maximum speed within PressDiff of the AMV.

if (verbose):

SatwindMinSpeed = min(fcst_spd[kk])

SatwindMaxSpeed = max(fcst_spd[kk])

#Compute U anv V for both AMVs and forecast

# fix this

amv_uwind = -amv_spd * np.sin(math.radians(amv_dir))

amv_vwind = -amv_spd * np.cos(math.radians(amv_dir))

#   fcst_uwind = -fcst_spd[:] * np.sin(math.radians(fcst_dir[:]))

#   fcst_vwind = -fcst_spd[:] * np.cos(math.radians(fcst_dir[:]))

dr=0.017453

fcst_uwind = -fcst_spd * np.sin(dr*fcst_dir)

fcst_vwind = -fcst_spd * np.cos(dr*fcst_dir)

#Calculate the vector difference between the AMV and model background at all levels.

VecDiff = np.sqrt((amv_uwind - fcst_uwind) ** 2 + (amv_vwind - fcst_vwind) ** 2)

#Find the model level of best-fit pressure, from the minimum vector difference.

MinVecDiff = min(VecDiff[kk])

imin=-1

for i, item in enumerate(VecDiff):

if MinVecDiff == VecDiff[i]:

if i in kk[0]:

imin = i

if (imin ==-1 ):

if (verbose):

print " AMV location lat,lon,prs ({0},{1},{2}) failed to find min vector difference in layers around AMV".format(amv_lat,amv_lon,amv_prs)

return undef

#Use a parabolic fit to find the best-fit pressure.

#p2 - Minimized model pressure at level imin (hPa)

#v2 - Minimized vector difference at level imin (m/s)

#p1 - 1 pressure level lower in atmosphere than p2

#p3 - 1 pressure level above in atmosphere than p2

#v1 - Vector difference 1 pressure level lower than p2

#v3 - Vector difference 1 pressure level above than p2

p2 = fcst_prs[imin]

v2 = VecDiff[imin]

# assumes fcst data level 0 at surface and (fcst_num_levels-1) at model top

#if bottom model level

if imin == 0:

SatwindBestFitPress = p2

else:

p3 = fcst_prs[imin+1]

p1 = fcst_prs[imin-1]

v3 = VecDiff[imin+1]

v1 = VecDiff[imin-1]

#if top of allowed region

if p3 < TopPress:

SatwindBestFitPress = p2

#check not collinear

elif (v1 != v2 and v2 != v3):

SatwindBestFitPress = p2 - (0.5 *

((((p2 - p1) * (p2 - p1) * (v2 - v3)) - ((p2 - p3) * (p2 - p3) * (v2 - v1))) /

(((p2 - p1) * (v2 - v3)) - ((p2 - p3) * (v2 - v1)))))

if (SatwindBestFitPress < p3) or (SatwindBestFitPress > p1):

if (verbose):

print " Best Fit not found between two pressure layers"

print " SatwindBestFitPress {0} p1 {1} p2 {2} p3 {3} imin {4} ".format(SatwindBestFitPress,p1,p2,p3,imin)

SatwindBestFitPress = p2

else:

SatwindBestFitPress = p2

#Find best fit U and V by linear interpolation.

if (verbose):

if p2 == SatwindBestFitPress:

SatwindBestFitU = fcst_uwind[imin]

SatwindBestFitV = fcst_vwind[imin]

else:

if p2 < SatwindBestFitPress:

LevBelow = imin - 1

LevAbove = imin

Prop = (SatwindBestFitPress - p1) / (p2 - p1)

else:

LevBelow = imin

LevAbove = imin + 1

Prop = (SatwindBestFitPress - p2) / (p3 - p2)

SatwindBestFitU = fcst_uwind[LevBelow] * (1.0 - Prop) + fcst_vwind[LevAbove] * Prop

SatwindBestFitV = fcst_uwind[LevBelow] * (1.0 - Prop) + fcst_vwind[LevAbove] * Prop

# Check to see if the best fit pressure is constrained.

SatwindGoodConstraint = 0

flag = 2

if MinVecDiff <= 4.0:

SatwindGoodConstraint = 1

flag = 1

for ilev in range(fcst_num_levels):

if fcst_prs[ilev] >= TopPress:

if ((fcst_prs[ilev] < (SatwindBestFitPress - 100.)) or  \

(fcst_prs[ilev] > (SatwindBestFitPress + 100.))) and  \

(VecDiff[ilev] <= (MinVecDiff + 2.0)):

SatwindGoodConstraint = 0

if SatwindGoodConstraint == 1:

amv_bfit = SatwindBestFitPress

flag = 0

else:

amv_bfit = undef

if (verbose):

print "*** AMV best-fit ***"

print "AMV -> p/minspd/maxspd: {0} {1} {2}".format(amv_prs,SatwindMinSpeed,SatwindMaxSpeed)

print "Bestfit -> p1,p2,p3,v1,v2,v3: {0} {1} {2} {3} {4} {5}".format(p1,p2,p3,v1,v2,v3)

print "Bestfit -> pbest,bfu,bfv,obu,obv,bgu,bgv: {0} {1} {2} {3} {4} {5} {6}".format(

SatwindBestFitPress,SatwindBestFitU,SatwindBestFitV,amv_uwind,amv_vwind,fcst_uwind[imin],fcst_vwind[imin])

print "Good Constraint: {0}".format(SatwindGoodConstraint)

print "Minimum Vector Difference: {0}".format(VecDiff[imin])

print "Vector Difference Profile: "

print VecDiff

print "Pressure Profile: "

print fcst_prs

if (abs(SatwindBestFitU - amv_uwind) > 4.0) or  (abs(SatwindBestFitV - amv_vwind) > 4.0):

print 'U Diff: {0}'.format(abs(SatwindBestFitU - amv_uwind))

print 'V Diff: {0}'.format(abs(SatwindBestFitV - amv_vwind))

return amv_bfit,flag

# Appendix G: Matlab Scripts

The Matlab scripts were similar to the ones used by Iliana Genkova in the earlier AMV intercomparison studies, but were modified to:

- Use distance in kilometers instead of degrees of latitude and longitude for determining collocated AMVs.

- Truncate or round values for better comparison if producers reported values with differing precision.

## Winds_Bulk_QINF.m

On ‘tinman’ 8 June 2014: /Users/daves/Documents/MATLAB/Intercomparison

% old data colums - see email to Dave Stettner

% doublecheck.... test with new data, sent away!

% new data columns

% 1. IDN

% 2. LAT[DEG]

% 3. LONG[DEG]

% 4. TBOX[PIX]

% 5. SBOX[PIX]

% 6. SPD[MPS]

% 7. DIR[DEG]

% 8. P[HPA]

% 9. LOWL

% 10.MSPD[MPS]

% 11.MDIR[DEG]

% 12.ALB[%]

% 13.CORR[%]

% 14.TMET

% 15.PERR[HPA]

% 16.HMET

% 17.QINF[%]

% 18.QIF[%]

clear;

% Set exp: 2,3,4

exp=4

qitype=17  % QI with forecast (18) or without (17)

if (exp == 2 )

fall= {'BrazilDatasetTwo.csv',  'EUMETSATDatasetTwo.csv', 'JapanDatasetTwoNew.csv', 'KoreaDatasetTwo.csv', 'NOAADatasetTwo.csv', 'NWCSAFDatasetTwo_O_ET_E.csv',};

fsym= {'b.'                 , 'ro'                     , 'y'                     , 'g^'                 , 'k+'                , 'ms'};

tbsize=[32 24 24 24 19 24];

end

if (exp == 3 )

fall= {'BrazilDatasetThree.csv', 'EUMETSATDatasetThree.csv', 'JapanDatasetThreeNew.csv', 'KoreaDatasetThree.csv', 'NOAADatasetThree.csv', 'NWCSAFDatasetThree_P_ET_E.csv',};

fsym= {'b.'                    , 'ro'                      ,  'y'                      , 'g^'                   , 'k+'                  , 'ms'};

tbsize=[24 24 24 24 24 24];

end

if (exp == 4 )

fall= {'BrazilDatasetFour.csv', 'EUMETSATDatasetFour.csv', 'JapanDatasetFourNew.csv', 'KoreaDatasetFour.csv', 'NOAADatasetFour.csv', 'NWCSAFDatasetFour_O_C_E.csv',};

fsym= {'b.'                   , 'ro'                     , 'y'                      , 'g^'                  , 'k+'                 , 'ms'};

tbsize=[24 24 24 24 24 24];

end

%    figure;

for i=1:6

fname=fall{i};

a=load(fname);

%Remove records with invalid pressure

abad=find(a(:,8)==0);

a(abad,:)=[];

%Remove records with invalid latitude

abad=find(a(:,2) >  90);

a(abad,:)=[];

abad=find(a(:,2) < -90);

a(abad,:)=[];

spd25=find(a(:,6)>=2.5);

a=a(spd25,:);

% Scale JMA QI 0 to 100

if ( i == 3)

a(:,qitype)=a(:,qitype)*100.;

end

good_50=find(a(:,qitype)>=50);

good_80=find(a(:,qitype)>=80);

figure;

subplot(2,3,1);

hold on;

plot(a(:,3),a(:,2),'r.');

plot(a(good_50,3),a(good_50,2),'b.');

plot(a(good_80,3),a(good_80,2),'g.');

legend('All AMV','QI>=50','QI>=80');

xlabel('Lon');

ylabel('Lat');

title(fname);

%     plot(a(good_80,3),a(good_80,2),fsym{i});

xlim([-50  50])

ylim([-50  50])

%     hold on;

disp('**************************');

disp(fname);

fprintf('Target box size in pixels: %d \n',tbsize(i));

fprintf('Total num winds: %d \n',length(a(:,1)));

fprintf('Winds QI>=50: %d \n',length(good_50));

fprintf('Winds QI>=80: %d \n',length(good_80));

fprintf('Lat_min, %6.2f \n', min(a(:,2)));

fprintf('Lat_max, %6.2f \n', max(a(:,2)));

fprintf('Lon_min, %6.2f \n', min(a(:,3)));

fprintf('Lon_max, %6.2f \n', max(a(:,3)));

fprintf(' \n');

fprintf('*** For AMV with QI>=50 ***\n');

alow=find(a(good_50,8)>=700);

amid=find(a(good_50,8)<700 & a(good_50,8)>400);

ahigh=find(a(good_50,8)<=400);

fprintf('SPD_min, %6.2f \n', min(a(good_50,6)));

fprintf('SPD_max, %6.2f \n', max(a(good_50,6)));

fprintf('SPD_mean, %6.2f \n', mean(a(good_50,6)));

fprintf('P_min, %6.2f \n', min(a(good_50,8)));

fprintf('P_max, %6.2f \n', max(a(good_50,8)));

fprintf('P_mean, %6.2f \n', mean(a(good_50,8)));

fprintf('Low_winds, %6.2f \n', 100*length(alow)/length(good_50) );

fprintf('Mid_winds, %6.2f \n', 100*length(amid)/length(good_50) );

fprintf('High_winds, %6.2f \n', 100*length(ahigh)/length(good_50) );

aa=a(good_50,:);

fprintf('Low_SPD_min, %6.2f \n', min(aa(alow,6)));

fprintf('Low_SPD_max, %6.2f \n', max(aa(alow,6)));

fprintf('Low_SPD_mean, %6.2f \n', mean(aa(alow,6)));

fprintf('Low_P_min, %6.2f \n', min(aa(alow,8)));

fprintf('Low_P_max, %6.2f \n', max(aa(alow,8)));

fprintf('Low_P_mean, %6.2f \n', mean(aa(alow,8)));

fprintf('Mid_SPD_min, %6.2f \n', min(aa(amid,6)));

fprintf('Mid_SPD_max, %6.2f \n', max(aa(amid,6)));

fprintf('Mid_SPD_mean, %6.2f \n', mean(aa(amid,6)));

fprintf('Mid_P_min, %6.2f \n', min(aa(amid,8)));

fprintf('Mid_P_max, %6.2f \n', max(aa(amid,8)));

fprintf('Mid_P_mean, %6.2f \n', mean(aa(amid,8)));

fprintf('High_SPD_min, %6.2f \n', min(aa(ahigh,6)));

fprintf('High_SPD_max, %6.2f \n', max(aa(ahigh,6)));

fprintf('High_SPD_mean, %6.2f \n', mean(aa(ahigh,6)));

fprintf('High_P_min, %6.2f \n', min(aa(ahigh,8)));

fprintf('High_P_max, %6.2f \n', max(aa(ahigh,8)));

fprintf('High_P_mean, %6.2f \n', mean(aa(ahigh,8)));

subplot(2,3,2);

hist(a(good_50,qitype),11); % QI

v=axis;

axis([50 100 v(3) v(4)]);

title(['QI' ' ' fname]);

subplot(2,3,3);

hist(a(good_50,6),11); % SPD

v=axis;

axis([0 100 v(3) v(4)]);

title(['SPD' ' ' fname]);

subplot(2,3,4);

hist(a(good_50,7),45); % DIR

v=axis;

axis([0 360 v(3) v(4)]);

title(['DIR' ' ' fname]);

subplot(2,3,5);

hist(a(good_50,8),21); % H

v=axis;

axis([100 1000 v(3) v(4)]);

title(['P' ' ' fname]);

if ( exp == 2)

saveas(gcf, ['Exp_2_QINF_' fname(1:6)], 'tif');

end

if ( exp == 3)

saveas(gcf, ['Exp_3_QINF_' fname(1:6)], 'tif');

end

if ( exp == 4)

saveas(gcf, ['Exp_4_QINF_' fname(1:6)], 'tif');

end

% THIS section doesn't make sense with the new data sets, as they don't have Height Assignment Method. Best to be deleted.

% %     subplot(2,3,6);

% %     aa=a(good_50,:);

% %     if i==1

% %         ind_0=find( a(good_50,26)==1 );

% %         aa(ind_0,26)=0;

% %         ind_1=find( a(good_50,26)==14 );

% %         aa(ind_1,26)=1;

% %         ind_2=find( a(good_50,26)==2 |  a(good_50,26)==3 |  a(good_50,26)==4 |  a(good_50,26)==5 );

% %         aa(ind_2,26)=2;

% %     end;

% %     hist(aa(:,26),0:3); % HAM

% %     title(['HAM' ' ' fname]);

%saveas(gcf,['bulk_' fname(1:3) '.tif'],'tif');

%     figure;

%     hist(a(good_50,2)); % Lat (zonal distribution)

%     title(['Lat' ' ' fname]);

%     saveas(gcf,['hist_lat_' fname(1:3) '.tif'],'tif');

%

%     figure;

%     hist(a(good_50,3)); % Lon

%     title(['Lon' ' ' fname]);

%     saveas(gcf,['hist_lon_' fname(1:3) '.tif'],'tif');

%

%     pause;

%     keyboard;

end;

## Winds_Bulk_QIF.m

On ‘tinman’ 8 June 2014: /Users/daves/Documents/MATLAB/Intercomparison

% old data colums - see email to Dave Stettner

% doublecheck.... test with new data, sent away!

% new data columns

% 1. IDN

% 2. LAT[DEG]

% 3. LONG[DEG]

% 4. TBOX[PIX]

% 5. SBOX[PIX]

% 6. SPD[MPS]

% 7. DIR[DEG]

% 8. P[HPA]

% 9. LOWL

% 10.MSPD[MPS]

% 11.MDIR[DEG]

% 12.ALB[%]

% 13.CORR[%]

% 14.TMET

% 15.PERR[HPA]

% 16.HMET

% 17.QINF[%]

% 18.QIF[%]

clear;

% Set exp: 2,3,4

exp=4

qitype=18  % QI with forecast (18) or without (17)

if (exp == 2 )

fall= {'ChinaDatasetTwoNew.csv',  'EUMETSATDatasetTwo.csv', 'JapanDatasetTwoNew.csv', 'KoreaDatasetTwo.csv', 'NOAADatasetTwo.csv', 'NWCSAFDatasetTwo_O_ET_E.csv',};

fsym= {'b.'                 , 'ro'                     , 'y'                     , 'g^'                 , 'k+'                , 'ms'};

tbsize=[32 24 24 24 19 24];

end

if (exp == 3 )

fall= {'ChinaDatasetThreeNew.csv', 'EUMETSATDatasetThree.csv', 'JapanDatasetThreeNew.csv', 'KoreaDatasetThree.csv', 'NOAADatasetThree.csv', 'NWCSAFDatasetThree_P_ET_E.csv',};

fsym= {'b.'                   , 'ro'                      ,  'y'                      , 'g^'                   , 'k+'                  , 'ms'};

tbsize=[24 24 24 24 24 24];

end

if (exp == 4 )

fall= {'ChinaDatasetFourNew.csv', 'EUMETSATDatasetFour.csv', 'JapanDatasetFourNew.csv', 'KoreaDatasetFour.csv', 'NOAADatasetFour.csv', 'NWCSAFDatasetFour_O_C_E.csv',};

fsym= {'b.'                  , 'ro'                     , 'y'                      , 'g^'                  , 'k+'                 , 'ms'};

tbsize=[24 24 24 24 24 24];

end

%    figure;

for i=1:6

fname=fall{i};

a=load(fname);

%Remove records with invalid pressure

abad=find(a(:,8)==0);

a(abad,:)=[];

%Remove records with invalid latitude

abad=find(a(:,2) >  90);

a(abad,:)=[];

abad=find(a(:,2) < -90);

a(abad,:)=[];

spd25=find(a(:,6)>=2.5);

a=a(spd25,:);

% Scale JMA QI 0 to 100

if ( i == 3)

a(:,qitype)=a(:,qitype)*100.;

end

good_50=find(a(:,qitype)>=50);

good_80=find(a(:,qitype)>=80);

figure;

subplot(2,3,1);

hold on;

plot(a(:,3),a(:,2),'r.');

plot(a(good_50,3),a(good_50,2),'b.');

plot(a(good_80,3),a(good_80,2),'g.');

legend('All AMV','QI>=50','QI>=80');

xlabel('Lon');

ylabel('Lat');

title(fname);

%     plot(a(good_80,3),a(good_80,2),fsym{i});

xlim([-50  50])

ylim([-50  50])

%     hold on;

disp('**************************');

disp(fname);

fprintf('Target box size in pixels: %d \n',tbsize(i));

fprintf('Total num winds: %d \n',length(a(:,1)));

fprintf('Winds QI>=50: %d \n',length(good_50));

fprintf('Winds QI>=80: %d \n',length(good_80));

fprintf('Lat_min, %6.2f \n', min(a(:,2)));

fprintf('Lat_max, %6.2f \n', max(a(:,2)));

fprintf('Lon_min, %6.2f \n', min(a(:,3)));

fprintf('Lon_max, %6.2f \n', max(a(:,3)));

fprintf(' \n');

fprintf('*** For AMV with QI>=50 ***\n');

alow=find(a(good_50,8)>=700);

amid=find(a(good_50,8)<700 & a(good_50,8)>400);

ahigh=find(a(good_50,8)<=400);

fprintf('SPD_min, %6.2f \n', min(a(good_50,6)));

fprintf('SPD_max, %6.2f \n', max(a(good_50,6)));

fprintf('SPD_mean, %6.2f \n', mean(a(good_50,6)));

fprintf('P_min, %6.2f \n', min(a(good_50,8)));

fprintf('P_max, %6.2f \n', max(a(good_50,8)));

fprintf('P_mean, %6.2f \n', mean(a(good_50,8)));

fprintf('Low_winds, %6.2f \n', 100*length(alow)/length(good_50) );

fprintf('Mid_winds, %6.2f \n', 100*length(amid)/length(good_50) );

fprintf('High_winds, %6.2f \n', 100*length(ahigh)/length(good_50) );

aa=a(good_50,:);

fprintf('Low_SPD_min, %6.2f \n', min(aa(alow,6)));

fprintf('Low_SPD_max, %6.2f \n', max(aa(alow,6)));

fprintf('Low_SPD_mean, %6.2f \n', mean(aa(alow,6)));

fprintf('Low_P_min, %6.2f \n', min(aa(alow,8)));

fprintf('Low_P_max, %6.2f \n', max(aa(alow,8)));

fprintf('Low_P_mean, %6.2f \n', mean(aa(alow,8)));

fprintf('Mid_SPD_min, %6.2f \n', min(aa(amid,6)));

fprintf('Mid_SPD_max, %6.2f \n', max(aa(amid,6)));

fprintf('Mid_SPD_mean, %6.2f \n', mean(aa(amid,6)));

fprintf('Mid_P_min, %6.2f \n', min(aa(amid,8)));

fprintf('Mid_P_max, %6.2f \n', max(aa(amid,8)));

fprintf('Mid_P_mean, %6.2f \n', mean(aa(amid,8)));

fprintf('High_SPD_min, %6.2f \n', min(aa(ahigh,6)));

fprintf('High_SPD_max, %6.2f \n', max(aa(ahigh,6)));

fprintf('High_SPD_mean, %6.2f \n', mean(aa(ahigh,6)));

fprintf('High_P_min, %6.2f \n', min(aa(ahigh,8)));

fprintf('High_P_max, %6.2f \n', max(aa(ahigh,8)));

fprintf('High_P_mean, %6.2f \n', mean(aa(ahigh,8)));

subplot(2,3,2);

hist(a(good_50,qitype),11); % QI

v=axis;

axis([50 100 v(3) v(4)]);

title(['QI' ' ' fname]);

subplot(2,3,3);

hist(a(good_50,6),11); % SPD

v=axis;

axis([0 100 v(3) v(4)]);

title(['SPD' ' ' fname]);

subplot(2,3,4);

hist(a(good_50,7),45); % DIR

v=axis;

axis([0 360 v(3) v(4)]);

title(['DIR' ' ' fname]);

subplot(2,3,5);

hist(a(good_50,8),21); % H

v=axis;

axis([100 1000 v(3) v(4)]);

title(['P' ' ' fname]);

if ( exp == 2)

saveas(gcf, ['Exp_2_QIF_' fname(1:6)], 'tif');

end

if ( exp == 3)

saveas(gcf, ['Exp_3_QIF_' fname(1:6)], 'tif');

end

if ( exp == 4)

saveas(gcf, ['Exp_4_QIF_' fname(1:6)], 'tif');

end

% THIS section doesn't make sense with the new data sets, as they don't have Height Assignment Method. Best to be deleted.

% %     subplot(2,3,6);

% %     aa=a(good_50,:);

% %     if i==1

% %         ind_0=find( a(good_50,26)==1 );

% %         aa(ind_0,26)=0;

% %         ind_1=find( a(good_50,26)==14 );

% %         aa(ind_1,26)=1;

% %         ind_2=find( a(good_50,26)==2 |  a(good_50,26)==3 |  a(good_50,26)==4 |  a(good_50,26)==5 );

% %         aa(ind_2,26)=2;

% %     end;

% %     hist(aa(:,26),0:3); % HAM

% %     title(['HAM' ' ' fname]);

%saveas(gcf,['bulk_' fname(1:3) '.tif'],'tif');

%     figure;

%     hist(a(good_50,2)); % Lat (zonal distribution)

%     title(['Lat' ' ' fname]);

%     saveas(gcf,['hist_lat_' fname(1:3) '.tif'],'tif');

%

%     figure;

%     hist(a(good_50,3)); % Lon

%     title(['Lon' ' ' fname]);

%     saveas(gcf,['hist_lon_' fname(1:3) '.tif'],'tif');

%

%     pause;

%     keyboard;

end;

## Winds_Match_QIF.m

On ‘tinman’ 22 June 2014: /Users/daves/Documents/MATLAB/Intercomparison

% collocation of the cgms study datastes - search for a spatial match

% within less than a specified distance

clear;

% Set exp: 2,3,4

exp=4

qitype=18  % QI with forecast (18) or without (17)

dist=55    % Distance in km

qi=50      % QI threshold

if (exp == 2 )

fall= {'EUMETSATDatasetTwo.csv',  'ChinaDatasetTwoNew.csv', 'JapanDatasetTwoNew.csv', 'NOAADatasetTwo.csv' , 'KoreaDatasetTwo.csv', 'NWCSAFDatasetTwo_O_ET_E.csv'};

tbsize=[24 32 24 19 24 24];

end

if (exp == 3 )

fall= {'EUMETSATDatasetThree.csv', 'ChinaDatasetThreeNew.csv', 'JapanDatasetThreeNew.csv', 'NOAADatasetThree.csv' , 'KoreaDatasetThree.csv', 'NWCSAFDatasetThree_P_ET_E.csv'};

tbsize=[24 32 24 19 24 24];

end

if (exp == 4 )

fall= {'EUMETSATDatasetFour.csv', 'ChinaDatasetFourNew.csv', 'JapanDatasetFourNew.csv', 'NOAADatasetFour.csv' , 'KoreaDatasetFour.csv', 'NWCSAFDatasetFour_O_C_E.csv'};

tbsize=[24 24 24 24 24 24];

end

fsym= {'b.'                    ,  'r.'                 , 'g.'                    , 'k.'                 , 'm.'                 , 'y.' };

set_eum=load(fall{1});

set_cma=load(fall{2});

set_jma=load(fall{3});

set_noa=load(fall{4});

set_kma=load(fall{5});

set_nwc_temp=load(fall{6});

set_nwc=sortrows(set_nwc_temp,6);

qivar=[];

qivar=find(set_eum(:,qitype)>=qi);

set_eum=set_eum(qivar,:);

qivar=[];

qivar=find(set_kma(:,qitype)>=qi);

set_kma=set_kma(qivar,:);

qivar=[];

qivar=find(set_cma(:,qitype)>=qi);

set_cma=set_cma(qivar,:);

qivar=[];

qivar=find(set_noa(:,qitype)>=qi);

set_noa=set_noa(qivar,:);

qivar=[];

qivar=find(set_nwc(:,qitype)>=qi);

set_nwc=set_nwc(qivar,:);

qivar=[];

set_jma(:,qitype)=set_jma(:,qitype)*100.;

qivar=find(set_jma(:,qitype)>=qi);

set_jma=set_jma(qivar,:);

i_out=0;

for i_amv=1:length(set_nwc(:,1))

%   disp(i_amv)

[val1, loc1]=min( deg2km(distance(set_eum(:,2), set_eum(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val1 < dist

[val2, loc2]=min( deg2km(distance(set_cma(:,2), set_cma(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val2 < dist

[val3, loc3]=min( deg2km(distance(set_jma(:,2), set_jma(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val3 < dist

[val4, loc4]=min( deg2km(distance(set_noa(:,2), set_noa(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val4 < dist

[val5, loc5]=min( deg2km(distance(set_kma(:,2), set_kma(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val5 < dist

i_out=i_out+1;

set_out_lat(i_out,:) =[ set_eum(loc1,02) set_kma(loc5,02) set_cma(loc2,02)  set_noa(loc4,02) set_nwc(i_amv,02) set_jma(loc3,02) ];

set_out_lon(i_out,:) =[ set_eum(loc1,03) set_kma(loc5,03) set_cma(loc2,03)  set_noa(loc4,03) set_nwc(i_amv,03) set_jma(loc3,03) ];

set_out_spd(i_out,:) =[ set_eum(loc1,06) set_kma(loc5,06) set_cma(loc2,06)  set_noa(loc4,06) set_nwc(i_amv,06) set_jma(loc3,06) ];

set_out_dir(i_out,:) =[ set_eum(loc1,07) set_kma(loc5,07) set_cma(loc2,07)  set_noa(loc4,07) set_nwc(i_amv,07) set_jma(loc3,07) ];

set_out_pres(i_out,:)=[ set_eum(loc1,08) set_kma(loc5,08) set_cma(loc2,08)  set_noa(loc4,08) set_nwc(i_amv,08) set_jma(loc3,08) ];

set_out_ham(i_out,:) =[ set_eum(loc1,16) set_kma(loc5,16) set_cma(loc2,16)  set_noa(loc4,16) set_nwc(i_amv,16) set_jma(loc3,16) ];

set_out_qi(i_out,:)  =[ set_eum(loc1,qitype) set_kma(loc5,qitype) set_cma(loc2,qitype)  set_noa(loc4,qitype) set_nwc(i_amv,qitype) set_jma(loc3,qitype) ];

end;

end;

end;

end

end;

end;

disp('**************');

disp(i_out);

figure;

x=1:i_out;

subplot(4,1,1);

plot(x,set_out_spd(:,1),fsym{1},x,set_out_spd(:,2),fsym{2},x, set_out_spd(:,3),fsym{3},x,set_out_spd(:,4),fsym{4},x,set_out_spd(:,5),fsym{5},x, set_out_spd(:,6), fsym{6} );

legend('EUM', 'KMA', 'CMA', 'NOA', 'NWC', 'JMA');

subplot(4,1,2);

plot(x,set_out_dir(:,1),fsym{1},x,set_out_dir(:,2),fsym{2},x, set_out_dir(:,3),fsym{3},x,set_out_dir(:,4),fsym{4},x,set_out_dir(:,5),fsym{5},x, set_out_dir(:,6),fsym{6} );

subplot(4,1,3);

plot(x,set_out_pres(:,1),fsym{1},x,set_out_pres(:,2),fsym{2},x, set_out_pres(:,3),fsym{3},x,set_out_pres(:,4),fsym{4},x,set_out_pres(:,5),fsym{5},x, set_out_pres(:,6),fsym{6} );

subplot(4,1,4);

plot(x,set_out_qi(:,1),fsym{1},x,set_out_qi(:,2),fsym{2},x, set_out_qi(:,3),fsym{3},x,set_out_qi(:,4),fsym{4},x,set_out_qi(:,5),fsym{5},x,set_out_qi(:,6),fsym{6} );

if ( exp == 2)

saveas(gcf,'Exp_2_qif_all.tif','tif');

end

if ( exp == 3)

saveas(gcf,'Exp_3_qif_all.tif','tif');

end

if ( exp == 4)

saveas(gcf,'Exp_4_qif_all.tif','tif');

end

figure;

plot(set_out_pres(:,1),set_out_pres(:,2),'r.',set_out_pres(:,1), set_out_pres(:,3),'g.', set_out_pres(:,1), set_out_pres(:,4),'k.', set_out_pres(:,1), set_out_pres(:,5),'m.', set_out_pres(:,1), set_out_pres(:,6),'y.');

legend('EUM vs KMA','EUM vs CMA',' EUM vs NOA','EUM vs NWC', 'EUM vs JMA');

xlabel('Pressure (EUM)');

ylabel('Pressure (Centres)');

title('Scatter Plot of Cloud Height');

if ( exp == 2)

saveas(gcf,'Exp_2_qif_pres_scat.tif','tif');

end

if ( exp == 3)

saveas(gcf,'Exp_3_qif_pres_scat.tif','tif');

end

if ( exp == 4)

saveas(gcf,'Exp_4_qif_pres_scat.tif','tif');

end

figure;

plot(x,abs(max(set_out_pres')-min(set_out_pres')),'.');

xlabel('AMV Number');

ylabel('Pressure difference');

title('Maximum Pressure difference');

if ( exp == 2)

saveas(gcf,'Exp_2_qif_pres_hist.tif','tif');

end

if ( exp == 3)

saveas(gcf,'Exp_3_qif_pres_hist.tif','tif');

end

if ( exp == 4)

saveas(gcf,'Exp_4_qif_pres_hist.tif','tif');

end

set_out_names = {'EUMETSAT', 'Korea', 'China', 'NOAA', 'NWCSAF', 'Japan'}

set_out_spd1 = [set_out_spd(:,1), set_out_spd(:,2), set_out_spd(:,3), set_out_spd(:,4), set_out_spd(:,5), set_out_spd(:,6), ((set_out_spd(:,1)+set_out_spd(:,6)+set_out_spd(:,2))/3)];

fall1 = {'EUMETSAT', 'Korea', 'China', 'NOAA', 'NWCSAF', 'Japan', 'Correct'}

set_out_qi1 = [set_out_qi(:,1), set_out_qi(:,2), set_out_qi(:,3), set_out_qi(:,4), set_out_qi(:,5), set_out_qi(:,6), ((set_out_qi(:,3)+set_out_qi(:,5))/2)];

for i=1:5

for n=i:5

fprintf('%s "VS" %s \n', set_out_names{i}, set_out_names{n+1});

[h,p,ci,stats]=ttest(set_out_spd(:,i),set_out_spd(:,n+1));

fprintf('  Speed:     h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n', h, p, ci(1), ci(2), (ci(1)+ci(2))/2);

[h,p,ci,stats]=ttest(set_out_dir(:,i),set_out_dir(:,n+1));

fprintf('  Direction: h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n', h, p, ci(1), ci(2), (ci(1)+ci(2))/2);

[h,p,ci,stats]=ttest(set_out_pres(:,i),set_out_pres(:,n+1));

fprintf('  Pressure:  h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n', h, p, ci(1), ci(2), (ci(1)+ci(2))/2);

[h,p,ci,stats]=ttest(set_out_qi(:,i),set_out_qi(:,n+1));

fprintf('  QI:        h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n\n', h, p, ci(1), ci(2), (ci(1)+ci(2))/2);

end;

end;

%

% %"Correct" Speed Comparison

% for i=1:6

%     fprintf('%s %s \n', 'Speed Correct VS.', fall1{i});

%     [h,p,ci,stats]=ttest(set_out_spd1(:,7),set_out_spd1(:,i));

%     fprintf('Speed "Correct": h = %d, p = %f, ci = %f %f, Mean: %f \n', h, p, ci(1), ci(2), mean(set_out_spd1(:,7)-set_out_spd1(:,i)));

% end;

% fprintf('\n');

% "Correct" QI Comparision

for i=1:6

fprintf('%s %s \n', 'QI Correct VS.', fall1{i});

[h,p,ci,stats]=ttest(set_out_qi1(:,7),set_out_qi1(:,i));

fprintf('Speed "Correct": h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n', h, p, ci(1), ci(2), mean(set_out_qi1(:,7)-set_out_qi1(:,i)));

end;

## Winds_Match_QINF.m

On ‘tinman’ 26 June 2014: /Users/daves/Documents/MATLAB/Intercomparison

% collocation of the cgms study datastes - search for a spatial match

% within less than a specified distance

clear;

% Set exp: 2,3,4

exp=4

qitype=17  % QI with forecast (18) or without (17)

dist=55    % Distance in km

qi=50      % QI threshold

if (exp == 2 )

fall= {'EUMETSATDatasetTwo.csv',  'BrazilDatasetTwo.csv', 'JapanDatasetTwoNew.csv', 'NOAADatasetTwo.csv' , 'KoreaDatasetTwo.csv', 'NWCSAFDatasetTwo_O_ET_E.csv'};

tbsize=[24 32 24 19 24 24];

end

if (exp == 3 )

fall= {'EUMETSATDatasetThree.csv', 'BrazilDatasetThree.csv', 'JapanDatasetThreeNew.csv', 'NOAADatasetThree.csv' , 'KoreaDatasetThree.csv', 'NWCSAFDatasetThree_P_ET_E.csv'};

tbsize=[24 32 24 19 24 24];

end

if (exp == 4 )

fall= {'EUMETSATDatasetFour.csv', 'BrazilDatasetFour.csv', 'JapanDatasetFourNew.csv', 'NOAADatasetFour.csv' , 'KoreaDatasetFour.csv', 'NWCSAFDatasetFour_O_C_E.csv'};

tbsize=[24 24 24 24 24 24];

end

fsym= {'b.'                    ,  'r.'                 , 'g.'                    , 'k.'                 , 'm.'                 , 'y.' };

set_eum=load(fall{1});

set_brz=load(fall{2});

set_jma=load(fall{3});

set_noa=load(fall{4});

set_kma=load(fall{5});

set_nwc_temp=load(fall{6});

set_nwc=sortrows(set_nwc_temp,6);

qivar=[];

qivar=find(set_eum(:,qitype)>=qi);

set_eum=set_eum(qivar,:);

qivar=[];

qivar=find(set_kma(:,qitype)>=qi);

set_kma=set_kma(qivar,:);

qivar=[];

qivar=find(set_brz(:,qitype)>=qi);

set_brz=set_brz(qivar,:);

qivar=[];

qivar=find(set_noa(:,qitype)>=qi);

set_noa=set_noa(qivar,:);

qivar=[];

qivar=find(set_nwc(:,qitype)>=qi);

set_nwc=set_nwc(qivar,:);

qivar=[];

set_jma(:,qitype)=set_jma(:,qitype)*100.;

qivar=find(set_jma(:,qitype)>=qi);

set_jma=set_jma(qivar,:);

i_out=0;

for i_amv=1:length(set_nwc(:,1))

%   disp(i_amv)

[val1, loc1]=min( deg2km(distance(set_eum(:,2), set_eum(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val1 < dist

[val2, loc2]=min( deg2km(distance(set_brz(:,2), set_brz(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val2 < dist

[val3, loc3]=min( deg2km(distance(set_jma(:,2), set_jma(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val3 < dist

[val4, loc4]=min( deg2km(distance(set_noa(:,2), set_noa(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val4 < dist

[val5, loc5]=min( deg2km(distance(set_kma(:,2), set_kma(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val5 < dist

i_out=i_out+1;

set_out_lat(i_out,:) =[ set_eum(loc1,02) set_kma(loc5,02) set_brz(loc2,02)  set_noa(loc4,02) set_nwc(i_amv,02) set_jma(loc3,02) ];

set_out_lon(i_out,:) =[ set_eum(loc1,03) set_kma(loc5,03) set_brz(loc2,03)  set_noa(loc4,03) set_nwc(i_amv,03) set_jma(loc3,03) ];

set_out_spd(i_out,:) =[ set_eum(loc1,06) set_kma(loc5,06) set_brz(loc2,06)  set_noa(loc4,06) set_nwc(i_amv,06) set_jma(loc3,06) ];

set_out_dir(i_out,:) =[ set_eum(loc1,07) set_kma(loc5,07) set_brz(loc2,07)  set_noa(loc4,07) set_nwc(i_amv,07) set_jma(loc3,07) ];

set_out_pres(i_out,:)=[ set_eum(loc1,08) set_kma(loc5,08) set_brz(loc2,08)  set_noa(loc4,08) set_nwc(i_amv,08) set_jma(loc3,08) ];

set_out_ham(i_out,:) =[ set_eum(loc1,16) set_kma(loc5,16) set_brz(loc2,16)  set_noa(loc4,16) set_nwc(i_amv,16) set_jma(loc3,16) ];

set_out_qi(i_out,:)  =[ set_eum(loc1,qitype) set_kma(loc5,qitype) set_brz(loc2,qitype)  set_noa(loc4,qitype) set_nwc(i_amv,qitype) set_jma(loc3,qitype) ];

end;

end;

end;

end

end;

end;

disp('**************');

disp(i_out);

figure;

x=1:i_out;

subplot(4,1,1);

plot(x,set_out_spd(:,1),fsym{1},x,set_out_spd(:,2),fsym{2},x, set_out_spd(:,3),fsym{3},x,set_out_spd(:,4),fsym{4},x,set_out_spd(:,5),fsym{5},x, set_out_spd(:,6), fsym{6} );

legend('EUM', 'KMA', 'BRZ', 'NOA', 'NWC', 'JMA');

subplot(4,1,2);

plot(x,set_out_dir(:,1),fsym{1},x,set_out_dir(:,2),fsym{2},x, set_out_dir(:,3),fsym{3},x,set_out_dir(:,4),fsym{4},x,set_out_dir(:,5),fsym{5},x, set_out_dir(:,6),fsym{6} );

subplot(4,1,3);

plot(x,set_out_pres(:,1),fsym{1},x,set_out_pres(:,2),fsym{2},x, set_out_pres(:,3),fsym{3},x,set_out_pres(:,4),fsym{4},x,set_out_pres(:,5),fsym{5},x, set_out_pres(:,6),fsym{6} );

subplot(4,1,4);

plot(x,set_out_qi(:,1),fsym{1},x,set_out_qi(:,2),fsym{2},x, set_out_qi(:,3),fsym{3},x,set_out_qi(:,4),fsym{4},x,set_out_qi(:,5),fsym{5},x,set_out_qi(:,6),fsym{6} );

if ( exp == 2)

saveas(gcf,'Exp_2_qinf_all.tif','tif');

end

if ( exp == 3)

saveas(gcf,'Exp_3_qinf_all.tif','tif');

end

if ( exp == 4)

saveas(gcf,'Exp_4_qinf_all.tif','tif');

end

figure;

plot(set_out_pres(:,1),set_out_pres(:,2),'r.',set_out_pres(:,1), set_out_pres(:,3),'g.', set_out_pres(:,1), set_out_pres(:,4),'k.', set_out_pres(:,1), set_out_pres(:,5),'m.', set_out_pres(:,1), set_out_pres(:,6),'y.');

legend('EUM vs KMA','EUM vs BRZ',' EUM vs NOA','EUM vs NWC', 'EUM vs JMA');

xlabel('Pressure (EUM)');

ylabel('Pressure (Centres)');

title('Scatter Plot of Cloud Height');

if ( exp == 2)

saveas(gcf,'Exp_2_qinf_pres_scat.tif','tif');

end

if ( exp == 3)

saveas(gcf,'Exp_3_qinf_pres_scat.tif','tif');

end

if ( exp == 4)

saveas(gcf,'Exp_4_qinf_pres_scat.tif','tif');

end

figure;

plot(x,abs(max(set_out_pres')-min(set_out_pres')),'.');

xlabel('AMV Number');

ylabel('Pressure difference');

title('Maximum Pressure difference');

if ( exp == 2)

saveas(gcf,'Exp_2_qinf_pres_hist.tif','tif');

end

if ( exp == 3)

saveas(gcf,'Exp_3_qinf_pres_hist.tif','tif');

end

if ( exp == 4)

saveas(gcf,'Exp_4_qinf_pres_hist.tif','tif');

end

set_out_names = {'EUMETSAT', 'Korea', 'Brazil', 'NOAA', 'NWCSAF', 'Japan'}

set_out_spd1 = [set_out_spd(:,1), set_out_spd(:,2), set_out_spd(:,3), set_out_spd(:,4), set_out_spd(:,5), set_out_spd(:,6), ((set_out_spd(:,1)+set_out_spd(:,2))/2)];

fall1 = {'EUMETSAT', 'Korea', 'Brazil', 'NOAA', 'NWCSAF', 'Japan', 'Correct'};

for i=1:5

for n=i:5

fprintf('%s "VS" %s \n', set_out_names{i}, set_out_names{n+1});

[h,p,ci,stats]=ttest(set_out_spd(:,i),set_out_spd(:,n+1));

fprintf('  Speed:     h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n', h, p, ci(1), ci(2), (ci(1)+ci(2))/2);

[h,p,ci,stats]=ttest(set_out_dir(:,i),set_out_dir(:,n+1));

fprintf('  Direction: h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n', h, p, ci(1), ci(2), (ci(1)+ci(2))/2);

[h,p,ci,stats]=ttest(set_out_pres(:,i),set_out_pres(:,n+1));

fprintf('  Pressure:  h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n', h, p, ci(1), ci(2), (ci(1)+ci(2))/2);

[h,p,ci,stats]=ttest(set_out_qi(:,i),set_out_qi(:,n+1));

fprintf('  QI:        h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n\n', h, p, ci(1), ci(2), (ci(1)+ci(2))/2);

end;

end;

fprintf('\n')

%

%

%"Correct" Speed Comparison

for i=1:6

fprintf('%s %s \n', 'Speed Correct VS.', fall1{i});

[h,p,ci,stats]=ttest(set_out_spd1(:,7),set_out_spd1(:,i));

fprintf('Speed "Correct": h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n', h, p, ci(1), ci(2), mean(set_out_spd1(:,7)-set_out_spd1(:,i)));

end;

## Stats_QIF_One.m

On ‘tinman’ 27 June 2014: /Users/daves/Documents/MATLAB/Intercomparison

% collocation of the cgms study datasets - search for a spatial match

% within less than a specified distance in km

clear;

fall= {'EUMETSATDatasetOne.csv', 'ChinaDatasetOneNew.csv', 'JapanDatasetOneNew.csv', 'NOAADatasetOne.csv' , 'KoreaDatasetOne.csv', 'NWCSAFDatasetOne_O_C_WO_N.csv', 'BrazilDatasetOne.csv'};

fsym= {'b+'                    , 'r.'                    , 'g.'                    , 'k.'                 , 'm.'                 , 'y.'                           , 'b.' };

tbsize=[24 32 24 19 24 24 24];

%Use subpixel

fall{6}='NWCSAFDatasetOne_O_C_S_E.csv'

set_eum_temp=load(fall{1});

gooddir=find(set_eum_temp(:,7) <= 360);

set_eum=set_eum_temp(gooddir,:);

set_kma=load(fall{5});

set_cma=load(fall{2});

set_noa=load(fall{4});

set_nwc_temp=load(fall{6});

set_jma=load(fall{3});

set_brz=load(fall{7});

set_nwc=sortrows(set_nwc_temp,6);

qi=0       % VARIABLE

qivar=[];

qivar=find(set_eum(:,18)>=qi);

set_eum=set_eum(qivar,:);

qivar=[];

qivar=find(set_kma(:,18)>=qi);

set_kma=set_kma(qivar,:);

qivar=[];

qivar=find(set_cma(:,18)>=qi);

set_cma=set_cma(qivar,:);

qivar=[];

qivar=find(set_noa(:,18)>=qi);

set_noa=set_noa(qivar,:);

qivar=[];

qivar=find(set_nwc(:,18)>=qi);

set_nwc=set_nwc(qivar,:);

qivar=[];

qivar=find(set_jma(:,18)>=qi);

set_jma=set_jma(qivar,:);

qivar=[];

qivar=find(set_brz(:,18)>=qi);

set_brz=set_brz(qivar,:);

i_out=0;

dist=35

for i_amv=1:length(set_nwc(:,1))

%   disp(i_amv)

[val1 loc1]=min( deg2km(distance(set_eum(:,2), set_eum(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val1 < dist

[val2 loc2]=min( deg2km(distance(set_cma(:,2), set_cma(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val2 < dist

[val3 loc3]=min( deg2km(distance(set_jma(:,2), set_jma(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val3 < dist

[val4 loc4]=min( deg2km(distance(set_noa(:,2), set_noa(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val4 < dist

[val5 loc5]=min( deg2km(distance(set_kma(:,2), set_kma(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val5 < dist

[val6 loc6]=min( deg2km(distance(set_brz(:,2), set_brz(:,3), set_nwc(i_amv,2), set_nwc(i_amv,3)))); % lat/lon metrics

if val6 < dist

i_out=i_out+1;

set_out_lat(i_out,:) =[ set_eum(loc1,2) set_kma(loc5,2) set_cma(loc2,2)  set_noa(loc4,2) set_nwc(i_amv,2) set_jma(loc3,2) set_brz(loc6,2) ];

set_out_lon(i_out,:) =[ set_eum(loc1,3) set_kma(loc5,3) set_cma(loc2,3)  set_noa(loc4,3) set_nwc(i_amv,3) set_jma(loc3,3) set_brz(loc6,3) ];

set_out_spd(i_out,:) =[ set_eum(loc1,6) set_kma(loc5,6) set_cma(loc2,6)  set_noa(loc4,6) set_nwc(i_amv,6) set_jma(loc3,6) set_brz(loc6,6) ];

set_out_dir(i_out,:) =[ set_eum(loc1,7) set_kma(loc5,7) set_cma(loc2,7)  set_noa(loc4,7) set_nwc(i_amv,7) set_jma(loc3,7) set_brz(loc6,7) ];

% set_out_tem(i_out,:) =[ set_eum(loc1,13) set_kma(loc5,13) set_cma(loc2,13)  set_noa(loc4,13) set_nwc(i_amv,13) set_jma(loc3,13) ];

set_out_pres(i_out,:)=[ set_eum(loc1, 8)  set_kma(loc5, 8)  set_cma(loc2, 8)   set_noa(loc4, 8) set_nwc(i_amv, 8)  set_jma(loc3, 8)  set_brz(loc6, 8) ];

set_out_ham(i_out,:) =[ set_eum(loc1,16)  set_kma(loc5,16)  set_cma(loc2,16)   set_noa(loc4,16) set_nwc(i_amv,16)  set_jma(loc3,16)  set_brz(loc6,16) ];

set_out_qi(i_out,:)  =[ set_eum(loc1,18)  set_kma(loc5,18)  set_cma(loc2,18)   set_noa(loc4,18) set_nwc(i_amv,18)  set_jma(loc3,18)  set_brz(loc6,18) ];

set_out_h1(i_out,:) = [ set_eum(loc1,19) -set_kma(loc5,19) -set_cma(loc2,19)  -set_noa(loc4,19) set_nwc(i_amv,19) -set_jma(loc3,19) -set_brz(loc6,20) ];

set_out_v1(i_out,:) = [ set_eum(loc1,20) -set_kma(loc5,20) -set_cma(loc2,20)   set_noa(loc4,20) set_nwc(i_amv,20)  set_jma(loc3,20)  set_brz(loc6,19) ];

set_out_h2(i_out,:) = [ set_eum(loc1,21) -set_kma(loc5,21)  set_cma(loc2,21)   set_noa(loc4,21) set_nwc(i_amv,21) -set_jma(loc3,21)  set_brz(loc6,22) ];

set_out_v2(i_out,:) = [ set_eum(loc1,22) -set_kma(loc5,22)  set_cma(loc2,22)  -set_noa(loc4,22) set_nwc(i_amv,22)  set_jma(loc3,22) -set_brz(loc6,21) ];

end;

end;

end;

end;

end

end;

end;

set_out_names = {'EUMETSAT', 'Korea', 'China', 'NOAA', 'NWCSAF', 'Japan', 'Brazil'}

i_out

% set_out_spd1 = [set_out_spd(:,1), set_out_spd(:,2), set_out_spd(:,3), set_out_spd(:,4), set_out_spd(:,5), set_out_spd(:,6), set_out_spd(:,7) ];

% fall1 = {'EUMETSAT', 'Korea', 'China', 'NOAA', 'NWCSAF', 'Japan', 'Brazil'}

% set_out_qi1 = [set_out_qi(:,1), set_out_qi(:,2), set_out_qi(:,3), set_out_qi(:,4), set_out_qi(:,5), set_out_qi(:,6), set_out_qi(:,7) ];

for i=1:6

for n=i:6

fprintf('%s vs. %s \n', set_out_names{i}, set_out_names{n+1});

[h,p,ci,stats]=ttest(set_out_spd(:,i),set_out_spd(:,n+1));

fprintf('  Speed:  h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n', h, p, ci(1), ci(2), (ci(1)+ci(2))/2);

[h,p,ci,stats]=ttest(set_out_dir(:,i),set_out_dir(:,n+1));

fprintf('  Dir:    h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n', h, p, ci(1), ci(2), (ci(1)+ci(2))/2);

[h,p,ci,stats]=ttest(set_out_h1(:,i),set_out_h1(:,n+1));

fprintf('  Hdisp1: h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n', h, p, ci(1), ci(2), (ci(1)+ci(2))/2);

[h,p,ci,stats]=ttest(set_out_v1(:,i),set_out_v1(:,n+1));

fprintf('  Vdisp1: h = %d, p = %.2f, ci = %.f %.2f, Mean: %.2f \n', h, p, ci(1), ci(2), (ci(1)+ci(2))/2);

[h,p,ci,stats]=ttest(set_out_h2(:,i),set_out_h2(:,n+1));

fprintf('  Hdisp2: h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n', h, p, ci(1), ci(2), (ci(1)+ci(2))/2);

[h,p,ci,stats]=ttest(set_out_v2(:,i),set_out_v2(:,n+1));

fprintf('  Vdisp2: h = %d, p = %.2f, ci = %.2f %.2f, Mean: %.2f \n', h, p, ci(1), ci(2), (ci(1)+ci(2))/2);

end;

end;

%

% %"Correct" Speed Comparison

% for i=1:6

%     fprintf('%s %s \n', 'Speed Correct VS.', fall1{i});

%     [h,p,ci,stats]=ttest(set_out_spd1(:,7),set_out_spd1(:,i));

%     fprintf('Speed "Correct": h = %d, p = %f, ci = %f %f, Mean: %f \n', h, p, ci(1), ci(2), mean(set_out_spd1(:,7)-set_out_spd1(:,i)));

% end;

% fprintf('\n');

% "Correct" QI Comparision

%for i=1:6

%    fprintf('%s %s \n', 'QI Correct vs.', fall1{i});

%    [h,p,ci,stats]=ttest(set_out_qi1(:,7),set_out_qi1(:,i));

%    fprintf('Speed "Correct": h = %d, p = %f, ci = %f %f, Mean: %f \n', h, p, ci(1), ci(2), mean(set_out_qi1(:,7)-set_out_qi1(:,i)));

%end;

## Winds_Height_Diff.m

On ‘tinman’ 9 June 2014: /Users/daves/Documents/MATLAB/Intercomparison

% Experiment 3 & 4 AMV height difference

% collocation of the cgms study datasets - search for a spatial match

% within less than a specified distance, dist

% 1. IDN

% 2. LAT[DEG]

% 3. LONG[DEG]

% 4. TBOX[PIX]

% 5. SBOX[PIX]

% 6. SPD[MPS]

% 7. DIR[DEG]

% 8. P[HPA]

% 9. LOWL

% 10.GSPD[MPS]

% 11.GDIR[DEG]

% 12.ALB[%]

% 13.CORR[%]

% 14.TMET

% 15.PERR[HPA]

% 16.HMET

% 17.QINF[%]

% 18.QIF[%]

% 19.HDISP1

% 20.VDISP1

% 21.HDISP2

% 22.VDISP2

clear;

close all;

kindex=7

dist=2;   % distance in km (set to 2 for most) =12 for NWC (kindex=6)

if (kindex == 1 )

fall= {'ChinaDatasetThreeNew.csv',  'ChinaDatasetFourNew.csv'};

fsym= {'b.'                    , 'ro'};

site= 'CMA';

end

if (kindex == 2 )

fall= {'EUMETSATDatasetThree.csv',  'EUMETSATDatasetFour.csv'};

fsym= {'b.'                    , 'ro'};

site= 'EUM';

end

if (kindex == 3 )

fall= {'JapanDatasetThreeNew.csv',  'JapanDatasetFourNew.csv'};

fsym= {'b.'                    , 'ro'};

site= 'JMA';

end

if (kindex == 4 )

fall= {'KoreaDatasetThree.csv',  'KoreaDatasetFour.csv'};

fsym= {'b.'                    , 'ro'};

site= 'KMA';

end

if (kindex == 5 )

fall= {'NOAADatasetThree.csv',  'NOAADatasetFour.csv'};

fsym= {'b.'                    , 'ro'};

site= 'NOA';

end

if (kindex == 6 )

fall= {'NWCSAFDatasetThree_P_ET_E.csv',  'NWCSAFDatasetFour_P_C_E.csv'};

fsym= {'b.'                    , 'ro'};

site= 'NWC';

end

if (kindex == 7 )

fall= {'BrazilDatasetThree.csv',  'BrazilDatasetFour.csv'};

fsym= {'b.'                    , 'ro'};

site= 'BRZ';

end

set_three=load(fall{1});

set_four=load(fall{2});

qi=0;

qivar=[];

qivar=find(set_three(:,18)>=qi);

set_three=set_three(qivar,:);

qivar=[];

qivar=find(set_four(:,18)>=qi & set_four(:,2) < 90. );

set_four=set_four(qivar,:);

i_out=0;

for i_amv=1:length(set_four(:,1))

%   disp(i_amv)

[val1 loc1]=min( deg2km(distance(set_three(:,2), set_three(:,3), set_four(i_amv,2), set_four(i_amv,3)))); % lat/lon metrics

if val1 < dist

i_out=i_out+1;

set_out_lat(i_out,:) =[ set_three(loc1,2) set_four(i_amv,2) ];

set_out_lon(i_out,:) =[ set_three(loc1,3) set_four(i_amv,3) ];

set_out_spd(i_out,:) =[ set_three(loc1,6) set_four(i_amv,6) ];

set_out_dir(i_out,:) =[ set_three(loc1,7) set_four(i_amv,7) ];

set_out_pres(i_out,:) =[ set_three(loc1,8)  set_four(i_amv,8)  ];

set_out_ham(i_out,:) = [ set_three(loc1,16) set_four(i_amv,16) ];

set_out_qi(i_out,:) =  [ set_three(loc1,18) set_four(i_amv,18) ];

set_out_ind(i_out,:) = [ loc1 i_amv ];

end;

end;

disp('**************');

disp(i_out);

figure(1);

%x=1:i_out;

%subplot(2,2,1);

%plot(x,set_out_pres(:,1),fsym{1},x,set_out_pres(:,2),fsym{2});

%legend(site);

%subplot(2,2,2);

%plot(set_out_pres(:,1),set_out_pres(:,2),'k.');

%subplot(2,2,3);

%plot(x,set_out_pres(:,2)-set_out_pres(:,1),'.');

%subplot(2,2,4);

hist(set_out_pres(:,2)-set_out_pres(:,1),100);

title(site);

xlabel('pressure diff: Exp4 - Exp3');

saveas(gcf,['Pdiff_' site],'tif');