The STPS is a data pipeline that consists of linked nodes. The first node is always the Frame Synchronizer, but the others vary depending on the selected configuration.

The nodes deliver data to each other in one of the following formats. All unit types may have attached annotation consisting of quality and time.

- Frames - A frame must have a 2-4 bytes synchronization pattern, and it may have CRC or Reed Solomon parity. It may or may not be CCSDS compliant.

- Packets - A CCSDS compliant packet.

- Units - Anything that is not a frame or packet such as VCDUs and bitstream service MPDUs.

Every node implements one or more of the following interfaces to pass frames, packets, and units to each other. It is these interfaces that allow us to link the nodes together in many different configurations.

- Frame receiver

- Packet receiver

- Unit receiver

- Frame sender

- Packet sender

- Unit sender

The nodes have these interface characteristics:

- All nodes except for the Frame Synchronizer are frame, packet, or unit receivers.

- The Frame Synchronizer receives buffers of unsynchronized telemetry data.

- All nodes except for the output nodes are frame, packet, or unit senders.

- The output nodes write to output devices such as sockets and files and do not send to other nodes.

- All senders are capable of broadcasting the same data to multiple nodes.

- A STPS configuration defines which nodes must be created and specifies which nodes are linked to each other.

- Nodes may be linked in unusual and even silly ways as long as they satisfy the interface requirements. (For example, the Reed Solomon decoder can send frames directly to a socket.)

- A node may delete a frame, packet, or unit if it fails user-specified quality requirements.

A configuration defines a telemetry-processing pipeline, which is a set of nodes linked together in a proscribed fashion. A Java program creates a pipeline.

- All programs must be loaded with an XML-compliant configuration that defines all nodes and their linkages.

- The "batch" program reads an input file of unsynchronized telemetry and gives it to the pipeline. It runs once and terminates.

- The "server" program reads a UDP socket and gives the data to the pipeline. It run continuously and waits for setup and shutdown commands from a local or remote user.

- The output from the programs (socket or file) is defined in the configuration.

These nodes are frame processors and do not require CCSDS telemetry.

- They are usually linked in the table order when they exist.

- Only one of each type can exist in a pipeline.

| Node | Receives | Sends | Description |
| --- | --- | --- | --- |
| Frame Synchronizer | Buffers | Frames | Finds a frame synchronization pattern creates frames. Required. |
| PN Decoder | Frames | Frames | Removes pseudo-noise encoding. Optional. |
| CRC Decoder | Frames | Frames | Looks for frame CRC errors. Optional. |
| Reed Solomon Decoder | Frames | Frames | Looks for frame Reed Solomon parity errors. Optional. |
| Frame Status | Frames | Frames | Collects frame counts including frame error counts. Optional but almost always present. |

These nodes handle CCSDS services.

- The CADU Service node usually receives frames from the Frame Status node.

- CADU Service is required for CCSDS processing. It is a singleton node.

- The other CCSDS service nodes are optional, but more than one of each type may exist.

- The CADU Service node has conditional links, which are defined by spacecraft ID and virtual channel ID, to the service nodes.

- Each service node does VCDU sequence checking, so each one expects one virtual channel input stream, but it is not required.

- Other service nodes (insert zone for example) are not currently implemented but may be added later.

| Node | Receives | Sends | Description |
| --- | --- | --- | --- |
| CADU Service | Frames | CADUs | Sorts CADUs by spacecraft and virtual channel and sends them to the linked service node. |
| VCDU Service | CADUs | VCDUs | Extracts VCDUs from the CADUs |
| Bitstream Service | CADUs | B_PDUs | Extracts B_PDUs from the CADUs |
| Path Service | CADUs | Packets | Assembles packets and routes them to the proper Packet Pipeline node. |

These nodes handle CCSDS packet services.

- Each Path Service node receives CADUs for a virtual channel from the CADU Service node. It could handle more than one virtual channel.

- A Path Service node routes packets by their application IDs.

- Each Packet Pipeline node usually does one application ID for a virtual channel.

| Node | Receives | Sends | Description |
| --- | --- | --- | --- |
| Path Service | CADUs | Packets | Assembles packets and routes them to the proper Packet Pipeline node. |
| Packet Pipeline | Packets | Packets | Performs sequence and error checking. |

These nodes represent output devices.

- All nodes are optional. More than one of each type may exist.

- The frame annotation, which can be written with frames, packets, and units, contains a quality word and the time when the frame was processed by the Frame Synchronizer.

- Quality consists of error and information flags such as "Reed Solomon corrected frame" or "frame has a CRC error."

- Each node has an attached device to either a file, a TCP/IP socket, or a null device for testing.

- The output nodes do not send data to any other nodes. They write to devices.

| Node | Receives | Sends | Description |
| --- | --- | --- | --- |
| Frame Channel | Frames | None | Writes frames with no annotation to a device. |
| Frame Channel A | Frames | None | Writes frame annotation and frames to a device. The annotation precedes the frame. |
| Frame Channel B | Frames | None | Writes frame annotation and frames to a device. The frame precedes the annotation. |
| Unit Channel | Units | None | Writes units with no annotation to a device. |
| Unit Channel A | Units | None | Writes unit annotation and units to a device. The annotation precedes the unit. |
| Unit Channel B | Units | None | Writes unit annotation and units to a device. The unit precedes the annotation. |
| Packet Channel | Packets | None | Writes packets with no annotation to a device. |
| Packet Channel A | Packets | None | Writes packet annotation, frame annotation, and packets to a device. The annotation precedes the packet. |
| Packet Channel B | Packets | None | Writes packet annotation, frame annotation, and packets to a device. The packet precedes the annotation. |
| Null Channel | All | None | Discards all frames, packets, and units. |

These nodes are dedicated to the Terra and Aqua satellites.

- The Terra decoder is inserted between the Frame Status node and the CADU Service node.

- Terra and Aqua "Sorceror" are output nodes that create PDS or EDS files. More than one may exist.

- The only difference between the two "Sorceror" nodes is how they interpret the time field in the packet secondary header.

| Node | Receives | Sends | Description |
| --- | --- | --- | --- |
| Terra Decoder | Frames | Frames | Removes Terra's special pseudo-noise encoding. |
| Terra "Sorceror" | Packets | None | Create a PDS/EDS construction record and one or more PDS/EDS data files. |
| Aqua "Sorceror" | Packets | None | Create a PDS/EDS construction record and one or more PDS/EDS data files. |

Performance

- The server has maintained a 20 megabit/sec datarate in our tests.

- The test computer was a ???????.

- Frame size: 1024 bytes. Packet size: 642 bytes.

- The frames were Reed Solomon encoded at interleave 4.

- Output was simultaneously sent to a PDS file and a TCP/IP socket.

- Input was a UDP socket with a 32 kilobyte buffer size.

- The STPS is written in java 1.2.

The major components of the STPS are:

- A multithreaded server that runs continuously.

- One thread listens for UDP datagrams containing unsynchronized telemetry.

- A second thread listens for user commands to load and shut down a session, and to get status information.

- A graphical application viewer.

- It runs on either a remote computer or the local machine.

- It sends load and shutdown commands to the server through a socket.

- It periodically gets status information, which it displays on the screen.

- Configuration files may be either on the user's computer or the server's computer.

- A graphical editor

- Users may create or edit configuration files.

- This component is not yet available.

- A web viewer

- Users may view status information on a web page.

- A servlet will connect the browser to the STPS server.

- This component is not yet available.